# 3GPP TSG RAN WG1 #102-e														R1-2006186

# e-Meeting, August 17th – 28th, 2020

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.212 | CR | DRAFT | rev |  | Current version: | 14.13.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Applicability of fields to CSS in eMTC |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Incorporated |  |  |  |  |  |  |  |  |  |
| Source to TSG: |  |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_eMTC4-Core |  |  |  |  |  | Date: |  |  | 2020-08-07 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-14 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | The bitwidth/presence of the fields “repetition number” and “modulation order override” depend on the configuration of unicast RRC parameters. In a common search space (e.g. Type-2 common) the eNB may not be aware of the RRC configuration of the UE, and therefore the DCI size / fields cannot depend on any unicast RRC configuration. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Clarify that the default behavior (2 bits for “repetition number”, absence of field “modulation order override”) apply for common search space. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Misalignment on DCI size between eNB and UE. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.3.3.1.10 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

##### 5.3.3.1.10	Format 6-0A

<Unchanged parts are omitted>

-	Repetition number – 2 or 3 bits as defined in clause 8.0 of [3]. The 3-bit field applies when ce-pdsch-puschEnhancement-config is configured by higher layers and the DCI is mapped onto the UE-specific search space given by the C-RNTI as defined in [3], otherwise the 2-bit field applies.

-	HARQ process number – 3 bits

-	New data indicator – 1 bit

-	Redundancy version – 2 bits

-	TPC command for scheduled PUSCH – 2 bits as defined in clause 5.1.1.1 of [3]

-	UL index – 2 bits as defined in clauses 5.1.1.1, 7.2.1, 8 and 8.4 of [3] (this field is present only for TDD operation with uplink-downlink configuration 0)

-	Downlink Assignment Index (DAI) – 2 bits as defined in clause 7.3 of [3] (This field is present only for cases with TDD primary cell and either TDD operation with uplink-downlink configurations 1-6 or FDD operation. This field is reserved when the configured maximum repetition number is larger than 1 for MPDCCH, or when the higher layer parameter csi-NumRepetitionCE-r13 indicates more than one subframe)

-	CSI request – 1 bit as defined in clause 7.2.1 of [3]

-	SRS request –1 bit. The interpretation of this field is provided in clause 8.2 of [3]

-	DCI subframe repetition number – 2 bits as defined in clause 9.1.5 of [3]

-	Modulation order override – 1 bit as defined in clause 8.6.1 of [3]. This field is only present when ce-pdsch-puschEnhancement-config is configured by higher layers and the DCI is mapped onto the UE-specific search space given by the C-RNTI as defined in [3]

<Unchanged parts are omitted>