3GPP TSG RAN WG1 meeting #102-e					                           R1-2006197

E-Meeting, August 17th – 28th, 2020

Agenda Item: 8.5.2

Source: MediaTek

Title: Evaluation of DL-TDOA and DL-AoD Techniques under IIoT Scenarios

Document for:	Discussion

# Introduction

In Rel-16, several positioning techniques (DL-TDOA, DL-AoD, Multi-RTT, UL-RTOA, UL-AoA, E-CID) have been defined to support regulatory and commercial use cases. For commercial uses cases, the target horizontal positioning requirements were

- Positioning error <3 m (80% UEs) for indoor scenarios

- Positioning error <10 m (80% UEs) for outdoor scenarios

The Rel-16 positioning study has been concluded in TR 38.855.

To address the higher accuracy positioning requirements for new industrial applications, it has been agreed (see RP-193237) that in Rel-17 RAN1 should evaluate and specify enhancements and solutions to meet the following exemplary performance targets:

(a) For general commercial use cases (e.g., TS 22.261):

- sub-meter level position accuracy (< 1 m)

(b) For IIoT Use Cases (e.g., 22.804):

- position accuracy < 0.2 m

The target latency requirement is < 100 ms; for some IIoT use cases, latency in the order of 10 ms is desired.

In this contribution, we show our evaluation of DL-AoD & DL-TDOA technique under IIoT Scenario.

# Overview of IIoT Scenario

Regarding IIoT use cases, there are 5 indoor factory (InF) scenarios defined in TR 38.901:

- InF-SL: Indoor Factory with Sparse clutter and Low base station height

  - Both Tx and Rx are below the average height of the clutter

- InF-DL: Indoor Factory with Dense clutter and Low base station height

  - Both Tx and Rx are below the average height of the clutter

- InF-SH: Indoor Factory with Sparse clutter and High base station height

  - Tx or Rx elevated above the clutter

- InF-DH: Indoor Factory with Dense clutter and High base station height

  - Tx or Rx elevated above the clutter

- InF-HH: Indoor Factory with High Tx and High Rx

  - Tx & Rx both elevated above the clutter

Where clutter represents machinery, assembly lines, storage shelves, etc. Sparse clutter means that percentage of surface area occupied by clutter is smaller. Evaluation parameters for InF scenarios are given in Table 7.2-4 in TR 38.901.

One main difference between InF scenarios is the LOS probability. In the following we show the LOS probability (as a function of the distance between UE and gNB) for InF and InH scenarios:

Figure 1: LOS probability for InF scenarios

It can be seen that InF-SH has very high LOS probability, whereas InF-DL and InF-DH have very low LOS probability. In this contribution, we only consider InF-SH and InF-SL scenarios.

# Evaluation of DL-TDOA technique under IIoT Scenario

In simulations, we consider FR1 fc 4GHz, SCS 15kHz, and BW 50MHz.

The PRS configuration for each TRP:

- Number of PRS resources (Tx beams): 1

- PRS comb: 6

- PRS symbol number in one slot: 6

- PRS repetition: 5

- PRS time gap: 1

The other simulation settings are given in Appendix 2. Note that we assume perfect gNB synchronization.

Regarding UE’s algorithm for TOA estimation, in addition to conventional IFFT based algorithm, we also consider super resolution algorithm. The time resolution of conventional IFFT based algorithm is mainly limited by SCS and FFT size. With SCS 15kHz and BW 50MHz, the smallest time granularity for IFFT based algorithm is 1/(15000*4096) = 16ns.

By using super resolution algorithm, under the same SCS and FFT size, UE is able to search TOA with time granularity finer than 16ns. In our simulation, we set the TOA search time granularity as 1ns.

We note that super resolution algorithm has better TOA estimation accuracy than IFFT based algorithm in LOS channel, but not in NLOS channel condition. In simulations, we consider two different channel settings: all LOS channel condition or realistic channel condition. All LOS channel condition means that every UE-TRP link is LOS channel, whereas realistic channel condition means that a UE-TRP link may be LOS or NLOS channel.

In the following, we show our simulation results.

Figure 2: Performance of DL-TDOA positioning under InF-SH scenario

Figure 3: Performance of DL-TDOA positioning under InF-SL scenario

In above, best 10 TRPs means that the location server estimates UE’s position by using the RSTDs measured from the 10 TRPs with maximum PRS-RSRP reported by UE (so 9 RSTDs are used in this case).

We have following observations from the simulation results:

Observation 1: For DL-TDOA in InF-SH (inter-site distance (ISD) 50m):

- DL-TDOA can achieve positioning error <1m for 80% UEs if UE applies super resolution algorithm for TOA estimation.

- In addition, in comparison with UE applying IFFT bases algorithm, the location server may use less RSTDs to estimate the UE’s position while maintaining good positioning accuracy. In our simulation, we see that [UE super resolution algorithm, location server positioning with best 10 TRPs] has better performance than [UE IFFT based algorithm, location sever positioning with best 16 TRPs]

Observation 2: For DL-TDOA in InF-SL: (ISD 20m):

- With all LOS channel assumption, DL-TDOA can achieve positioning error <1m for 80% UEs if UE applies super resolution algorithm for TOA estimation. Also, as in observation 1, we see that [UE super resolution algorithm, location server positioning with best 10 TRPs] has better performance than [UE IFFT based algorithm, location sever positioning with best 16 TRPs]

- However, considering realistic channel model, UE applying super resolution algorithm doesn’t lead to better positioning accuracy. In this scenario, DL-TDOA can achieve positioning error <2m for 80% UEs with UE applying IFFT based algorithm for TOA estimation

# Evaluation of DL-AoD technique under IIoT Scenario

A brief description of DL-AoD technique is given in Appendix 2.

In simulations, we consider FR2 fc 30GHz, SCS 120kHz, and BW 100MHz.

The PRS configuration for each TRP:

- Number of PRS resources (Tx beams): 8

- PRS comb: 4

- PRS symbol number in one slot: 4

- PRS repetition: 10

- PRS time gap: 8

- PRS resource offset in slot level: for PRS resource k, the slot offset is k, where k = 0,1,…,7

See the following table for an illustration. Note that for each PRS beam, The UE fixes its Rx beam to measure PRS for the 10 repetitions, and then UE estimate the PRS-RSRP for each PRS beam based on these 10 measurements.

| PRS resource 0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| PRS resource 1 |  |  |  |  |  |  |  |  |
| PRS resource 2 |  |  |  |  |  |  |  |  |
| PRS resource 3 |  |  |  |  |  |  |  |  |
| PRS resource 4 |  |  |  |  |  |  |  |  |
| PRS resource 5 |  |  |  |  |  |  |  |  |
| PRS resource 6 |  |  |  |  |  |  |  |  |
| PRS resource 7 |  |  |  |  |  |  |  |  |
| slot | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 |

The PRS from multiple TRPs are arrange in time/frequency domain so that there is no interference between PRSs from these TRPs. The other simulation settings are given in Appendix 1.

In the following we show the simulation results for DL-AoD technique in InF-SH and InF-SL scenarios:

Figure 4: Performance of DL-AoD positioning under InF-SH scenario

Figure 5: Performance of DL-AoD positioning under InF-SL scenario

We have the following observations from the simulation results:

Observation 3: For DL-AoD in InF-SH:

- Even assuming all LOS channel, DL-AoD technique cannot achieve error <1m for 80% UEs

- Consider realistic channel condition. We see that DL-AoD can only achieve error < 2.4m for 80% UEs

Observation 4: For DL-AoD in InF-SL:

- Assuming all LOS channel, DL-AoD technique can achieve error <70cm for 80% UEs

- However, consider realistic channel condition, we then see that DL-AoD can only achieve error < 1.5m for 80% UEs

- Performance in InF-SH is worse than that in InF-SL. This is because ISD in InF-SH is larger than that in InF-SL. Note that under the same AoD estimation error, large ISD would lead to larger positioning error

# Conclusions

Observation 1: For DL-TDOA in InF-SH (inter-site distance (ISD) 50m):

- DL-TDOA can achieve positioning error <1m for 80% UEs if UE applies super resolution algorithm for TOA estimation.

- In addition, in comparison with UE applying IFFT bases algorithm, the location server may use less RSTDs to estimate the UE’s position while maintaining good positioning accuracy. In our simulation, we see that [UE super resolution algorithm, location server positioning with best 10 TRPs] has better performance than [UE IFFT based algorithm, location sever positioning with best 16 TRPs]

Observation 2: For DL-TDOA in InF-SL: (ISD 20m):

- With all LOS channel assumption, DL-TDOA can achieve positioning error <1m for 80% UEs if UE applies super resolution algorithm for TOA estimation. Also, as in observation 1, we see that [UE super resolution algorithm, location server positioning with best 10 TRPs] has better performance than [UE IFFT based algorithm, location sever positioning with best 16 TRPs]

- However, considering realistic channel model, UE applying super resolution algorithm doesn’t lead to better positioning accuracy. In this scenario, DL-TDOA can achieve positioning error <2m for 80% UEs with UE applying IFFT based algorithm for TOA estimation

Observation 3: For DL-AoD in InF-SH:

- Even assuming all LOS channel, DL-AoD technique cannot achieve error <1m for 80% UEs

- Consider realistic channel condition. We see that DL-AoD can only achieve error < 2.4m for 80% UEs

Observation 4: For DL-AoD in InF-SL:

- Assuming all LOS channel, DL-AoD technique can achieve error <70cm for 80% UEs

- However, consider realistic channel condition, we then see that DL-AoD can only achieve error < 1.5m for 80% UEs

- Performance in InF-SH is worse than that in InF-SL. This is because ISD in InF-SH is larger than that in InF-SL. Note that under the same AoD estimation error, large ISD would lead to larger positioning error

# Appendix 1: Simulation settings for InF scenarios

|  |  | FR1 Specific Values | FR2 Specific Values |
| --- | --- | --- | --- |
| Channel model |  | [InF-SL, InF-DL, InF-SH, InF-DH and/or InF-HH] | [InF-SL, InF-DL, InF-SH, InF-DHand/or InF-HH] |
| Layout | Hall size | InF-SL: 120x60 m InF-DL: 300x150 m InF-SH: 300x150 m InF-DH: 120x60 m InF-HH: 300x150 m |  |
|  | BS locations | 18 BSs on a square lattice with spacing D, located D/2 from the walls. - for the small hall (L=120m x W=60m): D=20m - for the big hall (L=300m x W=150m): D=50m |  |
|  | Room height | 10m |  |
| Total gNB TX power, dBm |  | 24dBm | 24dBm EIRP should not exceed 58 dBm |
| gNB antenna configuration |  | 1 omni-directional Tx antenna element – Note 1 | (M, N, P, Mg, Ng) = (4, 8, 2, 1, 1), dH=dV=0.5λ – Note 1 One TXRU per polarization per panel is assumed |
| gNB antenna radiation pattern |  | Single sector – Note 1 | 3-sector antenna configuration – Note 1 |
| Peneteration loss |  | 0dB |  |
| Number of floors |  | 1 |  |
| UE drop procedure |  | 100% indoor |  |
| UE mobility |  | 0km/h |  |
| UE antenna height |  | 1.5m |  |
| Min gNB-UE distance (2D), m |  | 0m |  |
| gNB antenna height |  | BS height = 1.5 m for InF-SL and InF-DL BS-height = 8 m for for InF-SH and InF-DH |  |
| Clutter density: r |  | Low clutter density: 20% High clutter density: 60% |  |
| Clutter height: hc |  | Low clutter density: 2 m High clutter density: 6 m |  |
| Clutter size: dclutter |  | Low clutter density: 10 m High clutter density: 2 m |  |
| gNB Synchronization |  | Perfect |  |
| Note 1: According to 3GPP TR 38.802 Note 2: According to 3GPP TR 38.901 |  |  |  |

# Appendix 2: Review of DL-AoD technique

The DL-AoD technique may work as follows:

Step 1: The network configures an UE to measure RSRP for several PRS resource sets

- Note: A PRS resource set is associated with a TRP

Step 2: Each TRP transmits PRS on time/frequency resources specified by PRS resources of a PRS resource set

- Note: A PRS resource is associated with a Tx beam and a PRS resource ID

Step 3: The UE measures PRS transmitted from TRPs and reports measured PRS-RSRP to the network

Step 4: The location server estimates the AoDs based on the UE’s PRS-RSRP report

Step 5: The location server estimates the UE’s position by using the estimated AoDs