3GPP TSG RAN WG1 #102-e		  R1-2006249

e-Meeting, August 17th – 28th, 2020

Agenda Item:     7.2.6

Source:	Spreadtrum Communications

Title:	Remaining issues on CSI enhancement for MU-MIMO

Document for:	Discussion and decision

Introduction

In this contribution, for the purpose of clarification, we discuss one ambiguity in current spec and provide a corresponding text proposal to fix it.

Discussion

Clarification on the total number of precoding matrices N3

On the number of PMI subbands, we have the following agreement [1],

Agreement in RAN1#AH1901 On FD compression unit, agree on Alt1 (PMI subband size = CQI subband size) as the default, along with Alt2.2 (PMI subband size = CQI subband size / R) as secondary - The value of R is fixed to 2 - FFS: Whether secondary implies a separate UE capability or restricted use cases - Include issues such as limitation on the number of FD compression units, CPU occupation, latency constraint and/or BW constraint - FFS: Whether FD compression unit is higher-layer configured or reported by the UE

Also, the corresponding description in specification can be found in TS 38.214 [2], copied as below,

5.2.2.2.5 Enhanced Type II Codebook ***** omitted ***** The parameter R is configured with the higher-layer parameter numberOfPMISubbandsPerCQISubband-r16. This parameter controls the total number of precoding matrices N3 indicated by the PMI as a function of the number of subbands in csi-ReportingBand, the subband size configured by the higher-level parameter subbandSize and of the total number of PRBs in the bandwidth part according to Table 5.2.1.4-2, as follows: ***** omitted *****

According to the sentence marked in yellow, N3 is determined as a function of the number of subbands in csi-ReportingBand. However, according to the description of csi-ReportingBand field in TS 38.331 [3], the number of subbands refers to the total number of subbands in the bandwidth part. This may cause confusion and should be fixed.

csi-ReportingBand Indicates a contiguous or non-contiguous subset of subbands in the bandwidth part which CSI shall be reported for. Each bit in the bit-string represents one subband. The right-most bit in the bit string represents the lowest subband in the BWP. The choice determines the number of subbands (subbands3 for 3 subbands, subbands4 for 4 subbands, and so on) (see TS 38.214 [19], clause 5.2.1.4). This field is absent if there are less than 24 PRBs (no sub band) and present otherwise, the number of sub bands can be from 3 (24 PRBs, sub band size 8) to 18 (72 PRBs, sub band size 4).

Therefore, we have the following text proposal,

Proposal 1: Adopt the following text proposal in TS38.214.

----------------------------------------------Start of Text Proposal for TS38.214------------------------

5.2.2.2.5	Enhanced Type II Codebook

***** unchanged part omitted *****

The parameter R is configured with the higher-layer parameter numberOfPMISubbandsPerCQISubband-r16. This parameter controls the total number of precoding matrices N3 indicated by the PMI as a function of the number of subbands which CSI shall be reported for indicated by csi-ReportingBand, the subband size configured by the higher-level parameter subbandSize and of the total number of PRBs in the bandwidth part according to Table 5.2.1.4-2, as follows:

***** unchanged part omitted *****

---------------------------------------------End of Text Proposal for TS38.214-------------------------------------

Conclusion

In this contribution, we discussed a confusion cause by current spec and gave a text proposal to fix it.

Proposal 1: Adopt the following text proposal in TS38.214.

----------------------------------------------Start of Text Proposal for TS38.214------------------------

5.2.2.2.5	Enhanced Type II Codebook

***** unchanged part omitted *****

The parameter R is configured with the higher-layer parameter numberOfPMISubbandsPerCQISubband-r16. This parameter controls the total number of precoding matrices N3 indicated by the PMI as a function of the number of subbands which CSI shall be reported for indicated by csi-ReportingBand, the subband size configured by the higher-level parameter subbandSize and of the total number of PRBs in the bandwidth part according to Table 5.2.1.4-2, as follows:

***** unchanged part omitted *****

---------------------------------------------End of Text Proposal for TS38.214-------------------------------------

Reference

[1] Final_Minutes_report_RAN1#AH_1901_v100

[2] 3GPP TS 38.214 V16.2.0

[3] 3GPP TS 38.331 V16.1.0