3GPP TSG RAN WG1 Meeting #102                            	         R1- 2006269

e-Meeting, August 17th – 28th, 2020

Agenda Item:	8.7.1.2

Source:	Spreadtrum Communications

Title:	Consideration on TRS/CSI-RS occasion(s) for idle/inactive UEs

Document for:	Discussion and decision

Introduction

In RAN 86 meeting, it was agreed to specify means to provide potential TRS/CSI-RS occasion(s) available in connected mode to idle/inactive-mode UEs, and minimize system overhead impact [1].

- Specify enhancements for idle/inactive-mode UE power saving, considering system performance aspects [RAN2, RAN1] - Study and specify paging enhancement(s) to reduce unnecessary UE paging receptions, subject to no impact to legacy UEs [RAN2, RAN1] - NOTE: RAN1 to check and update, if needed, evaluation methodology in RAN1 #100 meeting - Specify means to provide potential TRS/CSI-RS occasion(s) available in connected mode to idle/inactive-mode UEs, minimizing system overhead impact [RAN1] - NOTE: Always-on TRS/CSI-RS transmission by gNodeB is not required

In this paper, we discuss our views on this aspect.

Reasons to introduce TRS/CSI-RS in idle

When UE operates in RRC-IDLE or RRC-INACTIVE state, UE might need to perform RRM measurement, cell re-selection if triggered and paging monitoring every paging cycle. In general, UE is expected to perform the following activities in RRC-IDLE or RRC-INACTIVE state:

- Loop convergence, e.g., AGC/TTL/FTL and etc.

- RRM measurement for cell reselection.

- Paging monitoring

In LTE, all above activities could be performed based on CRS in each PO. For NR UE, these activities are based on SSB, which may not be presented close to PO. Therefore, in normal case (as shown in Figure 1), UE may need to wake up two times in every paging cycle. One is to perform AGC/TTL/FTL and RRM for serving cell based on SSB (SMTC), the other one is to monitor paging occasion.

Figure 1: Case-1: UE power consumption in each paging cycle

In worse case, e.g., for the poor channel condition or lager paging cycle (as shown in Figure 2), UE may need to wake-up to receive 2 SSB burst (i.e., wake up two times) for AGC, T/F tracking and serving cell measurement before PO monitoring.

Figure 2: Case-2: UE power consumption in each paging cycle

From above analysis, UE may need to wake up multiple times.

Observation 1: UE may need to wake up multiple times for AGC, T/F tracking, serving cell measurement and PO monitoring in each paging cycle.

TRS/CSI-RS occasion(s) for idle/inactive UEs

Function of TRS/CSI-RS

As mentioned above, UE may need to wake-up multiple times for AGC, T/F tracking, serving cell measurement and paging monitoring in each paging cycle. If UE can perform AGC, T/F tracking and RRM measurement for serving cell based on TRS/CSI-RS, the number of times UE wakes up will be significantly reduced in each paging cycle. For example, UE could only wake up one time and stay active to perform AGC, T/F tracking, serving cell measurement and paging monitoring in short period.

In RAN1#95, some benefits of introducing additional resource were identified [2].

Agreements: Observation: Under certain conditions and certain deployment scenarios, additional resource for RRM measurement can be beneficial for UE power saving, including at least the following aspects: - Minimizing/reducing the timing gap between measurement (e.g., SSB) and DRX ON duration (e.g., paging monitoring occasion/reception, data reception, etc.) - Additional resource around the measurement occasion, e.g., for AGC assistance - Reducing measurement activities by providing additional resource may provide sufficient measurement/T-F accuracy.

In our view, UE can perform AGC, T/F tracking and serving cell measurement based on TRS/CSI-RS in idle/inactive mode.

Proposal 1: UE can perform AGC, T/F tracking, serving cell measurement with TRS/CSI-RS in idle/inactive mode.

TRS/CSI-RS configuration

TRS/CSI-RS configuration for idle/inactive mode UE should consider both system overhead impact and signaling overhead. There are two TRS/CSI-RS configuration options for idle/inactive mode UE:

- Cell specific TRS/CSI-RS configuration, e.g., via RMSI broadcasts TRS/CSI-RS configuration

- UE-Specific RRC signaling, i.e., gNB configures RS when the UE is in CONNECTED mode via UE specific RRC signaling (e.g., via RRC release message)

In our view, both cell specific and UE-specific TRS/CSI-RS configuration should be considered at the beginning of the WI.

Proposal 2: Both cell specific and UE-specific TRS/CSI-RS configuration should be considered.

Evaluation

In this section, we will simply evaluate the energy-saving gain brought by TRS/CSI-RS transmission in idle/inactive mode.

Evaluation methodology

Baseline assumption in our evaluation is assumed as the following:

- SCS=30KHz

- Paging cycle=1280/640ms

- SSB burst periodicity=20ms

- UE bandwidth=20MHz

Following the power model defined in 38.840 [2], we calculate the power consumption with power scaling for different power states for IDLE mode UE shown in the following table according to [3].

Table 1: Power consumption for different power state

| Power state | Relative Power |
| --- | --- |
| SSB | 50 |
| TRS/CSI-RS | 50 |
| Neighbour cell measurement | 60 |
| PDCCH_Only | 50 |
| PDCCH+PDSCH | 120 |
| Micro sleep | 45 |
| Light sleep | 20 |
| Deep sleep | 1 |
| Light sleep to non-sleep | 100 |
| Deep sleep to non-sleep | 450 |

Paging traffic model in our evaluation is assumed as the following table:

Table 2: Paging traffic model for evaluation

| Paging rate | 0.01 |
| --- | --- |
| UEs per group | 100 |
| Group paging rate | 60% |

To simplify the evaluation, for Case 1, no neighbor cell measurement is assumed; For Case-2, one 5ms SMTC window is assumed for neighbor cell measurement after PO. In our evaluation, the TRS/CSI-RS transmission occasion is assumed before the PO.

Evaluation results

To evaluate the power gains of introducing TRS/CSI-RS transmission in idle/inactive mode, four different test cases are assumed as the following table:

Table 3: Four different test cases

|  | Evaluation assumptions |
| --- | --- |
| Test 1 | Paging cycle=1.28s |
|  | Case-1 |
| Test 2 | Paging cycle=1.28s |
|  | Case-2 |
| Test 3 | Paging cycle=640ms |
|  | Case-1 |
| Test 4 | Paging cycle=640ms |
|  | Case-2 |

For each test case, we evaluate the power saving gain with TRS/CSI-RS transmission compared to the scenario without TRS/CSI-RS transmission in idle/inactive mode. The evaluation time is 100 paging cycles for each test case. The power saving gains for different test cases are shown in the following tables:

Table 4: The evaluation results of test 1

|  | Without TRS/CSI-RS transmission | With TRS/CSI-RS transmission | Note |
| --- | --- | --- | --- |
| PDCCH-only energy | 2000 | 2000 | No neighbor cell measurement is assumed |
| PDCCH+PDSCH energy | 7200 | 7200 |  |
| SSB or TRS/CSI-RS processing | 20000 | 20000 |  |
| Deep sleep | 248000 | 251000 |  |
| Light sleep | 28000 | NA |  |
| Deep sleep to non-sleep | 45000 | 45000 |  |
| Light sleep to non-sleep | 5000 | NA |  |
| Neighbor cell measurement | NA | NA |  |
| Total Power consumption | 355200 | 325200 |  |
| Power saving gains | 8.45% |  |  |

Table 5: The evaluation results of test 2

|  | Without TRS/CSI-RS transmission | With TRS/CSI-RS transmission | Note |
| --- | --- | --- | --- |
| PDCCH-only energy | 2000 | 2000 | One 5ms SMTC window is assumed for neighbor cell measurement after PO |
| PDCCH+PDSCH energy | 7200 | 7200 |  |
| SSB or TRS/CSI-RS processing | 40000 | 40000 |  |
| Deep sleep | 244000 | 248000 |  |
| Light sleep | 112000 | 56000 |  |
| Deep sleep to non-sleep | 45000 | 45000 |  |
| Light sleep to non-sleep | 25000 | 15000 |  |
| Neighbor cell measurement | 60000 | 60000 |  |
| Total Power consumption | 535200 | 473200 |  |
| Power saving gains | 11.6% |  |  |

Table 6: The evaluation results of test 3

|  | Without TRS/CSI-RS transmission | With TRS/CSI-RS transmission | Note |
| --- | --- | --- | --- |
| PDCCH-only energy | 2000 | 2000 | No neighbor cell measurement is assumed |
| PDCCH+PDSCH energy | 7200 | 7200 |  |
| SSB or TRS/CSI-RS processing | 20000 | 20000 |  |
| Deep sleep | 122000 | 123000 |  |
| Light sleep | 28000 | NA |  |
| Deep sleep to non-sleep | 45000 | 45000 |  |
| Light sleep to non-sleep | 5000 | NA |  |
| Neighbor cell measurement | NA | NA |  |
| Total Power consumption | 229200 | 197200 |  |
| Power saving gains | 14.0% |  |  |

Table 7: The evaluation results of test 4

|  | Without TRS/CSI-RS transmission | With TRS/CSI-RS transmission | Note |
| --- | --- | --- | --- |
| PDCCH-only energy | 2000 | 2000 | One 5ms SMTC window is assumed for neighbor cell measurement after PO |
| PDCCH+PDSCH energy | 7200 | 7200 |  |
| SSB or TRS/CSI-RS processing | 40000 | 40000 |  |
| Deep sleep | 116000 | 120000 |  |
| Light sleep | 112000 | 56000 |  |
| Deep sleep to non-sleep | 45000 | 45000 |  |
| Light sleep to non-sleep | 25000 | 15000 |  |
| Neighbor cell measurement | 60000 | 60000 |  |
| Total Power consumption | 407200 | 345200 |  |
| Power saving gains | 15.3% |  |  |

As seen from the above evaluation results, TRS/CSI-RS transmission before PO can bring obvious power saving gain, especially when paging cycle is small. The power saving gain mainly comes from the power reduction of light sleep and the corresponding transition.

Observation 2: TRS/CSI-RS transmission for idle/inactive mode UE can bring obvious power saving gains.

Conclusion

As summary, we have the following proposals.

Proposal 1: UE can perform AGC, T/F tracking, serving cell measurement with TRS/CSI-RS in idle/inactive mode.

Proposal 2: Both cell specific and UE-specific TRS/CSI-RS configuration should be considered.

Reference

- RP-193239, “New WID: UE Power Saving Enhancements”, MediaTek Inc.

- 3GPP RAN1#95 Chairman’s Notes, November 12 - 16, 2018.

- RP-2003667, “Evaluations and Observations for R17 UE Power Saving Enhancements”, RAN1#101e, May 25th – June 5th, 2020.