3GPP TSG RAN WG1 #102-e		R1-2006346

e-Meeting, Aug. 17th – Aug. 29th, 2020

Source:	Panasonic

Title: 	Baseline coverage performance analysis in FR1

Agenda Item:		8.8.1.1

Document for:	Discussion

Introduction

A study item on NR coverage enhancement was approved [1]. One of objectives of this study item is to identify baseline coverage performance for both DL and UL for the following scenarios and services based on link-level simulation.

- The target scenarios and services include

  - Urban (outdoor gNB serving indoor UEs) scenario, and rural scenario (including extreme long distance rural scenario) for FR1

  - Indoor scenario (indoor gNB serving indoor UEs), and urban/suburban scenario (including outdoor gNB serving outdoor UEs and outdoor gNB serving indoor UEs) for FR2

  - TDD and FDD for FR1

  - VoIP and eMBB services for FR1

  - eMBB service as first priority and VoIP as second priority for FR2

  - LPWA services and scenarios are not included.

The agreements on baseline coverage performance evaluation methodology are summarized in Appendix A. In this document, we provide our views and proposals regarding to the baseline coverage performance evaluation methodology. In addition, we provide preliminary baseline coverage performance analysis in FR1.

Discussion

Generally, the identification of coverage issues uses the MCL methodology. The coupling loss is defined as the total long-term channel loss over the link between the UE antenna ports and the gNB antenna ports, and includes in practical antenna gains, path loss, shadowing, body loss, etc. The MCL is the limit value of the coupling loss at which the service can be delivered, and therefore defines the coverage of services. The MCL is evaluated via link budget analysis (supported by link-level simulation). In RAN1#101e, it was agreed to down select on the following options for link budget template.

- Option 1: Adopt single link budget template based on IMT-2020 self-evaluation with necessary revisions, including adding/removing/revising some parameters.

- Option 2: Adopt both templates, i.e., budget template in IMT-2020 self-evaluation and link budget template in TR36.824.

- Option 3: Adopt single link budget template in TR36.824 with necessary revisions, including adding/revising some parameters.

The link budget calculation based on IMT-2020 self-evaluation (i.e., Option 1) is based on available path loss, while the link budget based on TR36.824 (i.e., Option 3) is based on calculation of MCL. In our view, which option should be selected depends on how interference is handled and system level simulation is taken into account. If target ISD is interference limited operation, system level simulation is necessary to judge interference level and Option 1 would be suitable. If target ISD is just noise limited operation, the different between Option 1 and Option 3 is rather how these are expressed and would be no big difference. For Option 3, necessary revisions would be adding antenna gain and beamforming gain. If such addition of antenna and beamforming gain is given as semi-static parameters, it could be beneficial to simplify the link level evaluation than to have them in the link-level evaluation.

Observation 1: For link budget template, if target ISD is noise limited operation, either option can be workable.

Proposal 1: For link budget template in TR36.824, antenna and beamforming gain is added.

On target performance, following options were identified in RAN1#101e [2].

- Option 1: The target path loss is considered as the target performance.

  - Alt.1: Derived from the target ISD

  - Alt.2: Relative MPL

- Option 2: An MCL or MCL based metric is considered as the target performance.

  - Alt.1: Derived from the target ISD, considering shadow fading margin, penetration loss, etc.

  - Alt.2: Fixed target MCL, e.g., 147 dB for VoIP to achieve better performance than other RAT(s).

  - Alt.3: Relative MCL

Whether Option 1 or Option 2 is considered depends on which option is used for link budget template, i.e., based on available path loss or based on MCL in TR36.824. We are open to consider either way. The relative performance target (i.e., Option 1-Alt.2 or Option 2-Alt.3) can be used to identify which channel(s) need to be improved and it can make the performance of all channels similar. However, in order to identify the how much coverage improvement is necessary for each channel, absolute value of target performance (i.e., Option 1-Alt.1, Option Alt.1, or Option 2-Alt.2) should be used.

Proposal 2: The target performance should be derived from the target ISD or fixed target MCL.

Preliminary baseline coverage performance

Simulation assumptions

In our preliminary link-level simulation, eMBB PUSCH and PUCCH are evaluated. General assumptions used in our evaluation are summarized in Table 1.

Table 1: General simulation parameters

| Scenario | Urban | Rural |
| --- | --- | --- |
| eMBB Target data rate | DL: 10Mbps UL: 1Mbps | DL: 1Mbps UL: 100kbps |
| Frequency | 4 GHz (TDD) | 4 GHz (TDD), 2 GHz (FDD), 700 MHz (FDD) |
| Frame structure for TDD | DDDSU (S: 10D:2G:2U) DDDSUDDSUU (S: 10D:2G:2U) |  |
| Pathloss model | NLOS |  |
| BWP | 100 MHz for 4 GHz 20 MHz for 2 GHz and 700 MHz |  |
| SCS | 30 kHz for TDD, 15 kHz for FDD |  |
| Number of antenna elements for BS | 192 for 4 GHz | 64 for 4 GHz, 32 for 2 GHz, 16 for 700 MHz |
| Number of TxRUs for BS | 2 for 2 GHz and 700 MHz 64 TxRUs for 4 GHz gNB modelling in LLS for TDL: 4 gNB receive chains in LLS |  |
| Channel model | TDL-C |  |
| Delay spread | 300 ns |  |
| UE velocity | 3 km/h | 3 km/h for indoor, 120 km/h for outdoor |

Channel-specific parameters for link-level evaluation are summarized in Table 2 and 3.

Table 2: Link-level simulation parameters for PUSCH

| Parameters | Values |
| --- | --- |
| Target BLER | 10% iBLER |
| Number of UE transmit chains | 1 |
| Waveform | DFT-s-OFDM |
| PRBs / TBS / MCS | 30 PRBs for 1Mbps, 4 PRBs for 100kbps TBS and MCS is derived based on e.g., the number of PRBs, target data rate, frame structure and overhead. |
| Duration | 14 OFDM symbols |
| Frequency hopping | Enabled or disabled |
| DMRS configuration | PUSCH mapping Type A 120 km/h: Type 1, 3 DMRS symbol, no multiplexing with data For frequency hopping: Type 1, 2 DMRS symbol for each hop, no multiplexing data 3 km/h: Type 1, 2 DMRS symbol, no multiplexing with data For frequency hopping: Type 1, 2 DMRS symbol for each hop, no multiplexing data |
| Repetition | w/o repetition |
| HARQ configuration | w/o HARQ |

Table 3: Link-level simulation parameters for PUCCH

| Parameters | Values |
| --- | --- |
| PUCCH format | Format 1, 2 bits UCI Format 3, 11 or 22 bits UCI |
| Target performance | PUCCH format 1: DTX to ACK probability: 1% NACK to ACK probability: 0.1% ACK missed detection probability: 1% PUCCH format 3: BLER for ACK/NACK, SR: 1% |
| Number of UE transmit chains | 1 |
| Number of PRBs | 1 PRB |
| Duration | 14 OFDM symbols |
| Frequency hopping | Enabled |
| DMRS configuration | 2 DMRS symbols per hop for PUCCH format 3 |
| Number of repetitions | w/o repetition |

The required SINR obtained from link-level evaluation results are summarized in Table 4. In addition, baseline coverage performance based on required SINR and link budget template is also included in Table 4. Link budget template based on IMT-2020 self-evaluation is tentatively used. The detailed values for link budget template for different scenarios are summarized in Appendix B. The detailed values are obtained in [3]. The target performance is tentatively calculated based on [4].

Table 4: Baseline coverage performance (Urban)

| PUSCH | Frame structure | Frequency hopping | Required SINR (dB) | Available path loss (dB) | Target performance (dB) | Gap (dB) |
| --- | --- | --- | --- | --- | --- | --- |
|  | DDDSU | Disabled | -4.7 | 109.98 | 121.75 | 11.77 |
|  |  | Enabled | -4.5 | 109.78 |  | 11.97 |
|  | DDDSUDDSUU | Disabled | -6.5 | 111.78 |  | 9.97 |
|  |  | Enabled | -6.3 | 111.58 |  | 10.17 |
| PUCCH | PUCCH format |  | Required SINR (dB) | Available path loss (dB) | Target performance (dB) | Gap (dB) |
|  | Format 1 |  | -9.5 | 122.90 | 121.75 | -1.15 |
|  | Format 3 (11 bits) |  | -8.0 | 121.40 |  | 0.35 |
|  | Format 3 (22 bits) |  | -4.1 | 117.50 |  | 4.25 |

Table 5: Baseline coverage performance (Rural, Indoor, 4 GHz)

| PUSCH | Frame structure | Frequency hopping | Required SINR (dB) | Available path loss (dB) | Target performance (dB) | Gap (dB) |
| --- | --- | --- | --- | --- | --- | --- |
|  | DDDSU | Disabled | -3.5 | 126.51 | 131.57 | 5.06 |
|  |  | Enabled | -3.8 | 126.81 |  | 4.76 |
|  | DDDSUDDSUU | Disabled | -5.0 | 128.01 |  | 3.56 |
|  |  | Enabled | -5.5 | 128.51 |  | 3.06 |
| PUCCH | PUCCH format |  | Required SINR (dB) | Available path loss (dB) | Target performance (dB) | Gap (dB) |
|  | Format 1 |  | -9.5 | 131.88 | 131.57 | -0.31 |
|  | Format 3 (11 bits) |  | -8.0 | 130.38 |  | 1.19 |
|  | Format 3 (22 bits) |  | -4.1 | 126.48 |  | 5.09 |

Table 6: Baseline coverage performance (Rural, Outdoor, 4 GHz)

| PUSCH | Frame structure | Frequency hopping | Required SINR (dB) | Available path loss (dB) | Target performance (dB) | Gap (dB) |
| --- | --- | --- | --- | --- | --- | --- |
|  | DDDSU | Disabled | -3.9 | 130.04 | 131.57 | 1.53 |
|  |  | Enabled | -3.9 | 130.44 |  | 1.53 |
|  | DDDSUDDSUU | Disabled | -4.8 | 130.94 |  | 0.63 |
|  |  | Enabled | -5.6 | 131.74 |  | -0.17 |
| PUCCH | PUCCH format |  | Required SINR (dB) | Available path loss (dB) | Target performance (dB) | Gap (dB) |
|  | Format 1 |  | -9.8 | 135.17 | 131.57 | -3.60 |
|  | Format 3 (11 bits) |  | -8.2 | 133.57 |  | -2.00 |
|  | Format 3 (22 bits) |  | -4.1 | 129.47 |  | 2.10 |

Table 7: Baseline coverage performance (Rural, Indoor, 2 GHz)

| PUSCH | Frequency hopping | Required SINR (dB) | Available path loss (dB) | Target performance (dB) | Gap (dB) |
| --- | --- | --- | --- | --- | --- |
|  | Disabled | -0.9 | 126.27 | 125.55 | -0.72 |
|  | Enabled | -2.2 | 127.57 |  | -2.02 |
| PUCCH | PUCCH format | Required SINR (dB) | Available path loss (dB) | Target performance (dB) | Gap (dB) |
|  | Format 1 | -10.2 | 130.40 | 125.55 | -4.85 |
|  | Format 3 (11 bits) | -7.9 | 128.10 |  | -2.55 |
|  | Format 3 (22 bits) | -4.0 | 123.50 |  | 2.05 |

Table 8: Baseline coverage performance (Rural, Outdoor, 2 GHz)

| PUSCH | Frequency hopping | Required SINR (dB) | Available path loss (dB) | Target performance (dB) | Gap (dB) |
| --- | --- | --- | --- | --- | --- |
|  | Disabled | 0.3 | 127.09 | 125.55 | -1.54 |
|  | Enabled | -2.4 | 129.79 |  | -4.24 |
| PUCCH | PUCCH format | Required SINR (dB) | Available path loss (dB) | Target performance (dB) | Gap (dB) |
|  | Format 1 | -10.0 | 132.00 | 125.55 | -6.45 |
|  | Format 3 (11 bits) | -8.4 | 130.30 |  | -4.75 |
|  | Format 3 (22 bits) | -4.4 | 125.40 |  | 0.15 |

Table 9: Baseline coverage performance (Rural, Indoor, 700 MHz)

| PUSCH | Frequency hopping | Required SINR (dB) | Available path loss (dB) | Target performance (dB) | Gap (dB) |
| --- | --- | --- | --- | --- | --- |
|  | Disabled | -0.6 | 122.96 | 116.44 | -6.52 |
|  | Enabled | -2.3 | 124.66 |  | -8.22 |
| PUCCH | PUCCH format | Required SINR (dB) | Available path loss (dB) | Target performance (dB) | Gap (dB) |
|  | Format 1 | -10.2 | 126.69 | 116.44 | -10.25 |
|  | Format 3 (11 bits) | -7.8 | 125.09 |  | -8.65 |
|  | Format 3 (22 bits) | -3.8 | 120.39 |  | -3.95 |

Table 10: Baseline coverage performance (Rural, Outdoor, 700 MHz)

| PUSCH | Frequency hopping | Required SINR (dB) | Available path loss (dB) | Target performance (dB) | Gap (dB) |
| --- | --- | --- | --- | --- | --- |
|  | Disabled | -1.5 | 125.88 | 116.44 | -9.44 |
|  | Enabled | -2.5 | 126.88 |  | -10.44 |
| PUCCH | PUCCH format | Required SINR (dB) | Available path loss (dB) | Target performance (dB) | Gap (dB) |
|  | Format 1 | -9.9 | 128.99 | 116.44 | -12.55 |
|  | Format 3 (11 bits) | -8.4 | 126.99 |  | -10.55 |
|  | Format 3 (22 bits) | -4.4 | 122.69 |  | -6.25 |

Based on our preliminary evaluation for eMBB in FR1, around 10 dB and 5 dB coverage enhancement are necessary for PUSCH in urban and in rural indoor scenario, respectively. For PUCCH, if PUCCH format 3 with lager payload size (22 bits) is necessary, it could be bottleneck of PUCCH coverage.

Observation 2: Around 10 dB coverage enhancement is necessary for eMBB PUSCH in urban scenario.

Observation 3: Around 5 dB coverage enhancement is necessary for eMBB PUSCH in rural indoor scenario.

Observation 4: PUCCH format 3 with larger payload size could be bottleneck of PUCCH coverage in urban and rural indoor scenarios.

Conclusion

In this contribution, we provide our views regarding to the baseline coverage performance evaluation methodology and preliminary baseline coverage performance analysis in FR1. We made following observations and proposals.

Observation 1: For link budget template, if target ISD is noise limited operation, either option can be workable.

Proposal 1: For link budget template in TR36.824, antenna and beamforming gain is added.

Proposal 2: The target performance should be derived from the target ISD or fixed target MCL.

Observation 2: Around 10 dB coverage enhancement is necessary for eMBB PUSCH in urban scenario.

Observation 3: Around 5 dB coverage enhancement is necessary for eMBB PUSCH in rural indoor scenario.

Observation 4: PUCCH format 3 with larger payload size could be bottleneck of PUCCH coverage in urban and rural indoor scenarios.

Reference

[1]		RP-193240, “New SID on NR coverage enhancement,” China Telecom, RAN#86, December 2019.

[2]	R1-2005192, “[101-e-Post-NR-Cov-Enh] Email discussion/approval focusing on remaining evaluation assumptions,” Moderator (China Telecom), RAN1#101e.

[3]	RP-191531, “Link budge template – RIT,”

[4]	R1-2003834, “Evaluation methodology and preliminary baseline performance for NR coverage enhancements,” China Telecom, RAN1#101e.

- Appendix A: Previous agreements

Note that the agreements on link-level simulation assumptions are not included.

RAN1#101e

Agreements:

- The basic evaluation methodology is based on link-level simulation for FR1

  - Step 1: Obtain the required SINR for the physical channels under target scenarios and service/reliability requirements.

  - Step 2: Obtain the baseline performance based on required SINR and link budget template

  - Note: Aspects related to identifying target performance and coverage bottlenecks based on target performance metric is to be handled separately.

- The evaluation methodology based on system-level simulation is optional for FR1.

  - Note: The simulation assumptions for SLS are up to companies’ reports.

Agreements:

- The evaluation methodology for FR2 is the same as FR1.

Agreements:

- Down selection on the following options for the link budget template for FR1 in next meeting.

  - Option 1: Adopt single link budget template based on IMT-2020 self-evaluation with necessary revisions, including adding/removing/revising some parameters.

    - FFS: The template provided by FL in R1-2005005

  - Option 2: Adopt both templates, i.e., budget template in IMT-2020 self-evaluation and link budget template in TR36.824.

  - Option 3: Adopt single link budget template in TR36.824 with necessary revisions, including adding/revising some parameters.

Agreements:

- The link budget template for FR2 is the same as FR1.

Agreements:

- Down selection on the following options for antenna array gain for LLS based methodology for FR1 in next meeting.

  - Option 1: Antenna array gain is included in the link budget template.

    - FFS: array gain = 10 * log10 (number of antenna elements / number of TxRUs)

    - FFS: For TDL channel model

    - FFS: Values reflective of realistic implementation and network operation

  - Option 2: Antenna array gain is included in LLS

    - FFS: For CDL channel model

- Appendix B: Link budget template

Table 11: The detailed values for link budget template

|  | Urban | Rural |  |  |
| --- | --- | --- | --- | --- |
| Frequency | 4GHz | 4 GHz | 2 GHz | 700 MHz |
| Transmitter |  |  |  |  |
| (1) Number of transmit antennas | 1 | 1 | 1 | 1 |
| (1bis) Number of transmit antenna ports | 1 | 1 | 1 | 1 |
| (2) Maximal transmit power per antenna (dBm) | 23.00 | 23.00 | 23.00 | 23.00 |
| (3) Total transmit power = function of (1) and (2) (dBm) | 23.00 | 23.00 | 23.00 | 23.00 |
| (4) Transmitter antenna gain (dBi) | 0 | 0 | 0 | 0 |
| (5) Transmitter array gain (depends on transmitter array configurations and technologies such as adaptive beamforming, CDD, etc.) (dB) | 0 | 0 | 0 | 0 |
| (6) Control channel power boosting gain (dB) | 0 | 0 | 0 | 0 |
| (7) Data channel power loss due to pilot/control boosting (dB) | 0 | 0 | 0 | 0 |
| (8) Cable, connector, combiner, body losses, etc. (enumerate sources) (dB) (feeder loss must be included for and only for downlink) | 1.00 | 1.00 | 1.00 | 1.00 |
| (9a) Control channel e.i.r.p. = (3) + (4) + (5) + (6) – (8) dBm | 22.00 | 22.00 | 22.00 | 22.00 |
| (9b) Data channel e.i.r.p = (3) + (4) + (5) – (7) – (8) dBm | 22.00 | 22.00 | 22.00 | 22.00 |
| Receiver |  |  |  |  |
| (10) Number of receive antennas | 192 | 64 | 32 | 16 |
| (10bis) Number of receive antenna ports | 4 | 4 | 2 | 2 |
| (11) Receiver antenna gain (dBi) | 8.00 | 8.00 | 8.00 | 8.00 |
| (11bis) Receiver array gain (dB) | 16.81 | 12.04 | 12.04 | 9.03 |
| (12) Cable, connector, combiner, body losses, etc. (enumerate sources) (dB) (feeder loss must be included for and only for uplink) | 3.00 | 3.00 | 3.00 | 3.00 |
| (13) Receiver noise figure (dB) | 5.00 | 5.00 | 5.00 | 5.00 |
| (14) Thermal noise density (dBm/Hz) | -174.00 | -174.00 | -174.00 | -174.00 |
| (15a) Receiver interference density for control channel (dBm/Hz) | -161.70 | -161.70 | -161.70 | -161.70 |
| (15b) Receiver interference density for data channel (dBm/Hz) | -165.70 | -165.70 | -165.70 | -165.70 |
| (16a) Total noise plus interference density for control channel = 10log (10(((13)+(14))/10) + 10(15a)/10) dBm/Hz | -160.96 | -160.96 | -160.96 | -160.96 |
| (16b) Total noise plus interference density for data channel = 10log (10(((13)+(14))/10) + 10(15b)/10) dBm/Hz | -164.03 | -164.03 | -164.03 | -164.03 |
| (17a) Occupied channel bandwidth for control channel (for meeting the requirements of the traffic type) (Hz) | 360000 | 360000 | 180000 | 180000 |
| (17b) Occupied channel bandwidth for data channel (for meeting the requirements of the traffic type) (Hz) | 10800000 | 1440000 | 720000 | 720000 |
| (18a) Effective noise power for control channel = (16a) + 10log ((17a)) dBm | -105.40 | -105.40 | -108.41 | -108.41 |
| (18b) Effective noise power for data channel = (16b) + 10log ((17b)) dBm | -93.70 | -102.45 | -105.46 | -105.46 |
| (19a) Required SNR for the control channel (dB) |  |  |  |  |
| (19b) Required SNR for the data channel (dB) |  |  |  |  |
| (20) Receiver implementation margin (dB) | 2.00 | 2.00 | 2.00 | 2.00 |
| (21a) HARQ gain for control channel (dB) | 0 | 0 | 0 | 0 |
| (21b) HARQ gain for data channel (dB) | 0.5 | 0.5 | 0.5 | 0.5 |
| (22a) Receiver sensitivity for control channel = (18a) + (19a) + (20) – (21a) dBm |  |  |  |  |
| (22b) Receiver sensitivity for data channel = (18b) + (19b) + (20) – (21b) dBm |  |  |  |  |
| (23a) Hardware link budget for control channel = (9a) + (11) + (11bis) - (22a) dB |  |  |  |  |
| (23b) Hardware link budget for data channel = (9b) + (11) + (11bis) – (22b) dB |  |  |  |  |
| Calculation of available pathloss |  |  |  |  |
| (24) Lognormal shadow fading std deviation (dB) | 7 | 7 | 8 | 8 |
| (25a) Shadow fading margin for control channel (function of the cell area reliability and (24)) (dB) | 7.56 | 7.56 | 8.45 (Indoor) 10.45 (Outdoor) | 8.45 (Indoor) 10.45 (Outdoor) |
| (25b) Shadow fading margin for data channel (function of the cell area reliability and (24)) (dB) | 4.48 | 4.48 | 5.13 (Indoor) 6.61 (Outdoor) | 5.13 (Indoor) 6.61 (Outdoor) |
| (26) BS selection/macro-diversity gain (dB) | 0 | 0 | 0 | 0 |
| (27) Penetration margin (dB) | 26.25 | 12.50 (Indoor) 9.00 (Outdoor) | 12.50 (Indoor) 9.00 (Outdoor) | 12.50 (Indoor) 9.00 (Outdoor) |
| (28) Other gains (dB) (if any please specify) | 0 | 0 | 0 | 0 |
| (29a) Available path loss for control channel = (23a) – (25a) + (26) – (27) + (28) – (12) dB |  |  |  |  |
| (29b) Available path loss for data channel = (23b) – (25b) + (26) – (27) + (28) – (12) dB |  |  |  |  |