3GPP TSG RAN WG1 #102-e					    	                     R1-2006358

e-Meeting, August 17th – 28th, 2020

# Source:		     ETRI

# Title:		 Discussion on timing relationships for NTN

# Agenda item:    8.4.1

# Document for:  Discussion

Introduction

In RAN#88e, the following has been agreed on the work item of NR NTN[1].

The work item aims to specify the enhancements identified for NR NTN (non-terrestrial networks) especially LEO and GEO with implicit compatibility to support HAPS (high altitude platform station) and ATG (air to ground) scenarios according to the following principles: - FDD is assumed for core specification work for NR-NTN. - NOTE: This does not imply that TDD cannot be used for relevant scenarios e.g. HAPS, ATG - Earth fixed Tracking area is assumed with Earth fixed and moving cells - UEs with GNSS capabilities are assumed. - Transparent payload is assumed.

With these elements and [2], we discuss UL timing advance in more detail.

Discussion

Based on [3], for a timing advance command received on uplink slot n and for a transmission other than a PUSCH scheduled by a RAR UL grant or a fallbackRAR UL grant, or a PUCCH with HARQ-ACK information in response to a successRAR, the corresponding adjustment of the uplink transmission timing applies from the beginning of uplink slot n+k+1 where k=Nslotsubframe,μ∙(NT,1+NT,2+NTA,max+0.5)/Tsf. NTA,max is the maximum timing advance value in msec that can be provided by a TA command field of 12 bits.

For NTN, if the transmission timing is adjusted based on the value known to the UE(estimated RTT by UE or common TA indicated by gNB), the NTA only indicates the differential delay from the value known to the UE. Thus, k of the above paragraphs can be changed as follows for NTN.

k=Nslotsubframe,μ∙(NT,1+NT,2+NTA,max+NTA,NTN,max+0.5)/Tsf where N TA,NTN,max can be the maximum value in msec for one of {0, eTASL, cTASL, cTAFL, (eTASL+cTAFL), (cTASL+cTAFL)}. eTA and cTA denote estimated TA by UE and common TA indicated by gNB, respectively. Subscripts SL and FL are RTT of service link and feeder link, respectively. If N TA,NTN,max is used, the corresponding parameter can be indicated by gNB.

In addition, due to the fast mobility of LEO, the TA value in slot n+k+1  may be inaccurate, particularly for transparent payload with high subcarrier spacing. For this, an implicit TA update method by UE may be considered with the application of the TAC.

Proposal 1:	The slot interval in which the value indicated by the timing advance command is applied to the UL transmission timing may be changed according to the definition of NTA. If the slot interval in which the value indicated by the timing advance command is applied increases, an analysis of the accuracy of the TA value at the time of application is required.

Based on [3] for type-1 random access procedure, if the UE does not detect the corresponding random access response and If requested by higher layers, the UE is expected to transmit a PRACH no later than N T,1+0.75 msec after the last symbol of the window, or the last symbol of the PDSCH reception.

If the interval between RACH occasions increases to resolve preamble ambiguity and new PRACH design, the PRACH transmission time may be greater than N T,1+0.75 msec.

Proposal 2:	When configuring the RACH occasion for NTN, it may be necessary to change the time interval of PRACH transmission due to the request of higher layers.

Based on [2]~[5], Koffset can be applied in various cases. Therefore, it is necessary to discuss how to configure Koffset for each case. For reporting HARQ-ACK on PUCCH, PDSCH-to-HARQ_feedback timing indicator is used to indicate the transmission slot index. In this case, Koffset may be reflected by expanding the bit size of PDSCH-to-HARQ_feedback timing indicator. For transmission of PUSCH scheduled by DCI, RAR, and fallback RAR, the Time domain resource assignment field value is used to indicate the transmission slot index. In this case, Koffset may be reflected by expanding the bit size of Time domain resource assignment field. Alternatively, the new Koffset parameter may be configured as a table similar to PDSCH-to-HARQ_feedback timing indicator or Time domain resource assignment field. Since the value of Koffset may be set to a different value in each case, it may be indicated by MAC-CE or DCI with the RRC configuration.

Proposal 3:	Koffset may be configured through an expansion of the resource indicator field of NR or a new parameter field. If it is configured with a new parameter field, it may be configured as a table similar to the resource indicator field of NR.

Conclusions

In this contribution, we present our considerations for timing relationships in NTN scenarios.  Based on the discussion, we make the following proposal.

Proposal 1:	The slot interval in which the value indicated by the timing advance command is applied to the UL transmission timing may be changed according to the definition of NTA. If the slot interval in which the value indicated by the timing advance command is applied increases, an analysis of the accuracy of the TA value at the time of application is required.

Proposal 2:	When configuring the RACH occasion for NTN, it may be necessary to change the time interval of PRACH transmission due to the request of higher layers.

Proposal 3:	Koffset may be configured through an expansion of the resource indicator field of NR or a new parameter field. If it is configured with a new parameter field, it may be configured as a table similar to the resource indicator field of NR.

References

- RP-201256, “Solutions for NR to support non-terrestrial networks (NTN)”, 3GPP TSG RAN meeting #88e

- 3GPP TR 38.821 V16.0.0 (2019-12), “Solutions for NR to support non-terrestrial networks (NTN) (Release 16)”

- 3GPP TS 38.213 V16.2.0 (2020-06), “NR; Physical layer procedures for control (Release 16)”

- 3GPP TS 38.214 V16.2.0 (2020-06), “NR; Physical layer procedures for data (Release 16)”

- 3GPP TS 38.212 V16.2.0 (2020-06), “NR; Multiplexing and channel coding (Release 16)”