3GPP TSG RAN WG1 #102			R1-2006366

e-Meeting, August 17-29, 2020

Agenda item:	8.13.1

Source:	            Xiaomi

Title:	Discussion on Cross-carrier scheduling from SCell to P(S)Cell

Document for:	Discussion

Introduction

A new work item on “New WID on NR Dynamic spectrum sharing (DSS)” was approved in RAN#86 and the objectives are as follows [1].

This work item is limited to FR1, and includes the following objectives for NR Dynamic Spectrum Sharing (DSS):

- PDCCH enhancements for cross-carrier scheduling including [RAN1, RAN2]

  - PDCCH of SCell scheduling PDSCH or PUSCH on P(S)Cell

  - Study, and if agreed specify PDCCH of P(S)Cell/SCell scheduling PDSCH on multiple cells using a single DCI

    - The number of cells can be scheduled at once is limited to 2

    - The increase in DCI size should be minimized

- Note: The total PDCCH blind decoding budget should not be changed as a result of this work

- Note: These enhancements are not specific to DSS and are generally applicable to cross-carrier scheduling in carrier aggregation

This contribution is focusing on cross-carrier scheduling from SCell to P(S)Cell.

Discussion

Cross-carrier scheduling from SCell to P(S)Cell

NR already supports cross-carrier scheduling from P(S)Cell to SCell or from SCell to SCell as shown in Figure 1. Existing CA with cross-carrier scheduling framework can mostly be re-used in this WI.

From coverage perspective, P(S)Cell is better than SCells normally. System information (e.g., RMSI, OSI, paging) transmitted on P(S)Cell should be scheduled by PDCCH on P(S)Cell. Cross-carrier scheduling from SCell to P(S)Cell, can be used for the purpose of unicast data scheduling.

- Cross-carrier scheduling from P(S)Cell to SCell          (b) Cross-carrier scheduling from SCell to SCell

Figure 1 Legacy multi-carrier scheduling

Proposal 1: Existing CA with cross-carrier scheduling framework should be re-used.

Potential works for enhanced cross-carrier scheduling

Legacy PDCCH search space design for cross-carrier scheduling can be re-used. Legacy DCI for cross-carrier scheduling also can be re-used mostly.  The cross-carrier scheduling configuration in CrossCarrierSchedulingConfig IE should be updated for supporting cross-carrier scheduling from SCell to P(S)Cell.

From coverage perspective, P(S)Cell is better than SCells normally. In order to guarantee PDCCH robust, there are two options:

Option 1: UE monitor UE specific PDCCHs on both P(S)Cell and SCell scheduling P(S)Cell.

Option 2: Fallback scheduling mechanism for cross-carrier scheduling from SCell to P(S)Cell is introduced.

Opt 1 has few impacts to specifications, but opt 2 can reduce the number of blind decoding attempts obviously.

Proposal 2: Fallback scheduling mechanism for cross-carrier scheduling from SCell to P(S)Cell should be considered.

Conclusions

In this contribution, we discuss Cross-carrier scheduling from SCell to P(S)Cell:

Proposal 1: Existing CA with cross-carrier scheduling framework should be re-used.

Proposal 2: Fallback scheduling mechanism for cross-carrier scheduling from SCell to P(S)Cell should be considered.

- References

- 3GPP RP-193260 “New WID on NR Dynamic spectrum sharing (DSS)”