3GPP TSG-RAN WG1 Meeting #102	Tdoc R1-2006368

eMeeting, August 17th – 28th, 2020

Agenda Item:	8.1.2.2

Source:	Ericsson

Title:	On inter-cell operation for mTRP

Document for:	Discussion

# 1	Introduction

The WID [1] objective 2 states:

- Enhancement on the support for multi-TRP deployment, targeting both FR1 and FR2:

  - ….

  - Identify and specify QCL/TCI-related enhancements to enable inter-cell multi-TRP operations, assuming multi-DCI based multi-PDSCH reception

  - ....

In this contribution we discuss a necessary proposal to achieve this objective from the WID.

# 2   Discussion

In our view, the Rel-16 multi-TRP schemes are re-used for the Rel-17 inter-cell m-TRP operations, there is no need to discuss new procedures or new multi-TRP schemes.

Rel-16 multi-TRP schemes are subject to the enhancements for inter-cell operations. No new multi-TRP schemes are discussed for this purpose.

The multi-TRP operation for multi-PDSCH reception relies on two abstractions:

- CSI report configurations, which includes descriptions of the reference signals that are used as targets for the measurements

- TCI states, which contains information about QCL:ed reference signals to simplify PDSCH reception at the UE.

Both the CSI report configurations and the TCI states are configured via RRC.

To facilitate inter-cell multi-TRP operation, we can make use of these two abstractions, by simply modifying the configuration of the CSI reports and the TCI states:

To facilitate inter-cell multi-TRP operation, the CSI report configurations and the TCI needs to be updated.

However, there is no need to introduce any fundamentally new procedure: we simply extend the existing procedures. Here, the CSI report configuration is needed to be able to report on SSB’s of non-serving cells. This will be handled in the multi-beam part of this WI. The measurement of SSBs has a longer time scale compared to measurements on CSI-RS, which is a fast operation. The fast operation is more critical to multi-TRP operation (while beam management is “slower”).

Hence, we suggest that in this multi-TRP part of the WI, we only need to consider the CSI-RS based measurements from non-serving cells and can assume that the discovery and selection of SSBs has already been done (and will be handled in multi-beam part of the WI, see [2]). Thereby, it is enough to modify the TCI states so that reference in non-serving cells can be used.

To provide a reference to a non-serving cell, the physical cell identity (PCI) should be included in the RRC configuration of the TCI states, at least for TCI states containing SSB:

Include a PCI in the TCI state (at least for TCI states referring to an SSB) to facilitate the use of reference signals from non-serving cells as QCL sources.

The PCI could be included in the TCI state itself, meaning that it would apply to the one or both QCL-info(s) that are part of the TCI state, or it can be part of the separate QCL-info. Both options would provide the required functionality.

By extending the TCI states with a PCI, it becomes possible to use SSBs from non-serving cells as QCL sources. This includes performing measurements on CSI-RS: any CSI-RS resource has a QCL source, and by allowing that QCL source to point to an SSB in a non-serving cell, it becomes possible to perform measurements on a CSI-RS, even when the CSI-RS is transmitted from a non-serving cell. It also includes reception of a PDSCH or a PDCCH from a non-serving cell, as a TCI state containing a TRS is used as the QCL source for DM-RS and the TRS has a TCI state with an SSB+PCI as the QCL source.

By introducing a PCI in a TCI state, the UE may be configured to perform measurements on CSI-RS transmitted from a non-serving cell.

In addition, since multi-PDCCH is part of the Rel.16 multi-TRP, there would be a need to consider the properties of the UL transmissions in particular PUCCH transmission towards a TRP of a non-serving cell. This may be performed as part of the unified TCI/common beam work in Release-17. In any case, there would be a need to introduce the PCI in the configuration that relates to

- Spatial relations.

- Pathloss reference RS for UL power control

Introduce a PCI in the configurations related to UL transmissions: spatial relations and pathloss reference RS.

# Conclusion

Based on the discussion in the previous section we propose the following:

Proposal 1	Rel-16 multi-TRP schemes are subject to the enhancements for inter-cell operations. No new multi-TRP schemes are discussed for this purpose.

Proposal 2	Include a PCI in the TCI state (at least for TCI states referring to an SSB) to facilitate the use of reference signals from non-serving cells as QCL sources.

Proposal 3	Introduce a PCI in the configurations related to UL transmissions: spatial relations and pathloss reference RS.

# References

RP-193133, New WID: Further enhancements on MIMO for NR, Samsung, RAN#86, Sitges, December 2019.

R1-2005842, Enhancements on multi-beam operation, Ericsson