3GPP TSG RAN WG1 #102-e	R1-2006377

e-Meeting, August 17th – 28th, 2020

Agenda Item:	7.2.3

Source: 	LG Electronics

Title: 	Text proposal on PDCCH monitoring in IAB

Document for:	Discussion and decision

Introduction

This contribution provides text proposal on PDCCH monitoring behavior for Rel-16 IAB-MT.

PDCCH monitoring for IAB-node MT

To indicate resource availability information for DU soft resource, DCI format 2_5 is introduced in Rel-16 IAB. An IAB-MT monitors PDCCH with DCI format 2_5 in CSS and/or USS, where CRC for DCI format 2_5 is scrambled by AI-RNTI.

However, in the current specification on TS 38.213, search space sets for monitoring DCI format with CRC scrambled by an AI-RNTI is not specified. As provided in Proposal 1, it should be added that for a DCI format with CRC scrambled by an AI-RNTI, an IAB-MT monitors PDCCH candidates in a Type3-PDCCH CSS set and a USS set.

Proposal 1: Adopt following text proposal for TS 38.213.

-------------------------------------------------------- Omitted -----------------------------------------------------

10.1   UE procedure for determining physical downlink control channel assignment

A set of PDCCH candidates for a UE to monitor is defined in terms of PDCCH search space sets. A search space set can be a CSS set or a USS set. A UE monitors PDCCH candidates in one or more of the following search spaces sets

-     a Type0-PDCCH CSS set configured by pdcch-ConfigSIB1 in MIB or by searchSpaceSIB1 in PDCCH-ConfigCommon or by searchSpaceZero in PDCCH-ConfigCommon for a DCI format with CRC scrambled by a SI-RNTI on the primary cell of the MCG

-     a Type0A-PDCCH CSS set configured by searchSpaceOtherSystemInformation in PDCCH-ConfigCommon for a DCI format with CRC scrambled by a SI-RNTI on the primary cell of the MCG

-     a Type1-PDCCH CSS set configured by ra-SearchSpace in PDCCH-ConfigCommon for a DCI format with CRC scrambled by a RA-RNTI, a MsgB-RNTI, or a TC-RNTI on the primary cell

-     a Type2-PDCCH CSS set configured by pagingSearchSpace in PDCCH-ConfigCommon for a DCI format with CRC scrambled by a P-RNTI on the primary cell of the MCG

-     a Type3-PDCCH CSS set configured by SearchSpace in PDCCH-Config with searchSpaceType = common for DCI formats with CRC scrambled by INT-RNTI, SFI-RNTI, TPC-PUSCH-RNTI, TPC-PUCCH-RNTI, TPC-SRS-RNTI, CI-RNTI, PS-RNTI, or AI-RNTI and, only for the primary cell, C-RNTI, MCS-C-RNTI, or CS-RNTI(s), and

- -     a USS set configured by SearchSpace in PDCCH-Config with searchSpaceType = ue-Specific for DCI formats with CRC scrambled by C-RNTI, MCS-C-RNTI, SP-CSI-RNTI, CS-RNTI(s), SL-RNTI, SL-CS-RNTI, SL-L-CS-RNTI, or AI-RNTI.

-------------------------------------------------------- Omitted -----------------------------------------------------