3GPP TSG RAN WG1 Meeting #102-e	R1-2006417

E-meeting, August 17 – 28, 2020

Agenda Item:	6.2.1

Source:	Huawei, HiSilicon

Title:	Corrections on transmission in preconfigured UL resources for eMTC

Document for:	Discussion and Decision

# Introduction

The CRs on eMTC have been approved in RAN#88e [1]. This contribution discusses the corrections to the specs.

# Sequence-group hopping

As shown in TS 36.211 (see below), in legacy the sequence-group hopping for PUSCH can be disabled for a certain UE through the higher-layer parameter Disable-sequence-group-hopping despite being enabled on a cell basis unless the PUSCH transmission corresponds to a Random Access Response Grant or a retransmission of the same transport block as part of the contention based random access procedure.

However, since Disable-sequence-group-hopping is not included in PUR-Config, the current specification is incorrect. Generally, there are two alternatives to solve this issue:

Alt 1: Send LS to RAN2 to include Disable-sequence-group-hopping in PUR-Config, and the RAN1 specfication does not need to be updated.

Alt 2: Modify RAN1 specification (see TP below) so that PUR is treated similar to RACH.

Proposal 1: RAN1 needs to decide how to address sequence-group hopping for PUR PUSCH.

-----------------------------Start of Text Proposal to TS 36.211------------------------------------------------------

#### 5.5.1.3	Group hopping

The sequence-group number  in slot  is defined by a group hopping pattern  and a sequence-shift pattern  according to

There are 17 different hopping patterns and 30 different sequence-shift patterns. Sequence-group hopping can be enabled or disabled by means of the cell-specific parameter Group-hopping-enabled provided by higher layers. Sequence-group hopping for PUSCH can be disabled for a certain UE through the higher-layer parameter Disable-sequence-group-hopping despite being enabled on a cell basis unless the PUSCH transmission corresponds to a Random Access Response Grant or a retransmission of the same transport block as part of the contention based random access procedure or the PUSCH transmission corresponds to preconfigured uplink resources.

-----------------------------End of Text Proposal to TS 36.211------------------------------------------------------

# Conclusion

The contribution discusses some remaining issues of PUR, with following proposal:

Proposal 1: RAN1 needs to decide how to address sequence-group hopping for PUR PUSCH.

# References

- RP-200698, “Corrections to Additional MTC Enhancements for LTE”, RAN1, RAN#88e.