3GPP TSG-RAN WG1 Meeting #102	R1-2006431

e-Meeting, August 17th – 28th, 2020

Source:	Nokia, Nokia Shanghai Bell

Title: 		On remaining NR TEI issues

Agenda item:	7.2.12

Document 		Discussion and Decision

# Introduction

In this contribution we provide a discussion on newly endorsed TEI issue

- TRS BW for R16

# TRS BW for R16

RAN approved document RP-201333 with the following message

- Task RAN1 (cc: RAN2) to define TRS bandwidth sizes of 28, 32, 36, 40, 44, 48 RBs. - All TRS configured for a given BWP with the newly defined TRS bandwidth sizes for a UE span the same set of RBs. - All allocated PDSCH RBs are confined within the bandwidth spanned by TRS + up to 3RBs beyond either/both of the highest RB and lowest RB of the TRS. - Only supported for 10MHz UE channel bandwidth, 52 RB BWP size, and 15kHz SCS, in FDD bands. - Note: No new performance requirement on UE is introduced here. - A “per-band” UE capability is to be defined for this optional UE feature, that indicates per band support for one of: - “All newly defined TRS bandwidth sizes” - “All newly defined TRS bandwidth sizes except 28 RB size” - Introduce from Release 16 as part of TEI16

Based on above RAN agreement which reassembles Alternative 1 previously discussed in RAN1, we provide a TP for TS38.214.

Proposal: Adopt the following TP for TS38.214 to address RP-201333 in RAN1

##### 5.1.6.1.1 CSI-RS for tracking <unchanged text parts omitted> Each CSI-RS resource, defined in Clause 7.4.1.5.3 of [4, TS 38.211], is configured by the higher layer parameter NZP-CSI-RS-Resource with the following restrictions: - the time-domain locations of the two CSI-RS resources in a slot, or of the four CSI-RS resources in two consecutive slots (which are the same across two consecutive slots), as defined by higher layer parameter CSI-RS-resourceMapping, is given by one of - , , or for frequency range 1 and frequency range 2, - , , , , , or for frequency range 2. - a single port CSI-RS resource with density given by Table 7.4.1.5.3-1 from [4, TS 38.211] and higher layer parameter density configured by CSI-RS-ResourceMapping. - if carrier Ngridsize,μ=52, NBWP,isize=52, μ=0 and the carrier is configured in paired spectrum, the bandwidth of the CSI-RS resource, as given by the higher layer parameter freqBand configured by CSI-RS-ResourceMapping, is X resource blocks, where X ≥ 28 resources if the UE indicates allTRS for the trs-BandwidthSize-15kHz capability and X ≥ 32 if the UE indicates allTRS-Except28PRB for the trs-BandwidthSize-15kHz capability; in these cases, if the UE is configured with CSI-RS comprising X<52 resource blocks, the UE does not expect to receive PDSCH in resource blocks of the active BWP which are more than 3 RBs above the highest resource block of the CSI-RS or more than 3 RBs below the lowest resource block of the CSI-RS and all CSI-RS resource configurations shall span the same set of resource blocks. Otherwise, the bandwidth of the CSI-RS resource, as given by the higher layer parameter freqBand configured by CSI-RS-ResourceMapping, is the minimum of 52 andNBWP,isize resource blocks, or is equal toNBWP,isize resource blocks. For operation with shared spectrum channel access, freqBand configured by CSI-RS-ResourceMapping, is the minimum of 48 andNBWP,isize resource blocks, or is equal toNBWP,isize resource blocks. <unchanged text parts omitted>

# Conclusion

In this contribution we provided TP for TS38.214 to reflect agreement from RP-201333.