3GPP TSG-RAN WG1 Meeting #102-e	R1-2006442

e-Meeting, August 17th – 28th, 2020

Agenda Item:	7.2.4.1

Source:	Ericsson

Title:	Discussion on maximum data rate for SL

Document for:	Discussion, Decision

# 1	Introduction

In RAN1#101-e meeting, RAN1 received a LS from RAN2 in [1] requesting RAN1 to define the maximum data rate for NR sidelink. In this contribution, we discussion the calculation of maximum data rate for SL and the corresponding draft reply LS can be found [2].

# 2 	Discussion

So far TR 38.306 only defines the formula to calculate the maximum data rate of UL and DL, which is given as follows:

wherein:

- J is the number of aggregated component carriers in a band or band combination.

- Rmax = 948/1024.

- vLayers is the maximum number of supported layers, which is 2.

- Qm is the maximum supported modulation order given by higher layer parameter.

- μ is the numerology (as defined in TS 38.211).

- f is the scaling factor.

- Tsμ is the average OFDM symbol duration in a subframe for numerology μ, i.e. Tsμ=10-314⋅2μ

  - Note that normal cyclic prefix is assumed

- NPRBBW,μ is the maximum RB allocation in bandwidth BW with numerology μ, as defined in 5.3 TS 38.101-1 and 5.3 TS 38.101-2, where BW is the UE supported maximum bandwidth in the given band

- OH is the overhead which is determined according to Proposal 2 below.

In this regard, we have the following proposals:

Maximum data rate formula should be modified considered single carrier operation.

Overhead for SL should be specified to calculate the maximum data rate for FR1 and FR2 operation, considering following assumptions:

- PSCCH overhead corresponds to 2 symbols and 10 PRBs.

- PSFCH resource in a slot is zero.

- 2 symbols DMRS overhead in a slot, no CSI-RS and PT-RS overhead.

# 3	Conclusion

Based on the discussion in the previous sections we propose the following:

Proposal 1	Maximum data rate formula should be modified considered single carrier operation.

Proposal 2	Overhead for SL should be specified to calculate the maximum data rate for FR1 and FR2 operation, considering following assumptions:

- PSCCH overhead corresponds to 2 symbols and 10 PRBs.

- PSFCH resource in a slot is zero.

- 2 symbols DMRS overhead in a slot, no CSI-RS and PT-RS overhead.

# 4	References

R1-2003258, “LS on UE capability”, RAN2.

R1-2006432, "[Draft] Reply LS on SL UE capability", Ericsson, RAN1#102-e eMeeting, August 17-28, 2020.