3GPP TSG-RAN WG1 Meeting #102-e	R1-2006466

e-Meeting, August 17th – 28th, 2020

Agenda Item:	8.4.4

Source:	Ericsson

Title:	On other enhancements for NTN

Document for:	Discussion

# 1	Introduction

The work item on solutions for NR to support non-terrestrial networks (NTN) was approved at RAN#86 and the work item description is updated in [1]. One objective is to specify the following topics if beneficial and needed:

- Enhancement on the PRACH sequence and/or format and extension of the ra-ResponseWindow duration (in the case of UE with GNSS capability but without pre-compensation of timing and frequency offset capabilities) [RAN1/2].

- Feeder link switch [RAN2,RAN1]

- Beam management and Bandwidth Parts (BWP) operation for NTN with frequency reuse [RAN1/2]

  - Including signalling of polarization mode

We consider service link switch to belong to beam management as agreed in Section 6.24 in TR 38.821 [2]. In this contribution, we discuss these topics for NTN.

# 2	Random access

The necessity of potential enhancement on the PRACH sequence and/or format and extension of the ra-ResponseWindow duration is conditional on the case of UE with GNSS capability but without pre-compensation of timing and frequency offset capabilities. Therefore, RAN1 should first identify valid scenarios, where GNSS-equipped UEs cannot perform timing and frequency pre-compensation for uplink synchronization.

RAN1 to identify scenarios where GNSS-equipped UEs cannot perform timing and frequency pre-compensation for uplink synchronization.

The following  discussion on PRACH present our detailed views on random access in NTN for the scenarios, where timing/frequency pre-compensation cannot be assumed.

At RAN1#99, it was agreed that at least for the case without pre-compensation of timing and frequency offset, at least the following options for enhanced PRACH formats and/or preamble sequences can be considered:

- Option-1: A single Zadoff-Chu sequence based on larger SCS, repetition number

- Option-2: A solution based on multiple Zadoff-Chu sequences with different roots

- Option-3: Gold/m-sequence as preamble sequence with additional process, e.g., modulation and transform precoding

- Option-4: A single Zadoff-Chu sequence with combination of scrambling sequence

This is a typical case where multiple alternatives are proposed to address the same objective. It is important to limit the potential normative work to the most promising option to minimize the efforts from specification and testing to NW/UE implementations. Otherwise, it would just adversely result in many “NR universes” and intractable bundles of features.

NR NTN features (including PRACH) should have synergies with NR terrestrial solutions as much as possible to help NTN benefit from economies of scale.

RAN1 to limit the potential normative work of enhanced PRACH for NTN to the most promising option to minimize the efforts from specification and testing to NW/UE implementations.

To facilitate limiting the scope of enhanced PRACH in NTN, we provide a comparison of the options in the table below.

Table 1: Comparison of different new PRACH design options

| Option | Description | Meet the design target for facilitating both UL timing and freq estimation? | Specification effort | Implementation complexity |
| --- | --- | --- | --- | --- |
| Option 1 | One ZC sequence with larger SCS, repetition number | No - A single ZC sequence is not sufficient | Small | Small |
| Option 2 | Multiple ZC sequences with new root pairs | Yes - Mathematical properties of ZC sequences can be exploited for time and freq estimation | Large - Significant efforts required to specify new root pairs | Moderate - Moderately modified implementation at NW/UE would be required |
|  | Multiple ZC sequences with an existing root and a complex-conjugate root | Yes - Mathematical properties of ZC sequences can be exploited for time and freq estimation | Small - The change would be merely to request transmission of a second ZC sequence that is the complex conjugate of an existing ZC sequence | Small - Slightly modified implementation at NW/UE would be required |
| Option 3 | Gold/m-sequence with additional process, e.g., modulation and transform precoding | Unclear - It requires multiple frequency hypotheses at gNB, which is not desirable | Extremely large - Deviate from well-established ZC sequences - With new types of sequences, there is huge impact on specification such as sequence selection, PRACH format design (SCS, CP, GP, etc.), PRACH occasion configuration, many new RRC parameters, etc. | Extremely Large - Completely new implementation at NW/UE would be required - In particular, it requires multiple frequency hypotheses at gNB |
| Option 4 | One ZC sequence with scrambling sequence based on gold/m-sequences | Unclear - It requires multiple frequency hypotheses at gNB, which is not desirable | Extremely large - Since it relies on scrambling based on Gold/m-sequences, specification effort is similar to that required for Option 3. | Extremely Large - New implementation at NW/UE would be required - In particular, it requires multiple frequency hypotheses at gNB |

Based on the comparison of different new PRACH design options in Table 1, we make the following proposal.

RAN1 not to deviate from Zadoff-Chu sequences in enhancing PRACH for NTN.

In Annex, we provide evaluation results on a PRACH design using two ZC sequences with an existing root and a complex-conjugate root. The results show that the considered PRACH design can sufficiently address the scenarios, where GNSS-equipped UEs cannot perform timing and frequency pre-compensation for uplink synchronization.

Simulation results show that the proposed PRACH design using two ZC sequences with an existing root and a complex-conjugate root, can provide satisfactory PARCH detection performance with sufficient time/frequency estimation accuracy for uplink synchronization, in case GNSS-equipped UEs cannot perform the pre-compensation task.

# 3	Feeder link switch

The impact of feeder link switch on higher layer protocols has been extensively discussed in RAN2/RAN3, as captured in TR 38.821. As the satellite moves, it is possible that the feeder link switches from one satellite gateway to another while the service link remains the same, i.e., satellite is still visible to the UEs served before the switch. This scenario is illustrated in Figure 1, where the satellite feeder link switches from Gateway 1 (T1) to Gateway 2 (T2).

Figure 1: Feeder link switches while service link remains the same

For the bent-pipe architecture, a feeder link switch usually means that the serving RAN node will also change with the gateway. In TR 38.821, the following scenarios were considered for bent-pipe LEO satellites.

Soft feeder link switch where the satellite can simultaneously support two feeder links: For this case, handover-based solutions were proposed to enable service continuity despite feeder link switch. The key idea is that the satellite supports simultaneous transmissions of both gNB’s signals (which use different PCIDs) during the switch to enable a smooth handover. This is illustrated in Figure 2.

Figure 2: Soft feeder link switch

The existing layer 1 specification already supports transmitting multiple cells on different time/frequency resources. This leads us to the following observation:

For a soft feeder link switch scenario, there are no major RAN1 issues for the solution identified in TR 38.821.

Hard feeder link switch where the satellite can only support one feeder link at a time: For this case, two possible solutions were identified. One option was based on conditional handover with accurate timing control. The other option was to drop the UEs altogether and use enhanced connection re-establishment procedures.

Before discussing RAN1 aspects, it needs to be determined if the hard feeder link switch is a likely scenario in practice. Typically, satellites have the capability to connect to multiple gateways by using multiple antennas.

In TR 38.821, the following two cases were considered for bent-pipe LEO architecture:

- Case 1: Different gNB’s before and after the switch. In this case, the target gateway after feeder link switch is served by a different gNB compared to the source gateway.

- Case 2: Same gNB before and after the switch. In this case, the gateways before and after the switch are connected to the same gNB.

Satellite gateways are deployed on locations far apart from one another. Usually, a gNB will be either co-located with a gateway or connected to gateway close to it. The validness of the scenario where the same gNB is connected to multiple satellite gateways is questionable.

If the scenarios, (1) hard feeder link switch, (2) same gNB before and after the switch, are not considered in Rel-17, it appears that no RAN1 work is needed for feeder link switch.

Satellites typically have the capability to connect to multiple gateways by using multiple antennas.

For feeder link switch, the validness of the scenario where the same gNB is connected to multiple satellite gateways is questionable. In contrast, the feeder link switch involving different gNB’s before and after the switch is more typical.

It is up to RAN2 to discuss if the “hard feeder link switch” scenario, where the satellite can only support one feeder link at a time, should be considered in Rel-17.

It is up to RAN2 to discuss if the feeder link switch involving same gNB before and after the switch should be considered in Rel-17.

# 4	Beam management, BWP operation, and signaling of polarization

## 4.1	Beam management

For LEO earth-fixed beams NR needs to manage a frequent switching of the service link, from a first serving satellite to a second serving satellite. Since this switching of the service links associated with different satellites can occur as frequently as about once every 1-10 minutes depending on e.g. target operational elevation angle, a solution for managing this scenario for UEs in RRC connected is required. To support this scenario, handover-based mobility management can be used. There are also proposals that NR beam management framework can be used to reduce the overhead associated with handover.

RAN1 to discuss the scope of beam management, i.e., whether NR beam management framework (TCI state and spatial relations) should be restricted within the same satellite or support the switching of the service links associated with different satellites.

Satellite devices are in many cases equipped with highly directive beams with gains exceeding 40 dBi with 3-dB beam widths of around 1º. Due to this it will be challenging (if not impossible) for a satellite device to autonomously and rapidly detect reference signals, e.g. SSBs, from target satellite, point its receive and transmit beam towards the new satellite. To facilitate the switch and finding the new satellite we believe that the network, i.e. source satellite, should indicate the ephemeris of the target satellite. If NR beam management framework can be used to support the switching of the service links associated with different satellites, the TCI state and spatial relations can then be associated with the satellite ephemeris.

A first satellite providing coverage before a service link switch should assist UEs in RRC connected with signaling of the ephemeris of the second satellite providing coverage after the switch.

Finally, we believe that a UE should refrain from initiating an RRC connection towards a satellite that is just about to hand over its service link to a next satellite. Doing so may reduce unnecessary network signaling, reduce PRACH transmissions from UEs not supporting GNSS in RRC connected, and obviously eliminate the service link switch which may pose a weak link in an RRC connection. To support this functionality the network may indicate to the UEs in RRC idle that it will soon initiate a service link switch.

The NR network should be able to indicate the timing of the service link switch to UEs in RRC idle and RRC inactive.

## 4.2	BWP operation

During Release 16, the concept of using BWPs to enable a frequency reuse was discussed. It was argued that mapping different BWPs to different parts of the system bandwidth and different beams would allow layer 1 based mobility within a large cell. The large cell would be defined by an anchor beam transmitting BWP#0 and comprising multiple smaller spotbeams each associated with a BWP dedicated for data transmission as illustrated below. Optionally also the anchor beam can be narrow and be swept over the area defining the cell.

Figure 3: An illustration of using BWP for frequency reuse.

At a first glance this may appear to be an appealing concept, but we expect that it will negatively impact the supported system capacity. Release 15 supports mobile terminated (MT) and mobile originated (MO) system access using the initial BWP#0. System access is already expected to pose a capacity bottleneck. Increasing the cell size will not improve the situation. If a narrow anchor beam is used for BWP#0 and is swept over the cell area, then a paging transmission needs to be repeated for every new pointing direction of the beam. It is of course possible to discuss an enabling of MT/MO access also for BWP#1-#3, but this will then raise new questions e.g. on how idle mode mobility between these BWPs are to be supported.

In addition, using BWPs to enable a frequency reuse can already be supported by existing NR specification. It is a choice of network configuration and implementation. There is no need for additional enhancements.

BWP based frequency reuse will negatively impact the supported system capacity.

Using BWPs to enable a frequency reuse can already be supported by existing NR specification. It is a choice of network configuration and implementation.

RAN1 to conclude that there is no need for additional enhancements for using BWPs to enable a frequency reuse.

## 4.3	Signaling of polarization

Most satellite networks make use of polarization. Right hand and left hand circular polarizations (RHCP, LHCP) are popular choices as they travel unaffected through the atmosphere. It was observed in TR 38.821 that polarization can be used in cell planning as a tool to reduce inter-cell interference. It is also useful for supporting rank-2 MIMO, and for supporting co-existence in shared frequency bands. The last item is a highly relevant aspect due to the many plans to deploy mega NGSO constellations in shared Ka-, Ku- and V-bands. High profile constellations such as the one provided by Starlink will e.g. make use of the same spectrum. Not only is the spectrum sharing important between these NGSO constellations, but it is also required between them and existing GEO deployments. ITU Recommendation ITU-R S.1431 provides methods to enhance sharing between NGSO FSS systems and e.g. points out that two systems can share the same spectrum over the same geographical region by means of using different polarizations. Supporting such cases is important for NR NTN to be an attractive alternative for NGSO operators.

NGSO and GSO constellations may make use of shared spectrum e.g. in Ka-, Ku- and V-bands. The requirement on efficient coordination is high, and polarization is an important tool.

NR NTN should support configuration of DL and UL transmit polarization including RHCP and LHCP.

In pure LOS conditions the satellite transmit polarization should be detectable by a UE with high precision. In theory it could be required that a UE detects the DL polarization, and makes use of the same polarization in the uplink. In practice, however, there are cases where reflections in buildings, landscape and water will impair the polarization before it reaches the UE. Assume e.g. that a satellite device receives a radio wave that was reflected in a window at the Brewster angle, at which the polarization component aligned with the direction of the wave cancels out. Even though the signal was circularly polarized at the transmitter, the receiving UE will only observe a linearly polarized wave, and it will be impossible for the UE to determine the polarization used by the satellite. To support a well-defined polarization both in the DL and the UL, we believe that the network should at least broadcast to the UEs the UL transmit polarization. With this signaling in place, we believe that it is reasonable to also support signaling of the transmit polarization used for the DL.

In some cases, a UE cannot be expected to reliably detect the used DL polarization.

The gNB should indicate the DL and UL transmit polarizations used in NR NTN using e.g. broadcast signaling.

It is further expected that a mega NGSO constellation will be utilized to serve multiple types of devices such as reduced capability devices, normal handhelds, vehicular devices, fixed devices, among others. Some of these devices will use linear polarization, some will use circular polarization, and others may support both. For optimal performance, it will be necessary for network to know the polarization capability of these devices.

NTN UE should report its polarization capability to the network.

# Conclusion

In the previous sections, we discuss enhancements for NTN that fall in the category of “specify if beneficial and needed”. We made the following observations:

Observation 1	NR NTN features (including PRACH) should have synergies with NR terrestrial solutions as much as possible to help NTN benefit from economies of scale.

Observation 2	Simulation results show that the proposed PRACH design using two ZC sequences with an existing root and a complex-conjugate root, can provide satisfactory PARCH detection performance with sufficient time/frequency estimation accuracy for uplink synchronization, in case GNSS-equipped UEs cannot perform the pre-compensation task.

Observation 3	For a soft feeder link switch scenario, there are no major RAN1 issues for the solution identified in TR 38.821.

Observation 4	Satellites typically have the capability to connect to multiple gateways by using multiple antennas.

Observation 5	For feeder link switch, the validness of the scenario where the same gNB is connected to multiple satellite gateways is questionable. In contrast, the feeder link switch involving different gNB’s before and after the switch is more typical.

Observation 6	BWP based frequency reuse will negatively impact the supported system capacity.

Observation 7	Using BWPs to enable a frequency reuse can already be supported by existing NR specification. It is a choice of network configuration and implementation.

Observation 8	NGSO and GSO constellations may make use of shared spectrum e.g. in Ka-, Ku- and V-bands. The requirement on efficient coordination is high, and polarization is an important tool.

Observation 9	In some cases, a UE cannot be expected to reliably detect the used DL polarization.

Based on the discussion in the previous sections we propose the following:

Proposal 1	RAN1 to identify scenarios where GNSS-equipped UEs cannot perform timing and frequency pre-compensation for uplink synchronization.

Proposal 2	RAN1 to limit the potential normative work of enhanced PRACH for NTN to the most promising option to minimize the efforts from specification and testing to NW/UE implementations.

Proposal 3	RAN1 not to deviate from Zadoff-Chu sequences in enhancing PRACH for NTN.

Proposal 4	It is up to RAN2 to discuss if the “hard feeder link switch” scenario, where the satellite can only support one feeder link at a time, should be considered in Rel-17.

Proposal 5	It is up to RAN2 to discuss if the feeder link switch involving same gNB before and after the switch should be considered in Rel-17.

Proposal 6	RAN1 to discuss the scope of beam management, i.e., whether NR beam management framework (TCI state and spatial relations) should be restricted within the same satellite or support the switching of the service links associated with different satellites.

Proposal 7	A first satellite providing coverage before a service link switch should assist UEs in RRC connected with signaling of the ephemeris of the second satellite providing coverage after the switch.

Proposal 8	The NR network should be able to indicate the timing of the service link switch to UEs in RRC idle and RRC inactive.

Proposal 9	RAN1 to conclude that there is no need for additional enhancements for using BWPs to enable a frequency reuse.

Proposal 10	NR NTN should support configuration of DL and UL transmit polarization including RHCP and LHCP.

Proposal 11	The gNB should indicate the DL and UL transmit polarizations used in NR NTN using e.g. broadcast signaling.

Proposal 12	NTN UE should report its polarization capability to the network.

# References

RP-201256, “Solutions for NR to support non-terrestrial networks (NTN),” 3GPP TSG RAN #88e, June 2020.

TR 38.821, Solutions for NR to support non-terrestrial networks.

# Annex: Evaluation of a New PARCH design for NTN

## A1	Design considerations

- It is known that there are several peaks in the ambiguity function of ZC sequences in the Delay-Doppler plane, leading to many timing and Doppler ambiguities. We can understand the timing and frequency offset ambiguities of ZC sequences by examining their theoretical properties. To this end, we introduce the following notation.

- NZC: the length of a ZC sequence

- u: the root of a ZC sequence, and 0<u<NZC

- p: the inverse modulo NZC of u, i.e., p*u mod NZC=1, and 0<p<NZC

- fSC: the subcarrier spacing of an OFDM signal

- Δf: the frequency offset between transmitted and received signals

- n0: the delay of received signals relative to the transmitted signal

Let us consider the following form of ZC sequences:

xun=exp-jπunn+1NZC, n=0, 1,…, NZC-1.

If NZC is prime, each u is associated with a unique inverse modulo NZC. It can be shown that if k=Δf/fSC (and for simplicity k is assumed to be an integer), the peak of cross correlation of the transmitted and received signals is located at the position of n0+kp mod NZC. Clearly, both delay and frequency shift cause cyclic shift in the received ZC sequences, resulting in a composite cyclic shift from which the effect of delay cannot be separated from the effect of frequency shift.

The above analysis also sheds light on how to design a PRACH format to resolve the timing and frequency offset ambiguities. Intuitively, two equations can be used to solve for two unknowns (delay and frequency offset). In particular, if a transmitter sends two signals based on two ZC sequences (that have different properties), the receiver can resolve the timing and frequency offset ambiguities by processing the two received signals.

For example, for 2 ZC sequences with roots u and -u (which is the same as NZC-u) respectively, the peak of cross correlation of the transmitted and received signals locate at two positions:

- Position 1: s1=n0+kp mod NZC

- Position 2: s2=n0-kp mod NZC

In this case, the second ZC sequence can be treated as the complex conjugate of the first ZC sequence, as shown in Figure 4. With two equations, the delay n0 can be estimated as

n0=s1+s2 mod NZC2

Once the delay is estimated, the frequency offset can then be readily estimated.

Note that for simplicity we assume that the frequency offset is an integer multiple of the subcarrier spacing. For more general case, it can be shown that the squared autocorrelation of ZC sequence is given by

γn0,Δf 2=sincΔffSC-un02.

Here the sinc function is defined as sincx=1Nsin(πx)sin(πx/N). Then, by processing the two received ZC sequences with roots u and -u respectively, we could estimate the delay n0 and the frequency offset Δf accordingly.

Figure 4: PRACH format with 2 ZC sequences, where the second ZC sequence is the complex conjugate of the first ZC sequence.

Note that the PRACH format illustrated in Figure 4 has minimal specification impact. The transmission of the first ZC sequence follows an existing NR PRACH format, and the change would be merely to request one additional transmission of a second ZC sequence that is the complex conjugate of the first ZC sequence.

## A2	Evaluation results on the proposed new PRACH format

We evaluated the performance of the proposed PRACH format with the simulation parameters tabulated below.

Table 2: NR PRACH format for link level simulation

| PRACH format | LRA | SCS | BW | TSEQ | TCP | TGP |
| --- | --- | --- | --- | --- | --- | --- |
| 0 | 839 | 1.25 kHz | 1.08 MHz | 24576 Ts | 3168 TS | 2975 TS |
|  | Note: TS=130720 ms |  |  |  |  |  |

Table 3: Parameters for NR PRACH link level simulations

| Antenna configuration | 1 TX, 2 RX |
| --- | --- |
| Band | S band |
| Carrier frequency | 2 GHz |
| Number of UEs | 2 |
| Channel | TDL-D 90° |
| UE power offset | 3 dB |
| Differential delay | Uniformly drawn from [0, 95] μs (Note: this is about the same range as the CP of PRACH format 0. If the CP is extended to e.g. 800 μs, the maximum timing error can be as large as ~800 μs and can be further extended by using multiple timing hypotheses.) |
| Doppler spread | 1 Hz |
| Maximum differential Doppler shift δmax | - 100 kHz - 500 kHz |
| Differential Doppler Shift | Uniformly drawn from [-δmax +δmax] |
| False alarm rate | 0.1% (or 0.164% for each preamble) |
| Number of preambles | 64 |
| Maximum timing estimation error for MDR calculation | 2.55 μs |

The preamble sequences, the differential delay, and the differential Doppler shift are chosen randomly for each UE (independently of the other UE). Specifically, in each simulation run, the root of the first ZC sequence (PRACH format 0 with 1.25 kHz subcarrier spacing) is chosen randomly from the allowed set of sequences. The PRACH missed detection rate (MDR) for both the UEs is plotted in Figure 5 for two different ranges of carrier frequency offset (CFO). We observe that 1% MDR is achieved with an SNR of around -7.3 dB for the stronger UE and -6.6 dB for the weaker UE, given a CFO range of +/- 100 kHz. For the extreme CFO range of +/- 500 kHz, 1% MDR is achieved with SNRs of -4.5 dB and -2.9 dB for the stronger UE and the weaker UE, respectively. In short, the proposed PRACH design helps meet the MDR performance requirements despite large CFOs.

Figure 5: Missed detection rate vs. SNR for both UEs for two different CFO ranges.

In Figure 6, we plot the CDF of the frequency estimation error when the CFO is randomly chosen from [-100 kHz 100 kHz]. We obtain the plot for the SNR corresponding to 1% MDR (which is around -7.3 dB for the stronger UE and -6.6 dB for the weaker UE). For a wide range of SNR, the CFO estimation error is well within half of the PRACH subcarrier spacing (i.e., 625 Hz in this case). For example, in Figure 6, the CFO estimation error is within +/-500 Hz with a probability of around 0.98 for the weaker UE and >0.99 for the stronger UE. In Figure 7, we plot similar results for the extreme case where the CFO is randomly chosen from [-500 kHz, 500 kHz], which is not typical even in LEO NTNs. We obtain the plot for the SNR corresponding to 1% MDR (which is around -4.5 dB for the stronger UE and -2.9 dB for the weaker UE). Despite the extremely large CFO, the CFO estimation error is limited to within +/-500 Hz with a probability of around 0.90 for the weaker UE and 0.94 for the stronger UE. In summary, the results show that the proposed PRACH design can cope with the large differential Doppler shifts expected in a LEO NTN system.

Figure 6: CDF of CFO estimation error for both UEs at SNR corresponding to 1% MDR under a CFO range [-100 kHz, 100 kHz].

Figure 7 CDF of CFO estimation error for both UEs at SNR corresponding to 1% MDR under a CFO range [-500 kHz, 500 kHz].