3GPP TSG-RAN WG1 Meeting #102-e	R1-2006471

e-Meeting, August 17th – 28th 2020

Agenda Item:	6.2.1

Source:	Ericsson

Title:	Multi-TB maintenance issues for LTE-MTC

Document for:	Discussion, Decision

# Issue #1: Early UL termination in 36.213 clause 8.0

In 36.213 V16.2.0 clause 8.0, it should be clarified that in case of multi-TB transmission with early termination of UL transmission, the ACK transmitted over MPDCCH concerns all TBs.

A BL/CE UE configured with mpdcch-UL-HARQ-ACK-FeedbackConfig shall upon detection on a given serving cell of an MPDCCH with DCI format 6-0A/6-0B intended for the UE in the UE-specific search space indicating HARQ-ACK corresponding to transport block(s) associated to HARQ process(es) in the most recent PUSCH transmission with N>1, drop the remaining PUSCH transmission(s) (if any) corresponding to the transport block(s) no later than subframe n+k, where - subframe n is the last subframe in which the MPDCCH is transmitted; and - for FDD, k = 4; - for TDD the value of k is given in Table 8-2 for the corresponding TDD UL/DL configuration; If the value of k is not given in Table 8-2 for subframe n, denote subframe n' as the first downlink/special subframe which has a value in Table 8-2 after subframe n, and substitute n with n' in the above procedure; - value of is determined by the repetition number field in the corresponding DCI associated with the most recent PUSCH transmission; - if the UE is configured with higher layer parameter ce-PUSCH-SubPRB-Config-r15, and the PUSCH resource assignment in the corresponding DCI associated with the most recent PUSCH transmission is using uplink resource allocation type 5, where is defined in [3] and is determined according to procedure in subclause 8.1.6, otherwise.

- Clarify that ACK concerns all TBs in 36.213 clause 8.0 according to the above TP.

# Issue #2: Missing ‘else’ in 36.213 clause 8.6.1

In 36.213 V16.2.0 clause 8.6.1, an ‘else’ is missing in one place:

- if NTB=1 is indicated by the corresponding DCI, for the TB is determined by the 'Redundancy version' in the ‘Scheduling TBs for Unicast’ field in DCI format 6-0A - else if NTB=2 is indicated by the corresponding DCI, and the HARQ process IDs for each of the scheduled TBs are h1 and h2 (h1<h2), of the scheduled TB with HARQ process ID h1 is determined by the ‘Redundancy version for TB 1’ in the ‘Scheduling TBs for Unicast’ field in DCI format 6-0A, and of the scheduled TB with HARQ process ID h2 is determined by - if the UE is configured with higher layer parameter pusch-HoppingConfig set to’on’ and the repetition number field in the DCI indicates PUSCH repetition, the ‘Redundancy version for TB 1’ in the ‘Scheduling TBs for Unicast’ field in DCI format 6-0A - otherwise the ‘Redundancy version for TB 2’ in the ‘Scheduling TBs for Unicast’ field in DCI format 6-0A - else if NTB = 4 or 6, for all schedueld TBs - else - if the UE is configured with higher layer parameter pusch-HoppingConfig set to 'on' and the repetition number field in the DCI indicates PUSCH repetition, for all TBs - otherwise of all TBs is determined by the ‘Redundancy version for all TBs’ in the ‘Scheduling TBs for Unicast’ field in DCI format 6-0A.

- Insert the missing ‘else’ in 36.213 clause 8.6.1 according to the above TP.