3GPP TSG- RAN WG1 Meeting #102-e                                                   R1-2006477

# e-Meeting, August 17th – 28th, 2020

Agenda Item:	7.1

Source:	Apple Inc.

Title:	Discussion on SRS Collision handling

Document for:	Discussion/Decision

# Introduction

In this contribution, we provide some background and discussion on collision handling between two SRS resources for CR R1-2006478.

# Discussion

In 38.214 [1], the following is defined on collision handling between SRS and other signals.

### 6.2.1 UE sounding procedure The UE may be configured with one or more Sounding Reference Signal (SRS) resource sets as configured by the higher layer parameter SRS-ResourceSet. For each SRS resource set, a UE may be configured with SRS resources (higher layer parameter SRS-Resource), where the maximum value of K is indicated by UE capability [13, 38.306]. The SRS resource set applicability is configured by the higher layer parameter usage in SRS-ResourceSet. When the higher layer parameter usage is set to 'beamManagement', only one SRS resource in each of multiple SRS sets may be transmitted at a given time instant, but the SRS resources in different SRS resource sets with the same time domain behaviour in the same BWP may be transmitted simultaneously. < Unchanged parts are omitted > For PUCCH and SRS on the same carrier, a UE shall not transmit SRS when semi-persistent and periodic SRS are configured in the same symbol(s) with PUCCH carrying only CSI report(s), or only L1-RSRP report(s). A UE shall not transmit SRS when semi-persistent or periodic SRS is configured or aperiodic SRS is triggered to be transmitted in the same symbol(s) with PUCCH carrying HARQ-ACK and/or SR. In the case that SRS is not transmitted due to overlap with PUCCH, only the SRS symbol(s) that overlap with PUCCH symbol(s) are dropped. PUCCH shall not be transmitted when aperiodic SRS is triggered to be transmitted to overlap in the same symbol with PUCCH carrying semi-persistent/periodic CSI report(s) or semi-persistent/periodic L1-RSRP report(s) only. In case of intra-band carrier aggregation or in inter-band CA band combination if simultaneous SRS and PUCCH/PUSCH transmissions are not supported by UE, the UE is not expected to be configured with SRS from a carrier and PUSCH/UL DM-RS/UL PT-RS/PUCCH formats from a different carrier in the same symbol. In case of intra-band carrier aggregation or in inter-band CA band combination if simultaneous SRS and PRACH transmissions are not supported by UE, the UE shall not transmit simultaneously SRS resource(s) from a carrier and PRACH from a different carrier. In case a SRS resource with resourceType set as 'aperiodic' is triggered on the OFDM symbol(s) configured with periodic/semi-persistent SRS transmission, the UE shall transmit the aperiodic SRS resource and only the periodic/semi-persistent SRS symbol(s) overlapping within the symbol(s) are dropped, while the periodic/semi-persistent SRS symbol(s) that are not overlapped with the aperiodic SRS resource are transmitted. In case a SRS resource with resourceType set as 'semi-persistent' is triggered on the OFDM symbol(s) configured with periodic SRS transmission, the UE shall transmit the semi-persistent SRS resource and only the periodic SRS symbol(s) overlapping within the symbol(s) are dropped, while the periodic SRS symbol(s) that are not overlapped with the semi-persistent SRS resource are transmitted.

Further, in 38.306 [2], the following UE capabilities regard to SRS collision handling have been defined as follows.

| parallelTxSRS-PUCCH-PUSCH Indicates whether the UE supports parallel transmission of SRS and PUCCH/ PUSCH across CCs in an inter-band CA band combination. | BC | No | No | No |
| --- | --- | --- | --- | --- |
| parallelTxPRACH-SRS-PUCCH-PUSCH Indicates whether the UE supports parallel transmission of PRACH and SRS/PUCCH/PUSCH across CCs in an inter-band CA band combination. | BC | No | No | No |

From the specification above, it is clear that for intra-band CA case, to configure simultaneous transmission between SRS and PUCCH/PUSCH/PRACH is not allowed. One UE capability is introduced to report whether UE supports simultaneous transmission of SRS and PUCCH/PUSCH for inter-band CA, and another UE capability is introduced to report whether UE supports simultaneous transmission of SRS and PRACH. Thus the collision handling between SRS and uplink signals other than SRS is pretty clear.

One potential issue is about the collision handling between SRS resources. Current specification defines something related to collision handling and scheduling restriction, but it is unclear whether such type of definition is for intra-CC or intra-band CA or inter-band CA case.

In general, there are 3 issues to be clarified.

Issue #1: Is the following sentence defined for intra-CC or intra-band CA or inter-band CA case?

“When the higher layer parameter usage is set to 'beamManagement', only one SRS resource in each of multiple SRS sets may be transmitted at a given time instant, but the SRS resources in different SRS resource sets with the same time domain behaviour in the same BWP may be transmitted simultaneously.”

Issue #2: Is it correct that for other cases as defined in the following sentence, when SRS resources are configured with the same type of time domain behavior, to configure simultaneous transmission is not allowed?

“When the higher layer parameter usage is set to 'beamManagement', only one SRS resource in each of multiple SRS sets may be transmitted at a given time instant, but the SRS resources in different SRS resource sets with the same time domain behaviour in the same BWP may be transmitted simultaneously.”

Issue #3: Is the following paragraph defined for intra-CC or intra-band CA or inter-band CA case?

“In case a SRS resource with resourceType set as 'aperiodic' is triggered on the OFDM symbol(s) configured with periodic/semi-persistent SRS transmission, the UE shall transmit the aperiodic SRS resource and only the periodic/semi-persistent SRS symbol(s) overlapping within the symbol(s) are dropped, while the periodic/semi-persistent SRS symbol(s) that are not overlapped with the aperiodic SRS resource are transmitted. In case a SRS resource with resourceType set as 'semi-persistent' is triggered on the OFDM symbol(s) configured with periodic SRS transmission, the UE shall transmit the semi-persistent SRS resource and only the periodic SRS symbol(s) overlapping within the symbol(s) are dropped, while the periodic SRS symbol(s) that are not overlapped with the semi-persistent SRS resource are transmitted. ”

Proposal: RAN1 should clarify the exact UE behavior for collision handling between two SRS resources.

# Conclusion

In this contribution, we provide detail discussion on collision handling between SRS resources, where we achieve the following proposal:

Proposal: RAN1 should clarify the exact UE behavior for collision handling between two SRS resources.

# Reference

- 3GPP TS 38.214 v15.10.0

- 3GPP TS 38.306 v15.9.0