3GPP TSG RAN WG1 #102-e	R1-2006504

e-Meeting, August 17th – 28th, 2020

Agenda Item:	8.1.3

Source:	Apple Inc.

Title:	Views on Rel-17 SRS enhancement

Document for:	Discussion/Decision

# Introduction

In RAN#86 meeting, Rel-17 FeMIMO WI was approved in RP-193133 [1]. Four main areas haven been identified for the Rel-17 FeMIMO, namely

- Enhancement on multi-beam operation, mainly targeting FR2 while also applicable to FR1

- Enhancement on the support for multi-TRP deployment, targeting both FR1 and FR2

- Enhancement on SRS, targeting both FR1 and FR2

- Enhancement on CSI measurement and reporting

Regarding SRS enhancement, the following objectives are approved as part of Rel-17 FeMIMO [1]

- Enhancement on SRS, targeting both FR1 and FR2: - Identify and specify enhancements on aperiodic SRS triggering to facilitate more flexible triggering and/or DCI overhead/usage reduction - Specify SRS switching for up to 8 antennas (e.g., xTyR, x = {1, 2, 4} and y = {6, 8}) - Evaluate and, if needed, specify the following mechanism(s) to enhance SRS capacity and/or coverage: SRS time bundling, increased SRS repetition, partial sounding across frequency

In this contribution, we provide our views on the Rel-17 FeMIMO SRS enhancement in the following areas

- More efficient SRS resource set usage configuration

- Enhancement to AP-SRS triggering offset

- More flexible SRS configuration including (1) flexible starting symbol (2) More SRS repetition (3) SRS capability enhancement via TD-OCC

- Enhancement to SRS antenna switching

# More efficient SRS resource set usage configuration

In Rel-15/16, SRS-ResourceSet can only be configured with 4 possible “usage” among {beamManagement, codebook, nonCodebook, antennaSwitching}, and, for each SRS-ResourceSet, only one usage can be configured.

For example, when SRS-ResourceSet is configured with “usage” of “antennaSwitching”, it is intended to be used for reciprocity-based DL channel acquisition. However, the same SRS-ResourceSet should also be allowed to be used for antenna selection to be combined with other use cases, such as, codebook based or non-codebook based PUSCH operation. Therefore, we have the following proposal

Proposal 1 In Rel-17, support to configure a single SRS-ResourceSet with multiple usage simultaneously, especially the cases “codebook + antennaSwitching” and “nonCodebook + antennaSwitching”

# Enhancement to AP-SRS triggering offset

In Rel-15/16, like AP-CSI-RS, the AP-SRS triggering offset is semi-statically configured by RRC as slotOffset in SRS-ResourceSet, and, UE is only allowed to exactly follow the RRC configured slot offset, i.e., if the indicated AP-SRS transmission occasion collide with “DL” symbols, etc., UE has to drop the SRS transmission.

Furthermore, there are quite some restrictions in terms of SRS-ResourceSet configurations

- Only one SRS resource set can be configured for usage of codebook

- Only one SRS resource set can be configured for usage of nonCodebook

- Many restrictions and maximum two SRS resource sets can be configured for usage of antennaSwitching

All the above restrictions may limit the flexibility of AP-SRS triggering. Therefore, we have the following proposal

Proposal 2 In Rel-17, consider to enhance SRS to allow more flexible triggering slot offset for AP-SRS

# More flexible SRS configuration

In Rel-15/16, the following restrictions on SRS configurations exist

- In Rel-15, SRS can only be transmitted among the last 6 symbols in a slot

- In Rel-15, SRS is allowed to be transmitted in any symbol in a slot, however, current specification and capability design is only clear for NRU and NR positioning. It is not clear whether this relaxation is allowed for the other use cases.

- Maximum 4 SRS repetitions

Therefore, we have the following proposal

Proposal 3 In Rel-17, consider to enhance SRS to allow more flexible SRS configurations in the areas of (1) flexible SRS location in a slot (2) More SRS repetition (3) SRS capacity enhancement via TD-OCC

# Enhancement to SRS antenna switching

In Rel-15/16, for SRS-ResourceSet configured with usage of antennaSwitching, only the following antenna configurations are supported

- 1T1R, 2T2R, 4T4R

- 1T2R, 1T4R

- 2T4R

As part of Rel-17 FeMIMO WID, it was agreed to specify SRS switching for up to 8 antennas (e.g., xTyR, x = {1, 2, 4} and y = {6, 8})}. However, in practical deployment, we do not see the use case for 6 and 8 Rx antennas. Therefore, we have the following proposal

Proposal 4 More deployment justification is needed to specify SRS antenna switching for 6 or 8 Rx antennas

# Conclusion

In RAN#86 meeting, Rel-17 FeMIMO WI was approved in RP-193133 [1]. Four main areas haven been identified for the Rel-17 FeMIMO, namely

- Enhancement on multi-beam operation, mainly targeting FR2 while also applicable to FR1

- Enhancement on the support for multi-TRP deployment, targeting both FR1 and FR2

- Enhancement on SRS, targeting both FR1 and FR2

- Enhancement on CSI measurement and reporting

In this contribution, we provide our views on the Rel-17 FeMIMO SRS enhancement in the following areas

- More efficient SRS resource set usage configuration

- Enhancement to AP-SRS triggering offset

- More flexible SRS configuration including (1) flexible starting symbol (2) More SRS repetition (3) SRS capability enhancement via TD-OCC

- Enhancement to SRS antenna switching

We have the following proposals

Proposal 1 In Rel-17, support to configure a single SRS-ResourceSet with multiple usage simultaneously, especially the cases “codebook + antennaSwitching” and “nonCodebook + antennaSwitching”

Proposal 2 In Rel-17, consider to enhance SRS to allow more flexible triggering slot offset for AP-SRS

Proposal 3 In Rel-17, consider to enhance SRS to allow more flexible SRS configurations in the areas of (1) flexible SRS location in a slot (2) More SRS repetition (3) SRS capacity enhancement via TD-OCC

Proposal 4 More deployment justification is needed to specify SRS antenna switching for 6 or 8 Rx antennas

# Reference

- RP-193133, "New WID: Further enhancements on MIMO for NR", 3GPP TSG RAN Meeting #86, Sitges, Spain, December 9-12, 2019