3GPP TSG RAN WG1 #102-e	R1-2006554

e-Meeting, August 17th – 28th, 2020

Source:	Sharp

Title:	Remaining issues on UL signals/channels for NR-U

Agenda Item:	7.2.2.1.3

Document for:	Discussion and Decision

Background

The following agreements were achieved at RAN1#101 e-meeting [1].

Agreement: RRC parameters intraCellGuardBandDL-r16 and intraCellGuardBandUL-r16 can be configured at least as UE-specific, per cell per numerology. Agreement: - As per prior agreement, initial UL BWP is 20 MHz - FFS: The case of SUL in licensed band - For PUSCH scheduled by a RAR UL Grant (e.g., Msg3) or by DCI 0_0 addressed to TC-RNTI (Msg3 re-transmission) when UL Resource Allocation Type 2 is configured, the PUSCH is transmitted as follows: - PUSCH is transmitted in the same UL RB set of the active UL BWP as PRACH (Msg1) - FFS: The case where PRACH is configured in more than one RB set Agreement: - For PUSCH with UL resource allocation Type 2 scheduled by DCI 0_0 with CRC scrambled by C-RNTI / CS-RNTI / MCS-RNTI received in a CSS, PUSCH is allocated to a single UL RB set in the active UL BWP where the UL RB set is the lowest indexed one amongst UL RB set(s) that intersects the lowest indexed CCE of the PDCCH in the active DL BWP in which the UE detects the DCI. - If there is no intersection, PUSCH is allocated to RB set 0 of the active UL BWP.

Remaining issues

Our understanding of the remaining issues for UL signals/channels are as follows.

1) Resource allocation for msg3 PUSCH

2) DCI size matching issue for DCI format 0_0

3) Support for interlace allocation for DCI format 0_2

We provide our views on the above remaining issues.

  - Resource allocation for msg3 PUSCH

As agreed at the last meeting, configuration of intra-cell guard bands is moved to UE-specific configuration (i.e., in servingCellConfig). Thus, for UEs in initial access procedure, since the UE is not provided UE-specific configuration, the RB-set determination shouldn’t be based on the UE-specific configuration. It is straightforward to use nominal intra-cell guard band configuration for msg3 PUSCH transmission.

Proposal 1: For msg3 PUSCH, the RB-set configuration is provided by nominal intra-cell guard band configuration.

For a PUSCH transmission before receiving RRC reconfiguration message (i.e., PUSCH scheduled by DCI format 0_0 monitored in a CSS set) should be handled in the same way as msg3 PUSCH. Thus,

Proposal 2: For a PUSCH scheduled by a DCI format 0_0 monitored in a CSS set, the RB-set configuration is provided by nominal intra-cell guard band configuration.

  - DCI size matching issue for DCI format 0_0

As agreed at the previous meeting, DCI format 0_0 monitored in USS includes RB-set indication bits (i.e., Y bits). That leads to an issue when the DCI size requirement is not satisfied, in which case the size alignment procedure for DCI format 0_0 monitored in CSS and USS would be triggered.

For interlace indication bits (i.e., X bits), the issue occurs when the SCS for the active UL BWP is different from the initial UL BWP, in which case the number of the interlace indication bits are re-determined by the SCS of the initial UL BWP, but the PUSCH is allocated to the active UL BWP. For example, when the SCS of the active UL BWP is 15 kHz and the SCS of the initial UL BWP is 30 kHz, the DCI format 0_0 monitored in USS includes only 5 bits but there are 10 interlaces in the active UL BWP. Handling this issue would be so complicated. Similar issues occur for RB-set indication bits.

We propose a simple solution for the above issue. That is, FDRA size determination for a DCI format 0_0 monitored in USS and CSS based on Rel-15 although the actual information is smaller than the size determined by Rel-15 procedure. In Rel-15, the FDRA size determination is based on , where  is the number of resource blocks for the active UL BWP or the initial UL BWP depending on the monitored search space type and the DCI size matching. Since the minimum number of resource blocks is 49 (according to RAN4) for 30 kHz SCS when the interlaced UL transmission is configured, the minimum size of FDRA field is 11 bits (=log2(49*50/2)). 11 bits can contain both the maximum number of interlace indication bits (6 bits when the SCS is 15 kHz) and the maximum number of RB-set indication bits (4 bits when the number of RB-sets are 5).

Even when the FDRA size is increased by reusing Rel-15 procedure, it doesn’t increase the number of DCI format sizes since the size of the DCI format 0_0 monitored in CSS is always smaller than the size of the DCI format 1_0 monitored in CSS.

Proposal 3: When the interlaced UL transmission is configured, the size of the FDRA field in the DCI format 0_0 monitored in CSS and USS is determined as in Rel-15.

Proposal 4: FDRA field interpretation is based on the active UL BWP.

  - Support for interlace allocation for DCI format 0_2

As agreed at RAN1#99, the UE is only required to operate either with interlace PUCCH/PUSCH or non-interlace PUCCH/PUSCH. If the UE is configured with DCI format 0_2, then the configuration should follow RRC parameter for interlace configuration.

Proposal 5: DCI format 0_2 also supports interlace scheduling.

Text proposals

The following TPs is related to the above proposals

Text proposal#1 --------- beginning of text proposal for TS 38.212 ##### 7.3.1.1.1 Format 0_0 DCI format 0_0 is used for the scheduling of PUSCH in one cell. The following information is transmitted by means of the DCI format 0_0 with CRC scrambled by C-RNTI or CS-RNTI or MCS-C-RNTI: - Identifier for DCI formats – 1 bit - The value of this bit field is always set to 0, indicating an UL DCI format - Frequency domain resource assignment – bits, where is defined in clause 7.3.1.0 - For PUSCH hopping with resource allocation type 1: - MSB bits are used to indicate the frequency offset according to Clause 6.3 of [6, TS 38.214], where if the higher layer parameter frequencyHoppingOffsetLists contains two offset values and if the higher layer parameter frequencyHoppingOffsetLists contains four offset values - bits provides the frequency domain resource allocation according to Clause 6.1.2.2.2 of [6, TS 38.214] - For non-PUSCH hopping with resource allocation type 1: - bits provides the frequency domain resource allocation according to Clause 6.1.2.2.2 of [6, TS 38.214] - if any of the higher layer parameters useInterlacePUCCH-PUSCH in BWP-UplinkCommon and useInterlacePUCCH-PUSCH in BWP-UplinkDedicated is configured - 5+Y most significant bits provide the frequency domain resource allocation according to Clause 6.1.2.2.3 of [6, TS 38.214] if the subcarrier spacing for the active UL bandwidth part is 30 kHz and the DCI format 0_0 is monitored in a UE-specific search space. If the DCI 0_0 is monitored in a common search space Y = 0. - 6+Y most significant bits provide the frequency domain resource allocation according to Clause 6.1.2.2.3 of [6, TS 38.214] if the subcarrier spacing for the active UL bandwidth part is 15 kHz and the DCI format 0_0 is monitored in a UE-specific search space. If the DCI 0_0 is monitored in a common search space Y = 0. The value of Y is determined by log2NRB-set,ULBWPNRB-set,ULBWP+12 where NRB-set,ULBWP is the number of RB sets contained in the UL BWP as defined in clause 7 of [6, TS38.214]. - Time domain resource assignment – 4 bits as defined in Clause 6.1.2.1 of [6, TS 38.214] - Frequency hopping flag – 1 bit according to Table 7.3.1.1.1-3, as defined in Clause 6.3 of [6, TS 38.214] - Modulation and coding scheme – 5 bits as defined in Clause 6.1.4.1 of [6, TS 38.214] - New data indicator – 1 bit - Redundancy version – 2 bits as defined in Table 7.3.1.1.1-2 - HARQ process number – 4 bits - TPC command for scheduled PUSCH – 2 bits as defined in Clause 7.1.1 of [5, TS 38.213] - Padding bits, if required. - UL/SUL indicator – 1 bit for UEs configured with supplementaryUplink in ServingCellConfig in the cell as defined in Table 7.3.1.1.1-1 and the number of bits for DCI format 1_0 before padding is larger than the number of bits for DCI format 0_0 before padding; 0 bit otherwise. The UL/SUL indicator, if present, locates in the last bit position of DCI format 0_0, after the padding bit(s). - If the UL/SUL indicator is present in DCI format 0_0 and the higher layer parameter pusch-Config is not configured on both UL and SUL the UE ignores the UL/SUL indicator field in DCI format 0_0, and the corresponding PUSCH scheduled by the DCI format 0_0 is for the UL or SUL for which high layer parameter pucch-Config is configured; - If the UL/SUL indicator is not present in DCI format 0_0 and pucch-Config is configured, the corresponding PUSCH scheduled by the DCI format 0_0 is for the UL or SUL for which high layer parameter pucch-Config is configured. - If the UL/SUL indicator is not present in DCI format 0_0 and pucch-Config is not configured, the corresponding PUSCH scheduled by the DCI format 0_0 is for the uplink on which the latest PRACH is transmitted. - ChannelAccess-CPext – 2 bits indicating combinations of channel access type and CP extension as defined in Table 7.3.1.1.1-4 for operation in a cell with shared spectrum channel access; 0 bit otherwise. The following information is transmitted by means of the DCI format 0_0 with CRC scrambled by TC-RNTI: - Identifier for DCI formats – 1 bit - The value of this bit field is always set to 0, indicating an UL DCI format - Frequency domain resource assignment – number of bits determined by the following: - bits, where - is the size of the initial UL bandwidth part. - For PUSCH hopping with resource allocation type 1: - MSB bits are used to indicate the frequency offset according to Table 8.3-1 in Clause 8.3 of [5, TS 38.213], where if and otherwise - bits provides the frequency domain resource allocation according to Clause 6.1.2.2.2 of [6, TS 38.214] - For non-PUSCH hopping with resource allocation type 1: - bits provides the frequency domain resource allocation according to Clause 6.1.2.2.2 of [6, TS 38.214] - if the higher layer parameter useInterlacePUCCH-PUSCH in BWP-UplinkCommon is configured - 5 most significant bits provide the frequency domain resource allocation according to Clause 6.1.2.2.3 of [6, TS 38.214] if the subcarrier spacing for the active UL bandwidth part is 30 kHz - 6 most significant bits provide the frequency domain resource allocation according to Clause 6.1.2.2.3 of [6, TS 38.214] if the subcarrier spacing for the active UL bandwidth part is 15 kHz - Time domain resource assignment – 4 bits as defined in Clause 6.1.2.1 of [6, TS 38.214] - Frequency hopping flag – 1 bit according to Table 7.3.1.1.1-3, as defined in Clause 6.3 of [6, TS 38.214] - Modulation and coding scheme – 5 bits as defined in Clause 6.1.4.1 of [6, TS 38.214] - New data indicator – 1 bit, reserved - Redundancy version – 2 bits as defined in Table 7.3.1.1.1-2 - HARQ process number – 4 bits, reserved - TPC command for scheduled PUSCH – 2 bits as defined in Clause 7.1.1 of [5, TS 38.213] - Padding bits, if required. - UL/SUL indicator – 1 bit if the cell has two ULs and the number of bits for DCI format 1_0 before padding is larger than the number of bits for DCI format 0_0 before padding; 0 bit otherwise. The UL/SUL indicator, if present, locates in the last bit position of DCI format 0_0, after the padding bit(s). - If 1 bit, reserved, and the corresponding PUSCH is always on the same UL carrier as the previous transmission of the same TB - ChannelAccess-CPext – 2 bits indicating combinations of channel access type and CP extension as defined in Table 7.3.1.1.1-4 for operation in a cell with shared spectrum channel access; 0 bit otherwise -------- Unchanged contents are omitted ##### 7.3.1.1.3 Format 0_2 DCI format 0_2 is used for the scheduling of PUSCH in one cell. The following information is transmitted by means of the DCI format 0_2 with CRC scrambled by C-RNTI or CS-RNTI or SP-CSI-RNTI or MCS-C-RNTI: - Identifier for DCI formats – 1 bit - The value of this bit field is always set to 0, indicating an UL DCI format - Carrier indicator – 0, 1, 2 or 3 bits determined by higher layer parameter carrierIndicatorSizeForDCI-Format0-2, as defined in Clause 10.1 of [5, TS38.213]. - UL/SUL indicator – 0 bit for UEs not configured with supplementaryUplink in ServingCellConfig in the cell or UEs configured with supplementaryUplink in ServingCellConfig in the cell but only one carrier in the cell is configured for PUSCH transmission; otherwise, 1 bit as defined in Table 7.3.1.1.1-1. - Bandwidth part indicator – 0, 1 or 2 bits as determined by the number of UL BWPs nBWP, RRC configured by higher layers, excluding the initial UL bandwidth part. The bitwidth for this field is determined as log2(nBWP) bits, where - nBWP=nBWP, RRC+1 if nBWP, RRC≤3, in which case the bandwidth part indicator is equivalent to the ascending order of the higher layer parameter BWP-Id; - otherwise nBWP=nBWP, RRC, in which case the bandwidth part indicator is defined in Table 7.3.1.1.2-1; If a UE does not support active BWP change via DCI, the UE ignores this bit field. - Frequency domain resource assignment – number of bits determined by the following: - If the higher layer parameter useInterlacePUCCH-PUSCH in BWP-UplinkDedicated is not configured - NRBG bits if only resource allocation type 0 is configured, where NRBG is defined in Clause 6.1.2.2.1 of [6, TS 38.214] - log2NRBG, K1NRBG, K1+1/2 bits if only resource allocation type 1 is configured, or max log2NRBG, K1NRBG, K1+1/2, NRBG+1 bits if both resource allocation type 0 and 1 are configured, where NRBG, K1=NRBUL, BWP+NUL, BWPstartmodK1/K1, NRBUL, BWP is the size of the active UL bandwidth part, NUL, BWPstart is defined as in clause 4.4.4.4 of [4, TS 38.211] and K1 is given by higher layer parameter ResourceAllocationType1-granularity-ForDCIFormat0_2. If the higher layer parameter ResourceAllocationType1-granularity-ForDCIFormat0_2 is not configured, K1 is equal to 1. - If both resource allocation type 0 and 1 are configured, the MSB bit is used to indicate resource allocation type 0 or resource allocation type 1, where the bit value of 0 indicates resource allocation type 0 and the bit value of 1 indicates resource allocation type 1. - For resource allocation type 0, the NRBG LSBs provide the resource allocation as defined in Clause 6.1.2.2.1 of [6, TS 38.214]. - For resource allocation type 1, the log2NRBG, K1NRBG, K1+1/2 LSBs provide the resource allocation as follows: - For PUSCH hopping with resource allocation type 1: - NUL_hop MSB bits are used to indicate the frequency offset according to Clause 6.3 of [6, TS 38.214], where NUL_hop=1 if the higher layer parameter frequencyHoppingOffsetLists-ForDCIFormat0_2 contains two offset values and NUL_hop=2 if the higher layer parameter frequencyHoppingOffsetLists-ForDCIFormat0_2 contains four offset values - log2NRBG, K1NRBG, K1+1/2-NUL_hop bits provides the frequency domain resource allocation according to Clause 6.1.2.2.2 of [6, TS 38.214] - For non-PUSCH hopping with resource allocation type 1: - log2NRBG, K1NRBG, K1+1/2 bits provides the frequency domain resource allocation according to Clause 6.1.2.2.2 of [6, TS 38.214] - If the higher layer parameter useInterlacePUCCH-PUSCH in BWP-UplinkDedicated is configured - 5 + Y most significant bits provide the frequency domain resource allocation according to Clause 6.1.2.2.3 of [6, TS 38.214] if the subcarrier spacing for the active UL bandwidth part is 30 kHz. The 5 MSBs provide the interlace allocation and the Y LSBs provide the RB set allocation. - 6 + Y most significant bits provide the frequency domain resource allocation according to Clause 6.1.2.2.3 of [6, TS 38.214] if the subcarrier spacing for the active UL bandwidth part is 15 kHz. The 6 MSBs provide the interlace allocation and the Y LSBs provide the RB set allocation. - The value of Y is determined by log2NN+12 where N is the number of RB sets contained in the active UL bandwidth part as defined in clause 7 of [6, TS38.214]. If "Bandwidth part indicator" field indicates a bandwidth part other than the active bandwidth part and if both resource allocation type 0 and 1 are configured for the indicated bandwidth part, the UE assumes resource allocation type 0 for the indicated bandwidth part if the bitwidth of the "Frequency domain resource assignment" field of the active bandwidth part is smaller than the bitwidth of the "Frequency domain resource assignment" field of the indicated bandwidth part. --------- end of text proposal

Text proposal#2 --------- beginning of text proposal for TS 38.213 ## 8.3 PUSCH scheduled by RAR UL grant -------- Unchanged contents are omitted If useInterlace-PUCCH-PUSCH is provided by BWP-UplinkCommon or BWP-UplinkDedicated, the frequency domain resource allocation is by uplink resource allocation type 2 [6, TS 38.214]. A UE processes the frequency domain resource assignment field as follows - truncate the frequency domain resource assignment field to the X=6 LSBs if μ=0, or to the X=5 LSBs if μ=1 - for interlace allocation of a PUSCH transmission, interpret the X MSBs of the truncated frequency domain resource assignment field for the active UL BWP as for the X MSBs of the frequency domain resource assignment field in DCI format 0_0 [6, TS 38.214] - for RB set allocation of a PUSCH transmission, the RB set of the active UL BWP is the RB set of the PRACH transmission associated with the RAR UL grant where the RB-sets are provided by the nominal intra-cell guard band and RB set pattern as specified in [8, TS 38.101-1] A UE determines whether or not to apply transform precoding as described in [6, TS 38.214]. -------- Unchanged contents are omitted --------- end of text proposal

Text proposal#3 --------- beginning of text proposal for TS 38.214 ##### 6.1.2.2.3 Uplink resource allocation type 2 In uplink resource allocation of type 2, the resource block assignment information defined in [5, TS 38.212] indicates to a UE a set of up to M interlace indices, and for DCI 0_0 monitored in a UE-specific search space and DCI 0_1 a set of up to NRB-set,ULBWP contiguous RB sets, where M and interlace indexing are defined in Clause 4.4.4.6 in [4, TS 38.211]. For DCI 0_0 monitored in a UE-specific search space and DCI 0_1, the UE shall determine the resource allocation in frequency domain as an intersection of the resource blocks of the indicated interlaces and the union of the indicated set of RB sets and intra-cell guard bands defined in Clause 7 between the indicated RB sets, if any. For DCI 0_0 monitored in a common search space, the UE shall determine the resource allocation in frequency domain as an intersection of the resource blocks of the indicated interlaces and a single uplink RB set of the active UL BWP where the RB-sets are provided by the nominal intra-cell guard band and RB set pattern as specified in [8, TS 38.101-1]. For DCI 0_0 monitored in a CSS with CRC scrambled by an RNTI other than TC-RNTI, the uplink RB set is the lowest indexed one amongst uplink RB set(s) that intersects the lowest-indexed CCE of the PDCCH in which the UE detects the DCI 0_0 in the active downlink BWP. If there is no intersection, the uplink RB set is RB set 0 in the active uplink BWP. For DCI 0_0 monitored in a CSS with CRC scrambled by TC-RNTI, the uplink RB set is the same one in which the UE transmits the PRACH associated with the RAR UL grant where the RB-sets are provided by the nominal intra-cell guard band and RB set pattern as specified in [8, TS 38.101-1]. -------- Unchanged contents are omitted --------- end of text proposal

Conclusion

In this contribution, we have the following proposals:

Proposal 1: For msg3 PUSCH, the RB-set configuration is provided by nominal intra-cell guard band configuration.

Proposal 2: For a PUSCH scheduled by a DCI format 0_0 monitored in a CSS set, the RB-set configuration is provided by nominal intra-cell guard band configuration.

Proposal 3: When the interlaced UL transmission is configured, the size of the FDRA field in the DCI format 0_0 monitored in CSS and USS is determined as in Rel-15.

Proposal 4: FDRA field interpretation is based on the active UL BWP.

Proposal 5: DCI format 0_2 also supports interlace scheduling.

References

- Chairman’s note at RAN1#101 e-Meeting, May 2020