3GPP TSG RAN WG1 #102	R1-2006567

e-Meeting, August 17th – 28th, 2020

Source:	Sharp

Title:	Enhancement on inter-cell multi-TRP operation

Agenda Item:	8.1.2.2

Document for:	Discussion and Decision

# Background

At RAN#86, a new work item on “Further enhancements on MIMO for NR” was agreed. For enhancement on multi-TRP operations, the following objectives were agreed [1].

- Enhancement on the support for multi-TRP deployment, targeting both FR1 and FR2: - Identify and specify features to improve reliability and robustness for channels other than PDSCH (that is, PDCCH, PUSCH, and PUCCH) using multi-TRP and/or multi-panel, with Rel.16 reliability features as the baseline - Identify and specify QCL/TCI-related enhancements to enable inter-cell multi-TRP operations, assuming multi-DCI based multi-PDSCH reception - Evaluate and, if needed, specify beam-management-related enhancements for simultaneous multi-TRP transmission with multi-panel reception - Enhancement to support HST-SFN deployment scenario: - Identify and specify solution(s) on QCL assumption for DMRS, e.g. multiple QCL assumptions for the same DMRS port(s), targeting DL-only transmission - Evaluate and, if the benefit over Rel.16 HST enhancement baseline is demonstrated, specify QCL/QCL-like relation (including applicable type(s) and the associated requirement) between DL and UL signal by reusing the unified TCI framework

In this contribution, we share our initial views on enhancement of inter-cell multi-TRP operation.

# Discussion

## Inter-cell multi-TRP operations

In Rel-16, RAN1 have discussed inter-cell multi-TRP operations in RAN1#96.

Agreement To support multiple-PDCCH based multi-TRP/panel transmission with intra-cell (same cell ID) and inter-cell (different Cell IDs), following RRC configuration can be used to link multiple PDCCH/PDSCH pairs with multiple TRPs - one CORESET in a “PDCCH-config” corresponds to one TRP - FFS whether to increase the number of CORESETs per “PDCCH-config” more than 3 FFS: UE monitoring/decoding behavior for multiple PDCCHs. Include in LS to RAN2 Agreement For separate ACK/NACK payload/feedback for received PDSCHs where multiple DCIs are used, - PUCCH resources conveying ACK/NACK feedback can be TDM with separated HARQ-ACK codebook. - FFS TDM within a slot - FFS: the format of PUCCH from multiple TRP shall be same or different For issues related to PUCCH resources, study including: - FFS: if PUCCH resources conveying ACK/NACK feedback are overlapped at time, whether predefined dropping rule is needed to drop ACK/NACK feedback. - FFS: how to handle ACK/NACK overlapping with CSI reporting for different TRPs - FFS: how to handle PUCCH overlapping with PUSCH at the time domain for different TRPs - FFS: whether the UE can assume simultaneous ACK/NACK transmission from multiple PUCCH resources, and associated details of configurations/indication/UE capability. Include in LS to RAN2

However, we couldn’t reach consensus as follows in RAN1#98:

Conclusion The following is not supported in Rel-16 due to lack of consensus and time: - Enhancement on TCI framework by indicating SSB/PCI from a non-serving cell for multi-TRP/panel transmission for an inter-cell scenario

In our perspective, for inter-cell scenario, it is the simplest way to enhance TCI framework of SSB/CSI-RS from a non-serving cell. Therefore,

Proposal: SSB/CSI-RS and PCI should be introduced into the TCI framework.

# Conclusion

In this contribution, we have the following proposal:

Proposal: SSB/CSI-RS and PCI should be introduced into the TCI framework.

# References

- RP-193133, “New WID: Further enhancements on MIMO for NR”, RAN1#86, December 2019