3GPP TSG RAN WG1 #102-e	R1-2006578

e-Meeting, August 17th – 28th, 2020

Source:	Sharp

Title:	Evaluation results of coverage for FR1 Urban scenario

Agenda Item:	8.8.1.1

Document for:	Discussion and Decision

Background

At RAN1#101-e meeting [1], some of the evaluation parameters are agreed for FR1. Some of the agreed values are listed as below.

Table 1: Short summary of the agreed values

| Parameters | Values |
| --- | --- |
| carrier frequency | Urban: 4GHz (TDD), 2.6GHz (TDD) Rural: 4GHz (TDD), 2.6GHz (TDD), 2GHz (FDD), 700MHz (FDD) Rural with long distance: 700MHz (FDD), 4GHz (TDD) |
| BWP size | 100MHz for 4GHz and 2.6GHz. 20MHz for 2GHz (FDD) 20MHz (optional for 10MHz) for 700MHz. (FDD) |
| Frame structure | DDDSU (S: 10D:2G:2U) only for 4GHz DDDSUDDSUU (S: 10D:2G:2U) only for 4GHz DDDDDDDSUU (S: 6D:4G:4U) only for 2.6GHz Other frame structures can be reported by companies. |
| target data rate | For eMBB, 10 Mbps/1 Mbps for DL/UL for Urban, 1 Mbps/ 100 kbps for DL/UL for Rural 30 kbps for UL is optional for Rural with long distance scenario For VoIP, 320 bits with 20 ms interval TBD: TBS for SIP invite message. 1500 bytes can be a starting point |
| Target BLER | PUSCH: For eMBB, 10% iBLER, for VoIP, 2% rBLER PUCCH: For PUCCH format 1, 1% DTXtoACK error, 0.1% NACKtoACK error, 1% ACK miss detection probability For PUCCH format 3, 1% BLER for ACK/NACK/SR, FFS 10 or 1% BLER for CSI |
| PRBs/TBS/MCS | Any value of PRBs, and corresponding MCS index, reported by companies will be considered in the discussion. Companies are encouraged to use 30 PRBs for 1Mbps, 4 PRBs for 100kbps, 1 PRB for 30kbps as a starting point. |

Coverage evaluation

Evaluation assumption

Table 2 shows an evaluation assumption for coverage estimation based on the agreed assumption.

Table 2: Evaluation assumption for coverage estimation

| Carrier frequency | 700 MHz, 2.6 GHz, 4 GHz |
| --- | --- |
| Duplex type and configuration | FDD for 700 MHz DDDDDDDSUU (S: 6D:4G:4U) for 2.6 GHz DDDSU (S: 10D:2G:2U) for 4 GHz |
| Subcarrier spacing | 15 kHz for 700 MHz 30 kHz for 2.6 GHz and 4 GHz |
| BS antenna height | 25 m |
| UE antenna height | 1.5 m |
| coverage reliability for control channel | 95% |
| coverage reliability for data channel | 90% |
| shadow fading standard deviation | 7 dB |
| Pathloss model | NLOS O-to-I |
| Penetration margin | 26.25 dB |
| Target data rate for PDCCH | 128000 bps |
| Target data rate for PUCCH | 4000 bps |
| Target data rate for PDSCH | 10 Mbps |
| Target data rate for PUSCH | 1 Mbps |
| Target BLER for control channel | 0.01 |
| Target iBLER for data channel | 0.1 |
| Number of transmit antennas | 192 for 2.6 GHz and 4 GHz 16 for 700 MHz |
| Array gain assumption | 10*log10 (# transmit antennas / # antenna ports) |
| Number of antenna ports | 2 |
| Number of transmit chains | 1 |
| Transmission power | 44 dBm per 20 MHz for DL, 23 dBm for UL |
| Transmitter antenna gain | 8 dBi for DL, 0 dBi for UL |
| Cable, connector, body losses | 3 dB for DL, 1 DB for UL |
| Number of receive antennas | 4 for model A/B |
| Receiver noise figure | 7 dB for DL, 5 dB for UL |
| Thermal noise density | -174 dB |
| Receiver interference density | -169.30 dBm/Hz |
| Receiver implementation margin | 2 dB |
| HARQ gain for control channel | 0 dB |
| HARQ gain for data channel | 0.5 dB |

Table 2-1: Link level simulation assumption for all channels

| Channel estimation | 2D-MMSE |
| --- | --- |
| Receiver | MMSE |
| Channel model | TDL-C |

Table 2-2: Link level simulation assumption for PDCCH

| Number of OFDM symbols | 2 OFDM symbols |
| --- | --- |
| Aggregation level | 16 |
| DCI size | 64 bits including CRC |
| Channel coding | polar code |
| Allocated RBs | 48 RBs |

Table 2-3: Link level simulation assumption for PUCCH

| PUCCH format | PUCCH format 1 |
| --- | --- |
| Number of OFDM symbols | 14 OFDM symbols |
| UCI size | 2 bits |
| Allocated RBs | 1 RB |
| DTX to ACK probability | 0.01 |
| NACK to ACK probability | 0.001 |
| ACK miss detection probability | 0.01 |

Table 2-4: Link level simulation assumption for PDSCH

| Number of OFDM symbols | 12 OFDM symbols |
| --- | --- |
| MCS index | MCS 12 in Table 5.1.3.1-1 in [2] for 700 MHz MCS 0 in Table 5.1.3.1-1 in [2] for 2.6 GHz and 4 GHz |
| Allocated RBs | 50 for 700 MHz 250 for 2.6 GHz 254 for 4 GHz |
| TBS | 10248 bits for 700 MHz 7048 bits for DL slots, 1416 bits for mixed slots for 2.6 GHz 7176 bits for DL slots, 4232 bits for mixed slots for 4 GHz |
| DMRS configuration | Type 1 (symbol #2 and symbol #7) |

Table 2-5: Link level simulation assumption for PUSCH

| Number of OFDM symbols | 14 OFDM symbols |
| --- | --- |
| MCS index | MCS 0 in Table 5.1.3.1-1 in [2] |
| Allocated RBs | 30 RBs for 700 MHz 76 RBs for 2.6 GHz and 4 GHz |
| TBS | 1032 bits for 700 MHz 2536 bits for UL slots, no PUSCH allocation for mixed slots for 2.6 GHz and 4 GHz |
| DMRS configuration | Type 1 (symbol #2 and symbol #7) |

Table 2-5: Link level simulation assumption for msg 3 PUSCH

| Number of OFDM symbols | 14 OFDM symbols |
| --- | --- |
| MCS index | MCS 0 in Table 5.1.3.1-1 in [2] |
| Allocated RBs | 2 RBs |
| TBS | 56 bits |
| DMRS configuration | Type 1 (symbol #2, #7, #11) |

Evaluation results

Table 3 shows the maximum range for each channel for Model A and Model B.

Table 3-1: Maximum range for each channel for 700 MHz

|  | Model A [m] | Model B [m] |
| --- | --- | --- |
| PDCCH | 1263.26 | 2059.39 |
| PUCCH | 378.56 | 617.01 |
| PDSCH | 584.93 | 953.45 |
| PUSCH | 356.58 | 581.18 |
| msg3 PUSCH | 384.96 | 627.44 |

Table 3-2: Maximum range for each channel for 2.6 GHz

|  | Model A [m] | Model B [m] |
| --- | --- | --- |
| PDCCH | 1219.11 | 1990.92 |
| PUCCH | 365.29 | 595.38 |
| PDSCH | 1047.83 | 1711.16 |
| PUSCH | 344.08 | 560.80 |
| msg3 PUSCH | 371.46 | 605.44 |

Table 3-3: Maximum range for each channel for 4 GHz

|  | Model A [m] | Model B [m] |
| --- | --- | --- |
| PDCCH | 977.95 | 1597.01 |
| PUCCH | 293.03 | 477.58 |
| PDSCH | 840.55 | 1372.61 |
| PUSCH | 276.01 | 449.85 |
| msg3 PUSCH | 297.98 | 485.66 |

As shown above, the uplink channels can be identified as the bottle neck. Thus, potential enhancement for PUCCH, and PUSCH including msg3 should be further studied.

In addition, for 700 MHz band, PDSCH coverage is reduced due to higher MCS. We can consider PDSCH enhancement as well for 700 MHz band.

Proposal 1: Potential enhancements for PUCCH, and PUSCH including msg3 should be further studied.

Proposal 2: Potential enhancements for PDSCH at least for targeting 700 MHz band can be considered.

Conclusion

In this contribution, we have the following proposals:

Proposal 1: Potential enhancements for PUCCH, and PUSCH including msg3 should be further studied.

Proposal 2: Potential enhancements for PDSCH at least for targeting 700 MHz band can be considered.

References

- Chairman’s note at RAN1#101 e-Meeting, May 2020

- 3GPP TS38.214V16.2.0, July 2020