3GPP TSG RAN WG1 #102-e	R1-2006580

e-Meeting, August 17th – 28th, 2020

Source:	Sharp

Title:	PUCCH coverage enhancement

Agenda Item:	8.8.2.2

Document for:	Discussion and Decision

Background

At RAN1#101-e meeting [1], some of the evaluation parameters are agreed for FR1. Some of the agreed values are listed as below.

| Parameters | Values |
| --- | --- |
| carrier frequency | Urban: 4GHz (TDD), 2.6GHz (TDD) Rural: 4GHz (TDD), 2.6GHz (TDD), 2GHz (FDD), 700MHz (FDD) Rural with long distance: 700MHz (FDD), 4GHz (TDD) |
| BWP size | 100MHz for 4GHz and 2.6GHz. 20MHz for 2GHz (FDD) 20MHz (optional for 10MHz) for 700MHz. (FDD) |
| Frame structure | DDDSU (S: 10D:2G:2U) only for 4GHz DDDSUDDSUU (S: 10D:2G:2U) only for 4GHz DDDDDDDSUU (S: 6D:4G:4U) only for 2.6GHz Other frame structures can be reported by companies. |
| target data rate | For eMBB, 10 Mbps/1 Mbps for DL/UL for Urban, 1 Mbps/ 100 kbps for DL/UL for Rural 30 kbps for UL is optional for Rural with long distance scenario For VoIP, 320 bits with 20 ms interval TBD: TBS for SIP invite message. 1500 bytes can be a starting point |
| Target BLER | PUSCH: For eMBB, 10% iBLER, for VoIP, 2% rBLER PUCCH: For PUCCH format 1, 1% DTXtoACK error, 0.1% NACKtoACK error, 1% ACK miss detection probability For PUCCH format 3, 1% BLER for ACK/NACK/SR, FFS 10 or 1% BLER for CSI |
| PRBs/TBS/MCS | Any value of PRBs, and corresponding MCS index, reported by companies will be considered in the discussion. Companies are encouraged to use 30 PRBs for 1Mbps, 4 PRBs for 100kbps, 1 PRB for 30kbps as a starting point. |

Discussion

As indicated by our contribution [2], PUSCH and PUCCH are the coverage bottle neck. We provide our views on technical enhancement.

PUCCH enhancement

For PUCCH, solution based on an enhanced repetition (e.g., increasing number of repetitions) would be considered. In Rel-15, repetition is supported only for Long PUCCH which offers less flexibility in terms of TDD deployment. For TDD deployment, in order to utilize time domain resources in flexible slots, mini-slot repetition (e.g., repetition type B) like solution should be considered in Rel-17 Coverage enhancement. Thus, we support to study short PUCCH repetition.

Proposal 1: Study mini-slot repetition for PUCCH transmission.

For UCI not more than 2 bits, Rel-15 supports PUCCH format 0 with sequence selection. CGS has low PAPR characteristics.

For short PUCCH with UCI more than 2 bits, we need to study new PUCCH format since Rel-15 only supports PUCCH format 2 with CP-OFDM which causes higher PAPR. For the new format, we can consider DFT-s-OFDM and sequence selection. If the number of UCI bits is small (e.g., 3 or 4 bits), DMRS-less sequence selection may offer good performance. On the other hand, if the number of UCI bits is large, DFT-s-OFDM may be considered.

Proposal 2: For UCI not more than 2 bits, PUCCH format 0 repetition should be considered.

Proposal 3: For UCI more than 2 bits, mini-slot repetition with new PUCCH format should be considered.

Conclusion

In this contribution, we have the following proposals:

Proposal 1: Study mini-slot repetition for PUCCH transmission.

Proposal 2: For UCI not more than 2 bits, PUCCH format 0 repetition should be considered.

Proposal 3: For UCI more than 2 bits, mini-slot repetition with new PUCCH format should be considered.

References

- Chairman’s note at RAN1#101 e-Meeting, May 2020

- R1-2006578, RAN1#101 e-Meeting, May 2020