3GPP TSG RAN WG1 Meeting #102-e	R1-2006620

e-Meeting, August 17th – 28th, 2020

Agenda Item:	8.4.4

Source:	InterDigital

Title:	On feeder link switch

Document for:	Discussion and Decision

# Introduction

In RAN #86, the WI for NTN support in NR has been approved with following objectives for RAN1 [1]:

Enhancing features to address the identified issues due to long propagation delays, large Doppler effects, and moving cells in NTN, the following should be specified (see TR 38.821):

- Timing relationship enhancements [RAN1,RAN2]

- Enhancements on UL time and frequency synchronization [RAN1,RAN2]

- HARQ

  - Number of HARQ process [RAN1]

  - Enabling / disabling of HARQ feedback as described in the TR 38.821 [RAN1&2]

In addition, the following topics should be specified if beneficial and needed

- Enhancement on the PRACH sequence and/or format and extension of the ra-ResponseWindow duration (in the case of UE with GNSS capability but without pre-compensation of timing and frequency offset capabilities) [RAN1/2].

- Feeder link switch [RAN2,RAN1]

- Beam management and Bandwidth Parts (BWP) operation for NTN with frequency reuse [RAN1/2]

  - Including signalling of polarization mode

In this contribution, we discuss feeder link switch for transparent LEO scenario, and evaluate possible options captured in TR 38.821 [2].

# Discussion

## Feeder Link Switch for Transparent LEO

Rel-17 NTN will support both GEO and LEO satellite constellations, where GEO deployments are characterized by large propagation delay and cell size, however stationary and approximately fixed relative to a point on earth. Though propagation delay and cell size is less in LEO deployments, satellites move relative to a fixed point on earth with cells either moving along the ground, or fixed via e.g. beam steering techniques.

Furthermore, in the Rel-17 NTN WID, it is assumed that a transparent or “bent-pipe” configuration will be deployed, where the gNB is located on the ground and a satellite relays signalling between the gNB and the UE. This configuration is comprised of two “links”: that associated with the connection between the gNB (via a gateway) and satellite, defined as the “feeder-link” and that between the UE and satellite.

As transparent LEO satellites move around the earth, the satellite may pass a transition point where the serving gNB falls out of coverage, for example, due to the curvature of the earth. At this transition point (or within a time period before), the satellite must establish a new feeder link to an alternative gNB. The point at which the satellite establishes a connection to a new land-based gNB is referred to in the TR as a feeder link switch.

## ‘Hard’ and ‘Soft’ feeder link switch

Two methods to perform a feeder link switch have been captured in the TR: a “hard” feeder link switch and a “soft” feeder link switch. A hard feeder link switch requires the satellite to only maintain one feeder link at a time, thus before establishing a connection to a new gNB, the satellite must first sever the connection with the former serving gNB. A soft feeder link switch allows the satellite to maintain multiple feeder links for a period prior to severing the connection with the former serving gNB, assuming that the satellite can represent cells of two different gNBs over a given area using the same satellite, but via different land-based GWs.

As a hard feeder link switch entails the satellite severing the connection with the former serving gNB, all connected UEs must be simultaneously transition to the new gNB once the new feeder link is established. Given the large cell sizes and possible user density in NTN, this could lead to a RACH storm with frequent RACH collision. As well, given the increased propagation delay in NTN the signalling required to perform RRC re-establishment to a new gNB (e.g. RLF detection, cell selection, potential re-establishment failure) may introduce significant signalling latency and possible service interruption.

Observation 1: A hard feeder link switch can result in all connected mode UEs served by the satellite attempting mobility simultaneously, leading to RACH collisions, RLF and service interruption due to cumulative delay in RRC re-establishment signalling.

The TR captures possible methods to mitigate these issues for hard feeder link switch. For example, the HO command may be sent to all UEs prior to the switch with an activation time included in the HO command to properly time the HO to after the feeder link switch. However, synchronizing a potentially very large number UEs to perform handover without collision introduces additional complexity, in addition to service interruption for UEs which are scheduled to perform HO later.

To compensate for the portion of UEs which may not be able to perform handover, the TR further mentions that the network may provide additional information to the UE to reduce time for RRC re-establishment such as next cell identity. However, based on current Rel-16 specification this may assume that there is an interface between the target gNB and last serving gNB, for example, to retrieve UE context data. As simultaneous feeder links are not supported in a hard switch, the only method for gNBs exchange this information is via land-based connection. Since the source and target gNBs may be located very far away (e.g. on different continents) the exchange of information may also take significant time, should a land-based connection even exist. This may result at best possible service interruption to the UE.

Observation 2: Synchronizing UEs to perform HO without collision introduces complexity and additional signalling in the HO command. Providing assistance data to aid RRC re-establishment may assume a land-based connection between source and target gNBs, which cannot be guaranteed.

Alternatively, during a soft feeder link switch the satellite broadcasts cells from different gNBs (via different land based GWs), each cell may be associated with a separate PCI. To differentiate the two cells, the TR notes that the target gNB may use different synchronization raster points for CD-SSBs to allow the UE to differentiate the cells. This method would provide much less impact to current specifications.

Observation 3: Soft feeder link switch can support unique PCIs for cells from the source and target gNBs to be simultaneously relayed through the same satellite. The UE can distinguish the cells by different synchronization raster points for CD-SSBs.

Observation 4: Soft feeder link switch has less impact to current specification.

Given the lesser impact to current specifications and reduced possibility of service interruption, RLF, complexity and signalling overhead, it is therefore proposed that soft feeder link switch be supported in Rel-17 NTN.

Proposal 1: Rel-17 supports soft feeder link switch for transparent LEO NTN.

# Conclusion

In this contribution the following observations and proposals were made concerning feeder link switch in NTN:

Observation 1: A hard feeder link switch can result in all connected mode UEs served by the satellite attempting mobility simultaneously, leading to RACH collisions, RLF and service interruption due to cumulative delay in RRC re-establishment signalling.

Observation 2: Synchronizing UEs to perform HO without collision introduces complexity and additional signalling in the HO command. Providing assistance data to aid RRC re-establishment may assume a land-based connection between source and target gNBs, which cannot be guaranteed.

Observation 3: Soft feeder link switch can support unique PCIs for cells from the source and target gNBs to be simultaneously relayed through the same satellite. The UE can distinguish the cells by different synchronization raster points for CD-SSBs.

Observation 4: Soft feeder link switch has less impact to current specification.

Proposal 1: Rel-17 supports soft feeder link switch for transparent LEO NTN.

# References

- RP-193234, “Solutions for NR to support non-terrestrial networks (NTN)”, Thales

- 3GPP TR38.821 V16.0.0, “Solutions for NR to support non-terrestrial networks (NTN)”