3GPP TSG RAN WG1 102-e							R1-2006623

e-meeting, 17th Aug – 21st Aug 2020

Agenda item:	8.5.2

Source: 	CEWiT, Tejas Networks, Reliance Jio, IITM, Saankhya Labs, IITH

Title: 	Positioning evaluation results for additional commercial use cases

Document for:	Discussion and Decision

- Introduction

In Rel 16 initial positioning support is provided with both RAT based, and non-RAT based positioning techniques. In RAN#86 meeting, the study item on NR positioning enhancements was approved under release 17 [1]. From RAN1’s perspective, the SI includes the following objectives:

- Study enhancements and solutions necessary to support the high accuracy (horizontal and vertical), low latency, network efficiency (scalability, RS overhead, etc.), and device efficiency (power consumption, complexity, etc.) requirements for commercial uses cases (incl. general commercial use cases and specifically (I)IoT use cases as exemplified in section 3 above (Justification)):

  - Define additional scenarios (e.g. (I)IoT) based on TR 38.901 to evaluate the performance for the use cases (e.g. (I)IoT). [RAN1]

  - Evaluate the achievable positioning accuracy and latency with the Rel-16 positioning solutions in (I)IoT scenarios and identify any performance gaps. [RAN1]

  - Identify and evaluate positioning techniques, DL/UL positioning reference signals, signalling and procedures for improved accuracy, reduced latency, network efficiency, and device efficiency.
Enhancements to Rel-16 positioning techniques, if they meet the requirements, will be prioritized, and new techniques will not be considered in this case. [RAN1, RAN2]

NOTE 1:	Sidelink is not part of this objective.

NOTE 2:	Involve RAN4 for validating assumptions for the systems evaluations where appropriate.

NOTE 3:	The commercial use cases and requirements are applicable to a limited geographic area.

Rel 17 study focuses on the more advance commercial use cases rather than just satisfying the public safety requirements as in case of legacy positioning support in 3GPP. One such commercial use case is the Industrial IoT (IIoT) scenario agreed in RAN1 #101-e with more stringent positioning requirement in terms of accuracy, latency, and power consumption. In RAN 1 #101-e few agreements on commercial use case requirements are achieved. Further in RAN 1 #101-e evaluation parameters are finalized for new commercial use cases including IIoT. As mentioned in SID, main agenda behind the evaluation study is to find out the performance of Rel 16 positioning techniques for new use cases and understand the performance gaps for envisioned requirement in the study. The present contribution provides the initial evaluation results for Rel 17 commercial use case.

- Evaluation of InF scenarios:

In RAN1 #101-e meeting, two indoor factory scenarios are chosen for the positioning study based on the actual InF scenario can be experienced in practice. They are InF-SH and InF-DH as per following agreement.

Agreement:

- InF-SH and InF-DH models in TR 38.901 are adopted as the baseline scenarios for defining the channel models, parameters, and modelling techniques for performance evaluations in the Rel. 17 positioning enhancements at least for IIoT use cases

- Note: Modifications to parameters in the InF-DH models will be discussed separately.

- Note: Target performance and performance gap identification will be discussed separately.

- Note: Individual companies may consider additional InF models in TR 38.901 as complementary evaluation scenarios in their simulation investigation and the evaluation results can be considered to be captured in the TR 38.857.

Note: Target positioning requirements may not necessarily be reached for all scenarios

Common parameters and InF specific parameters table is agreed for simulation performance provided in annexure a) and b) which are used for simulation.

  - InF-SH scenario:

In InF-SH, sparse clutter density e.g. machinery, assembly lines, storage shelves, etc. is assumed and the deployed gNB/hotspot height is assumed above the effective clutter height.  Therefore, there

Figure 1: InF-SH LOS probability distribution

is better chance of the LOS path as shown in the fig 1 compared to other InF scenarios. We evaluated the timing-based positioning technique in DL i.e. DL-TDOA with different bandwidth of PRS as per Rel 16.  PRS is repeated over 4 symbols with staggering pattern. Fig 2 shows cumulative distribution of positioning error for different bandwidth. Similarly, the table 1 shows the positioning accuracy achieved for various percentile values i.e. 50,67,80,90 and 95%tile values.

Figure 2: InF-SH positioning error CDF with DL-TDOA method (a) with baseline paramters (b) with LOS links

Table 1: Positioning accuracy for different bandwidth

|  | Baseline |  |  |  |  | Forced LOS |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Bandwidth | 50% | 67% | 80% | 90% | 95% | 50% | 67% | 80% | 90% | 95% |
| 20MHz | 2.31m | 3.52m | 4.9m | 8.95m | - | 2.31m | 3.5m | 4.9m | 8.9m | - |
| 50Mhz | 1.23m | 1.62m | 2.32m | 3.73m | 6.13m | 1.22m | 1.63m | 2.7m | 4.6m | 7.5m |
| 100MHz | 0.6m | 0.85m | 1.41m | 1.78m | 4.18m | 0.63m | 0.95m | 1.34m | 2.21m | 4.9m |
| 200MHz | 0.3m | 0.52m | 0.95m | 2.70m | 4.18m | 0.32m | 0.5m | 0.96m | 1.77m | 3.2m |

Additionally, we evaluated the carried aggregate PRS transmission with 200MHz band (100MHz + 100MHz). It can be observed from the fig 2 that positioning error decreases with increase in bandwidth. Therefore, PRS bandwidth is critical parameter to achieve the desired accuracy requirements. Table 1 shows the cases in green where <1m accuracy error requirement can be achieved. This simulation is with only time arrival measurement and it suggest that on DL-TDOA is not sufficient to achieve the desired accuracy in Rel 17. Hybrid method should be used for improvising the positioning accuracy. Similarly, it can be observed that in no much performance different in baseline and forced LOS case. This is due to the clutter parameter chosen for the InF SH scenario.

  - InF-DH scenario:

In InF-DH, dense clutter density is assumed, and the deployed gNB/hotspot height is assumed above the effective clutter height.  Here again, there is better chance of the LOS path as shown in the fig 3.

Figure 3: InF-DH LOS probability distribution for different

We evaluated the timing-based positioning technique in DL i.e. DL-TDOA with different bandwidth of PRS as per Rel 16.  PRS is repeated over 4 symbols with staggering pattern. Fig 4 shows cumulative distribution of positioning error for different bandwidth. Similarly, the table 2 shows the positioning accuracy achieve for various percentile values i.e. 50, 67, 80, 90 and 95%tile values.

Figure 4: InF-DH positioning error CDF with DL-TDOA method

Table 2: Positioning accuracy for different bandwidth in InF-DH

|  | Baseline |  |  |  |  | Forced LOS |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Bandwidth | 50% | 67% | 80% | 90% | 95% | 50% | 67% | 80% | 90% | 95% |
| 20MHz | 2.47m | 3.3m | 5.2m | 9.5m | - | 2.41m | 3.13m | 5.2m | 9.5m | - |
| 50Mhz | 1.03m | 1.63m | 2.14m | 3.2m | 8.9m | 1.45m | 1.65m | 2.23m | 3.36m | 6.55m |
| 100MHz | 0.61m | 0.96m | 1.4m | 1.9m | 3.2m | 0.62m | 0.88m | 1.22m | 1.65m | 2.25m |
| 200MHz | 0.35m | 0.55m | 0.84m | 1.37m | 2.0m | 0.3m | 0.37m | 0.55m | 0.99m | 1.58m |

In this case also, we evaluated the carried aggregate PRS transmission with 200MHz band (100MHz + 100MHz). It can be observed from the fig. 4 that positioning error decreases with increase in bandwidth. Therefore, PRS bandwidth is critical parameter to achieve the desired requirements. Table 2 shows the cases in green where <1m accuracy error requirement can be achieved. In InF-DH case, the LOS path-based positioning will provide improvement over baseline.

Observation 1: Bandwidth of PRS is a critical parameter to define the accuracy of positioning in both the IIoT scenarios. Similarly, determining the LOS path will improve the accuracy of position at least in case of InF-DH scenario.

Observation 2: DL-TDOA will not satisfy the accuracy requirement for InF-SH and InF-DH for 80% tile of UEs or more. Need to perform the hybrid detection for improve the accuracy to achieve this.

Proposal 1: LOS path detection and hybrid positioning techniques should be studied in positioning enhancement study.

- Effect of network synchronization error on positioning

Network synchronization is one of the major issues in RAT-based positioning.

Following result shows that if clock at gNBs is mismatched then even having a large bandwidth doesn’t help. It is required that the gNBs/TPs participating in the positioning should be synchronised perfectly. At present, the GNSS based and ethernet based synchronization (syncE or PTP) techniques are used in practice. For Rel 17 scenarios, the required network synchronization is in nano-seconds rage because e.g. if network synchronization error is even 50ns then maximum achievable position accuracy will be limited to 50 ns x 3x108= 15 m. Therefore, this study is important in Rel 17. Below fig. 5 provides the comparison of between position error CDF with synchronization error and without synchronization error for 50 MHz bandwidth in InF-DH scenario. It can clearly be observed that the synchronization error degrades the accuracy significantly. Similarly, Table 3 provides the error performance for different percentile values. It is clearly visible that the positioning performance degradation is up to 45 percentage. (Note: assumed sync error model as truncated Gaussian with std 100ns and T2= 100ns ) Therefore, network synchronization error techniques should be studied in Rel 17 to achieve required accuracy. One such technique is discussed in our companion contribution [4]

Figure 5: InF-DH positioning error CDF with and without synchronization error

| InF-DH | Positioning Error without Sync Error |  |  |  |  | Positioning Error with Sync Error |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Bandwidth | 50% | 67% | 80% | 90% | 95% | 50% | 67% | 80% | 90% | 95% |
| 20MHz | 2.41m | 3.13m | 5.2m | 9.5m | - | 38.25m | 46.38m | 53.38m | 59m | 65.41m |
| 50Mhz | 1.45m | 1.65m | 2.23m | 3.36m | 6.55m | 39.15m | 43.82m | 49.99m | 56.48m | 61.05m |
| 100MHz | 0.62m | 0.88m | 1.22m | 1.65m | 2.25m | 28.81m | 38.67m | 46.71m | 53.61m | 58.95m |

Table 3: InF-DH positioning error percentile with and without synchronization error

Observation 3: Network synchronization error is critical factor in Rel 17 positioning enhancement as it degrades the positioning accuracy significantly. Tight synchronisation close to ideal is necessary for Rel 17 scenarios.

Proposal 2: Network synchronization error techniques should be studied in Rel 17 to achieve required accuracy.

- Conclusions

Following are the observation and proposals for positioning enhancement in Rel 17 proposed in present contribution.

Observation 1: Bandwidth of PRS is a critical parameter to define the accuracy of positioning in both the IIoT scenarios. Similarly, determining the LOS path will improve the accuracy of position at least in case of InF-DH scenario.

Observation 2: DL-TDOA will not satisfy the accuracy requirement for InF-SH and InF-DH for 80% tile of UEs or more. Need to perform the hybrid detection for improve the accuracy to achieve this.

Observation 3: Network synchronization error is critical factor in Rel 17 positioning enhancement as it degrades the positioning accuracy significantly. Tight synchronisation close to ideal is necessary for Rel 17 scenarios.

Proposal 1: LOS path detection and hybrid positioning techniques should be studied in positioning enhancement study.

Proposal 2: Network synchronization error techniques should be studied in Rel 17 to achieve required accuracy.

- References

[1] RP-193237 New SID on NR Positioning Enhancements

[2] TS 38.855 Study on NR positioning support (Release 16)

[3] TS 38.901 Study on channel model for frequencies from 0.5 to 100 GHz (Release 16)

[4] R1-2006621 Discussion on positioning enhancements for Rel 17

- Annexure

  - Common Evaluation parameters

|  | FR1 | FR2 |
| --- | --- | --- |
| Carrier frequency, GHz | 3.5GHz | 28GHz |
| Bandwidth, MHz | 100MHz additional: 20,50 and 200 MHz | 400MHz |
| Subcarrier spacing, kHz | 30kHz | 120kHz |
| gNB model parameters |  |  |
| gNB noise figure, dB | 5dB | 7dB |
| UE model parameters |  |  |
| UE noise figure, dB | 9dB | 13dB |
| UE max. TX power, dBm | 23dBm | 23dBm EIRP should not exceed 43 dBm. |
| UE antenna configuration | dH = 0.5λ, (M, N, P, Mg, Ng) = (1, 2, 2, 1, 1) | Baseline: Multi-panel Configuration 1 and Panel Configuration a – Note 1 - Multi-panel Configuration 1: (Mg, Ng) = (1, 2); Θmg,ng=90°; Ω0,1=Ω0,0+180°; (dg,H, dg,V)=(0,0) - Panel Configuration a: - Each antenna array has shape dH=dV=0.5λ - Config a: (M, N, P) = (2, 4, 2), - the polarization angles are 0° and 90° - The antenna elements of the same polarization of the same panel is virtualized into one TXRU |
| UE antenna radiation pattern | Omni, 0dBi | Antenna model according to Table 6.1.1-2 in TR 38.855 |
| PHY/link level abstraction | Explicit simulation of all links, individual parameters estimation is applied. Companies to provide description of applied algorithms for estimation of signal location parameters. |  |
| Network synchronization | The network synchronization error, per UE dropping, 1. T1= 0ns (perfect synchronization) 2. T1= 50ns, sigma = 100ns and T2 = 100ns |  |

  - InF specific parameters

|  |  | FR1 | FR2 |
| --- | --- | --- | --- |
| Channel model |  | InF-SH, InF-DH | InF-SH, InF-DH |
| Layout | Hall size | InF-SH: 300x150 m InF-DH: 120x60 m |  |
|  | BS locations | 18 BSs on a square lattice with spacing D, located D/2 from the walls. - for the small hall (L=120m x W=60m): D=20m - for the big hall (L=300m x W=150m): D=50m |  |
|  | Room height | 10m |  |
| Total gNB TX power, dBm |  | 24dBm | 24dBm EIRP should not exceed 58 dBm |
| gNB antenna configuration |  | (M, N, P, Mg, Ng) = (4, 4, 2, 1, 1), dH=dV=0.5λ | (M, N, P, Mg, Ng) = (4, 8, 2, 1, 1), dH=dV=0.5λ – Note 1 One TXRU per polarization per panel is assumed |
| gNB antenna radiation pattern |  | Single sector | 3-sector antenna configuration |
| Penetration loss |  | 0dB |  |
| Number of floors |  | 1 |  |
| UE horizontal drop procedure |  | Uniformly distributed over the horizontal evaluation area for obtaining the CDF values for positioning accuracy, the evaluation area should be at least the convex hull of the horizontal BS deployment. It can also be the whole hall area if the CDF values for positioning accuracy is obtained from whole hall area. |  |
| UE antenna height |  | 1.5m |  |
| UE mobility |  | 3km/h, additionally 10km/h |  |
| Min gNB-UE distance (2D), m |  | 0m |  |
| gNB antenna height |  | 8m |  |
| Clutter parameters: {density r, height hc,size dclutter} |  | Low clutter density: {20%, 2m, 10m} High clutter density: {40%, 2m, 2m} |  |

  - Parameters details for FR1 simulations

| Parameter | InF-SH | InF-DH |
| --- | --- | --- |
| Channel model (baseline, otherwise state any modifications) | 38.901 | 38.901 |
| Reference Signal Physical Structure and Resource Allocation (RE pattern) | Comb 3 with staggering pattern over 4 symbols | Comb 3 with staggering pattern over 4 symbols |
| Reference signal (type of sequence, number of ports, …) | Rel 16 DL-PRS | Rel 16 DL-PRS |
| Number of sites | 18 | 18 |
| Number of symbols used per slot per positioning estimate | 4 | 4 |
| Number of slots per positioning estimate | 1 | 1 |
| Power-boosting level | 0dB | 0dB |
| Uplink power control (applied/not applied) | Not applicable | Not applicable |
| interference modeling (ideal muting, or other) | Ideal muting | Ideal muting |
| Description of Measurement Algorithm (e.g. super resolution, interference cancellation, ….) | Newton Raphson and Gradient descent | Newton Raphson and Gradient descent |
| Description of positioning technique / applied positioning algorithm (e.g. Least square, Taylor series, etc) | DL-TDOA | DL-TDOA |
| Network synchronization assumptions | 1. 0ns 2. 50 ns | 1. 0ns 2. 50 ns |
| Beam-related assumption (beam sweeping / alignment assumptions at the tx and rx sides) | N/A | N/A |
| Precoding assumptions (codebook, nr of antenna elements used, etc) | (M, N, P, Mg, Ng) = (4, 4, 2, 1, 1) one TXRU per polarization per panel | (M, N, P, Mg, Ng) = (4, 4, 2, 1, 1) one TXRU per polarization per panel |
| Additional notes, if any |  |  |