3GPP TSG-RAN WG1 #102		 	R1-2006628

e-Meeting, August 17th – August 28th, 2020

Agenda item:	8.2.1

Title:	On NR operation between 52.6 GHz and 71 GHz

Source:	Convida Wireless

Document for:	Discussion

# Introduction

In RAN #88e, the objectives of the study item for supporting NR from 52.6 GHz to 71 GHz include [1][2]

- Candidate numerology (SCS, and CP length) to be supported by RAN1 specification.

  - Discussions may include how RAN1 should conclude on determination of the candidate numerologies

  - Discussion may also include identification of any coupling with other system parameters, such as bandwidth (number of PRB), FFT size, etc.

- Candidate bandwidths (or range of bandwidth) to be supported by RAN1 specification and related considerations (e.g. maximum FFT size)

  - Discussions may include how RAN1 should conclude on determination of the candidate bandwidths

- Candidate numerology (SCS, and CP length) to be supported by RAN1 specification.

  - Discussions may include how RAN1 should conclude on determination of the candidate numerologies

  - Discussion may also include identification of any coupling with other system parameters, such as bandwidth (number of PRB), FFT size, etc.

- Candidate bandwidths (or range of bandwidth) to be supported by RAN1 specification and related considerations (e.g. maximum FFT size)

  - Discussions may include how RAN1 should conclude on determination of the candidate bandwidths

In this contribution, we discuss several aspects of the operations of NR on frequencies from 52.6 GHz to 71 GHz.

# Supporting NR Operation from 52.6 GHz to 71 GHz

Frequency bands above 52.6 GHz have several advantages such as limited interference, wider bandwidth, high-gain beamforming/multi-beam systems, etc. Therefore, use cases like eMBB with high data rate, short range D2D communications with high data rate, wireless display transfer for AR/VR, etc. can be enabled.

To support NR from 52.6 GHz to 71 GHz, operation on both licensed and unlicensed spectrum with a range of ultra-wide-bandwidth frequency bands needs to be considered. For example, the whole 57 GHz – 71 GHz frequency range is available in USA, providing 14 GHz of contiguous bandwidth. In addition, the 66 GHz to 71 GHz band was recently opened for licensed use in international telecommunications union (ITU) Regions 1 and 3.

In RAN1 #101e, several companies suggested the subcarrier spacing (SCS) ranging from 120 KHz, 240 KHz, 480 KHz and 960 KHz (i.e., μ= 3, 4, 5 and 6) for NR operation both for licensed and unlicensed bands from 52.6 GHz to 71 GHz [4]- [6]. Besides, the assumed maximum channel bandwidth (BWchannel) for corresponding SCS/numerologies are 0.4 GHz, 0.8 GHz, 1.600 GHz, and 3.2 GHz, respectively. If higher subcarrier spacings (i.e., numerologies μ= 3, 4, 5 and 6) are adopted, then the maximum channel bandwidth that NR can support is limited to around 3 GHz for NR operation in frequency range of 52.6 GHz to 71 GHz. On the other hand, as pointed out by [7], NR should support different maximum possible bandwidth in different bands which requires configurable bandwidth. For example, multiples of 400 MHz which has been supported for FR2 can be considered up to 2 GHz.

In the following sections, we discuss the requirements for NR operation from 52.6 GHz to 71 GHz.

## Numerologies for NR from 52.6 GHz to 71 GHz

One of the major considerations for NR operation from 52.6 GHz to 71 GHz is the co-existence with other radio access technologies (RAT) such as IEEE WiFi 802.11 ad/ay. For example, 802.11 ad/ay may be operated from 57 GHz to 67 GHz in Europe and USA. In addition, the (minimum) channel bandwidth of 802.11 ad/ay is 2.16 GHz. Therefore, a suitable listen-before-talk (LBT) bandwidth for NR-U operation from 52.6 GHz to 71 GHz band may be around 2 GHz for co-existence with 802.11ad/ay. Assuming not changing the maximum FFT size (i.e. 4096), supporting higher SCS is beneficial to reach an LBT bandwidth of 2 GHz and mitigating the phase noise (PN). The other approach is via the carrier aggregation (CA) to accommodate the larger bandwidth (e.g. 2 GHz) with using the lower SCSs such as 120 KHz, 240 KHz and 480 KHz. For instance, multiples (e.g. 5) of 400 MHz with SCS = 120 KHz which has been supported for FR2 can be considered up to 2 GHz. Either using the larger SCS (i.e., SCS > 480 KHz) or the lower SCS (i.e., SCS ≤ 480 KHz) to achieve around 2 GHz has its own pros and cons.

In Rel-16 NR-U, LBT can be performed in units of 20 MHz. Furthermore, LBT can be performed for a single carrier with bandwidth greater than 20 MHz, i.e., integer multiples of 20 MHz. In other words, the channel bandwidth can be integer multiples of 20 MHz in Rel-16 NR-U. This condition can be satisfied for all supported numerologies (i.e., SCS=15 KHz, 30 KHz and 60 KHz) in frequency range 1 (FR1).  However, assuming LBT channel bandwidth of 2.16 GHz as in WiFi 802.11 ad/ay, this condition of channel bandwidth being integer multiple of LBT channel bandwidth cannot be met for some supported numerologies like SCS = 120 KHz with maximum BWchannel=400 MHz, 240 KHz with maximum BWchannel=800 MHz and 480 KHz with maximum BWchannel=1600 MHz.  Therefore, we propose to study the relationship between the supported numerologies with the CA and the LBT bandwidth (or the RB set) for supporting NR-U operation from 52.6 – 71 GHz. In addition, not only the increased scalable numerology but also the UE capability, processing time and power consumption need to be considered.

Proposal 1. To support various wide bands and use cases for NR operation from 52.6 GHz to 71 GHz, a wide range of numerologies with the carrier aggregation need to be studied, also considering the LBT bandwidth (or the RB set) for co-existing issues, UE capability, processing time and power consumption.

## Maximum channel bandwidth and carrier aggregation for NR operation from 52.6 GHz to 71 GHz

In this section, we briefly discuss some aspects of maximum channel BW and CA for NR operation form 52.6 GHz to 71 GHz.  There are four types of CA in NR can be supported, i.e., intra-band with contiguous or non-contiguous CA and inter-band with contiguous or non-contiguous CA. In addition, the nominal channel spacing can be found in [9] and [10] for FR1 and FR2, respectively. For example, the nominal channel spacing defined in [10] for intra-band contiguous CA in FR2 can be expressed as

Nominal channel spacing =BWchannel(1)+BWchannel(2)-2GBchannel(1)-GBchannel(2)2×0.06×2n0.06×2n,           (Eq. 1)

where  n=μ0-2 and  μ0=maxμchannel1,μchannel(2) and GBchannel(i=1,2) is the guard band bandwidth for channel 1 and 2. Form Eq. 1, the nominal channel spacing can be determined by each component carrier (CC) BWchannel(i), numerology and guard band information GBchannel(i). Assuming Eq. 1 can be extended for 52.6 GHz to 71 GHz CA case, there is a frequency gap may still need to be reserved between CC 1 and CC 2. To reduce the frequency gap between CC 1 and CC 2, a tighter channel spacing than the nominal spacing for any two single carriers can potentially be applied and lead to an increase in spectral efficiency. For example, one of approach for aggregated channel BW = 2 or 4 GHz is to aggregate 5 or 10 of 400 MHz with SCS = 120 KHz as shown in Figure 1 (a) and (b), respectively for NR-U bands from 57 GHz to 62 GHz. As shown in the example in Figure 1 (b), we reserve two guard bands at both sides of 2 aggregated bands. However, those guard bands between two adjacent 2 GHz bands may not be always necessary because WiFi 802.11ay support both the channel aggregation and the channel bundling. The channel bonding in WiFi 802.11ay usually refers to the merging of multiple contiguous channels into one wideband channel, and there is no channel spacing (or guard band) among multiple channels, which can be used as a whole band to form a single (wider) channel. Despite the guard band for CA with lower SCSs, the other factor for the impaction of the spectral efficiency is the phase noise (PN). It is well known that lower SCSs (e.g. SCS = 120 KHz, 240 KHz) with higher QAM (e.g. 64 and 256 QAM) are suffered from PN for NR operation from 52.6 – 71 GHz thus it requires to increase PT-RS density to maintain the EVM requirement addressed in [11].  Therefore, to support wide frequency range for NR operation from 52.6 GHz to 71 GHz using CA with lower SCS such as 120 KHz or 240 KHz, the spectral efficiency (including guard band, PT-RS overhead, etc.) for CA operation should be studied.

Proposal 2. The spectral efficiency (include guard band, PT-RS overhead, etc.) for large number of carrier aggregation should be studied for NR operation from 52.6 to 71 GHz.

Figure 1: An exemplary for aggregated channel BW for (a) 2 GHz (b) 4 GHz with SCS = 120 KHz and assuming the maximum BWchannel=400 MHz.

Another use case is using CA approach for supporting very wide frequency band. For example, to achieve 10 GHz aggregated channel BW, it requires at least 25 aggregated CCs if we assume each CC maximum BWchannel=400 MHz and its SCS = 120 KHz. Although NR allows more than 16 CCs can be aggregated, the signalling (e.g. PDCCH, etc.) overhead for scheduling large number of aggregated CCs for NR operation from 52.6 GHz to 71 GHz should also be studied for scheduling enhancement. In NR Rel-16, the fast activation/deactivation of SCell has been introduced. When UE switches to the dormancy BWP, it can achieve power saving by not monitoring PDCCH. Therefore, it behaves like deactivating the SCell. This mechanism of fast activation/deactivation of SCells could be a good starting point for scheduling enhancement for NR operation from 52.6 GHz to 71 GHz and above.

Proposal 3. The signal overhead for scheduling large number of aggregated carriers should be studied for NR operation from 52.6 to 71 GHz.

# Conclusion

In this contribution, we discussed the waveform consideration for supporting NR from 52.6 to 71 GHz. The following proposals were made:

Proposal 1. To support various wide bands and use cases for NR operation from 52.6 GHz to 71 GHz, a wide range of numerologies with the carrier aggregation need to be studied, also considering the LBT bandwidth (or the RB set) for co-existing issues, UE capability, processing time and power consumption.

Proposal 2. The spectral efficiency (include guard band, PT-RS overhead, etc.) for large number of carrier aggregation should be studied for NR operation from 52.6 to 71 GHz.

Proposal 3. The signal overhead for scheduling large number of aggregated carriers should be studied for NR operation from 52.6 to 71 GHz.

# References

- RP-200901, “Status Report to TSG: Study on supporting NR from 52.6 GHz to 71 GHz”, RAN #88e, Electronic Meeting, June 29 - July 3, 2020

- RP-200902, “Revised SID: New SID: Study on supporting NR from 52.6GHz to 71 GHz”, RAN #88e, Electronic Meeting, June 29 - July 3, 2020

- 3GPP TR 38.803, “Study on new radio access technology: Radio Frequency (RF) and co-existence aspects (Release 14)”, V14.2.0

- R1-2004247,	“On Required changes to NR using existing DL/UL NR Waveform”, Apple

- R1-2004489,	“Channel access mechanism for NR in 52.6 to 71GHz band”, Qualcomm Incorporated

- R1-2003849,	“On NR operations in 52.6 to 71 GHz”, Ericsson

- R1-2004299,	“Consideration on supporting above 52.6GHz in NR”, InterDigital, Inc.

- R1-2003812, “NR coexistence mechanisms for 60 GHz unlicensed band”, Nokia, Nokia Shanghai Bell

- 3GPP TS 38.101-1 User Equipment (UE) radio transmission and reception; Part 1: Range 1 Standalone (Release-16), V16.2.0

- 3GPP TS 38.101-2 User Equipment (UE) radio transmission and reception; Part 2: Range 2 Standalone (Release-16), V16.2.0

- 3GPP TS 38.104, “NR; Base Station (BS) radio transmission and reception (Release 16)”, V16.3.0