3GPP TSG RAN WG1 #102	                         	R1-2006646

e-Meeting, August 17th – 28th, 2020

Agenda Item:	7.2.11

Source: 	LG Electronics

Title: 	Discussion on NR Rel-16 UE features

Document for:	Discussion and decision

- Introduction

In previous RAN1 meeting [1], RAN1 has updated and endorsed NR Rel-16 UE features list as in [2]. Still, there are remaining points to be decided on further details of NR Rel-16 UE features. In this paper, we share LG’s views on the further updates of NR Rel-16 UE features, especially dedicated to NR Rel-16 items of IAB, MIMO and V2X.

- Discussion

  - UE features for IAB

During last e-meeting, most of UE features list for IAB was managed, which is summarized in [3], and the agreement was settled up in [1]. However no agreement was made up about whether FG 20-2 and FG 20-3 to be optional or mandatory. Therefore, we provide following suggestions for this remaining issues.

FG 20-2) Support up to 4 SMTCs configured for an IAB node MT per frequency location, including IAB-specific SMTC window periodicities

    - The variance of coverage of Rel-16 IAB node will diverge since the main objective of Rel-16 IAB is coverage enhancement, which means that flexibility and robustness in deployment should be guaranteed. At the same time, multi-hop and multi-parent scenario with TDMed IAB-DU and IAB-MT should be supported while the measurement of SSB for IAB-MT is not protected while the transmission of SS/PBCH block is protected by hard configuration of corresponding resources. One of the simplest way to keep measurement of parent IAB-DU’s SSB by IAB-MT is to configure multiple SMTCs to a single IAB-MT. Thus, supporting up to 4 SMTCs for IAB node is desirable to be a mandatory feature.

FG 20-3) Support RACH configuration for IAB-MT separately from the RACH configuration for UE access, including new IAB-specific offset and scaling factors

    - As pointed out in RAN1 #101-e, it is expected that the capability and channel between IAB node and access UE is different, which implies, sharing same RACH resource within IAB-MT and access UE will impact the network performance. Additionally, FG 20-3 is very efficient way in the resource management of view since less RACH resources will be needed for IAB node compared to access UE due to the uniqueness of IAB itself. However, it is rather enhancing than enabling. Thus, FG 20-3 is desirable to be optional.

Proposal 1) Adopt the RAN1 UE feature list for NR IAB below.

| 20-2 | Inter-IAB-node discovery and measurements: SSB reception configuration | Support up to 4 SMTCs configured for an IAB node MT per frequency location, including IAB-specific SMTC window periodicities |  | Yes | N/A | Separate configuration of SMTC windows for Inter-IAB node discovery and measurement is not possible | per IAB node | No | No | support mixture of FDD/TDD and/or FR1/FR2 | IAB-MT impact | Optional with capability signalling. [Devices supporting IAB backhaul must report this FG as supported] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 20-3 | Extension of RACH occasions and periodicities for backhaul RACH resources | Support RACH configuration for IAB-MT separately from the RACH configuration for UE access, including new IAB-specific offset and scaling factors |  | Yes | N/A | Separate configuration of RACH transmissions for access UEs and IAB nodes is not possible | per IAB node | No | No | support mixture of FDD/TDD and/or FR1/FR2 | IAB-MT impact | Optional with capability signalling. [Devices supporting IAB backhaul must report this FG as supported] |

  - UE features for MIMO

For NR UE features for Rel-16 eMIMO, there are still a number of remaining issues after email discussion [101-e-Post-NR-UE-Features-10]. Our views on those are as follows:

    - FG 16-1a-1

      - We’d like to suggest aligning signaling method on how to indicate what UE can or cannot support for L1-SINR measurement, i.e., via adding candidate value of 0 into a component or via adding that L1-SINR measurement into component 8. If candidate value of 0 is added to component 2 and 5, meaning that UE can report whether or not to support ‘CMR with dedicated IMR’ using this value, then it looks somehow duplicated with ‘SSB as CMR with dedicated IMR’ and ‘CSI-RS as CMR with dedicated IMR’ using component 8. Easiest way to align signaling methods may be deleting candidate values of 0 from component 2, 3 and 5, then keeping ‘CSI-RS (2Tx) resources for CMR’ into component 8.

    - FG 16-1g

      - During email discussion [101-e-Post-NR-UE-Features-10], a note “If the UE sets a value other than 0 in an FR1 band, it shall set that same value in all FR1 bands. If the UE sets a value other than 0 in an FR2 band, it shall set that same value in all FR2 bands. The UE supports a total number of resources equal to the maximum of the FR1 and FR2 value, but no more than the FR1 value across all FR1 serving cells and no more than the FR2 value across all FR2 serving cells” was proposed to accommodate inter-band CA/DC cases. Since this report is for ‘across all CCs’, ‘per BC’ or ‘per UE’ reporting is more reasonable than ‘per band’ to address inter-band CA/DC cases rather than adding the note and making the reporting granularity as ‘per band’. Considering that FR1 and FR2 may have different values, we suggest ‘per BC’ reporting for this FG.

      - Editorial suggestion: update the feature description to ‘Resources for beam management, pathloss measurement, BFD, RLM, and new beam identification for BFR’ to be aligned with updated component

    - FG 16-2c

      - Editorial suggestion for the first sentence and the feature description: Simultaneous reception with different QCL Type-D RSs

      - For the yellow part, there were proposals to enhance multi-PDCCH reception behavior according to UE capability but it is still a pending issue. To our understanding, if we delete the second sentence, it would automatically apply to the supported features in Rel-16. If there is no consensus on enhancement for PDCCH, this UE capability report would apply to PDSCH only, and if there is an enhancement for PDCCH in Rel-16, this UE capability report would apply to both PDSCH and PDCCH. So, to move forward considering very limited time for UE capability discussion, it would be considerable to delete the second sentence.

    - FG 16-2b-5

      - Support to delete the square brackets, and candidate values of 1 and 2.

    - FG 16-3a and 16-3b

      - Component 4 in 16-3a and 3b is not needed since the parameter combinations in component 2 already contain the supported L value. Moreover, the component may restrict the support on some parameter combinations depending on the number of Tx ports.

    - FG 16-5b

      - Our first preference is removing second bullet, since it is supported by Rel-15 UE capability (FG 2-13). If majority companies want to keep the second bullet for flexible UE capability reporting, we could live with second bullet.

    - FG 16-5c-2

      - Support deleting 1st bullet and keep 2_4 in the 2nd bullet.

    - FG 16-5c-3

      - During the email discussion [101-e-Post-NR-UE-Features-10], a note was added for the clarification purpose, and we are fine with this clarification.

Proposal 2) Adopt the RAN1 UE feature list for eMIMO below.

| 16-1a-1 | SSB/CSI-RS for L1-SINR measurement | Per slot limitations: - The max number of [unique] SSB/CSI-RS [(1Tx)] for CMR - The max number of CSI-IM/NZP-IMR resources - The max number of CSI-RS (2Tx) resources for CMR Memory limitations: - The max number of SSB/CSI-RS resources as CMR - The max number of CSI-IM/NZP IMR resources Other limitations: - Supported density of CSI-RS (CMR) - The max number of aperiodic CSI-RS resources across all CCs configured to measure L1-SINR (including CMR and IMR) shall not exceed MD_1 - Supported SINR measurements: {SSB as CMR with dedicated IMR, CSI-RS as CMR with dedicated [CSI-IM/NZP IMR] configured, CSI-RS as CMR without dedicated IMR configured, [CSI-RS (2Tx) resources for CMR]} | 2-21, 2-22 or 2-23, 2-23a | Yes | N/A |  | Per band | No | No |  | Component 1: Candidate values {8, 16, 32, 64} Component 2: Candidate values {[0,] 8, 16, 32, 64} Component 3: Candidate values {[0, 4,] 8, 16, 32, 64} Component 4: Candidate values {[8,] 16, 32, 64 [, 128]} Component 5: Candidate values {[0,] 8, 16, 32, 64 [, 128]} Component 6: Candidate values {‘1 only’, ‘3 only’, ‘1 and 3’} Component 7: Candidate values {[0, 1, 2, 4,] 8, 16, 32, 64} Component 8: Candidate values FFS Note: For Component 8, UE must at least report support of one [FFS: which one(s)] FFS: How CSI-RS is counted when it is configured as CMR without dedicated IMR |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 16-1g | Resources for beam management, [pathloss measurement, BFD, RLM, and new beam identification for BFR] | - The maximum number of [unique] SSB/CSI-RS/CSI-IM resources configured to measure within a slot across all CCs for any of L1-RSRP measurement, L1-SINR measurement, [pathloss measurement, BFD, RLM] and new beam identification - The maximum number of SSB/CSI-RS/CSI-IM resources configured across all CCs for any of L1-RSRP measurement, L1-SINR measurement, [pathloss measurement, BFD, RLM] and new beam identification | 2-24, 2-31 | Yes | N/A |  | [Per band] [Per BC] [Per UE] | No | No |  | Component-1: candidate value set is {4, 8, 16, 32, 64, 128, FFS} [Component-2: candidate value set is {4, 8, 16, 32, 64, 128, 256, FFS}] |
| 16-2c | Simultaneous reception with different QCL Type-D RSs | Supports simultaneous reception with different QCL Type-D RSs [based on multiple spatial domain receiver filters]. This applies to [PDCCHs]/PDSCHs |  | Yes | N/A |  |  | N/A | FR2 only |  |  |
| 16-2b-5 | Single-DCI based inter-slot TDM | - Support of single-DCI based inter-slot TDM - Support of RepNumR16 in PDSCH-TimeDomainResourceAllocation and the maximum value of RepNumR16 - Supported maximum TBS size - [Maximum number of TCI states] |  | Yes | N/A |  |  | No | No |  | Component 2 candidate values: {{2,3,4,5,6,7,8,16}} Component 3 candidate values {{3, 5, 10, 20, no restriction} KByte } Component 4 candidate values: {1,2} |
| 16-3a | Regular eType-II | Basic components: - {Max # of Tx ports in one resource, Max # of resources and total # of Tx ports} to support regular eType-II for R=1 - Support of parameter combinations 1-6 - Support of rank 1,2 - [Number of beams L per CSI-RS ports] | 2-35 | Yes | N/A |  | Per band and per BC | N/A | N/A |  | Candidate values for component 1: - Maximum 16 triplets - Max # of Tx ports in one resource: {[2,] 4,8,12,16,24,32} - Max # resources: {1 to 64} - Max # total ports: {2 to 256} |
| 16-3b | Port selection eType-II | Basic components: - {Max # of Tx ports in one resource, Max # of resources and total # of Tx ports} to support port selection eType-II for R=1 - 6 parameter combinations (combos with L=6 don’t apply) - Support of rank 1,2 - [Number of beams L per CSI-RS ports] | 2-35 | Yes | N/A |  | Per band and per BC | N/A | N/A |  | Candidate values for component 1: - Maximum 16 triplets - Max # of Tx ports in one resource: {[2,] 4,8,12,16,24,32} - Max # resources: {1 to 64} - Max # total ports: {2 to 256} |
| 16-5b | UL full power transmission fullpowerMode1 | - Supported UL full power transmission fullpowerMode1 - [Number of Tx to support mode 1: {2Tx, 4Tx, 2Tx_4Tx}] | 2-13, 2-14 | Yes | N/A |  | Per FS | No | No |  |  |
| 16-5c-2 | UL full power transmission fullpowerMode2 – SRS resources | - [Number of Tx to support mode 2: {2Tx, 4Tx, 2Tx_4Tx}] - The SRS configuration with different number of antenna ports for Mode 2: {[NULL,] 1_2, 1_4, [2_4], 1_2_4} | 16-5c | Yes | N/A |  | Per FS | No | No |  |  |
| 16-5c-3 | UL full power transmission fullpowerMode2 – full power TPMI groups | - TPMI group(s) which delivers full power: {2-port {2-bit bitmap}, one 4-port non-coherent group from {G0~G3},one 4-port partial-coherent group from {G0~G6}, [FFS: 4-port full-coherent {G0~G6}]} | 16-5c | Yes | N/A |  | Per FS | No | No |  | Note: When a full coherent UE operates in mode 2, the way it reports TPMIs should be the same as a partial-coherent UE Note: For 4 port partial-coherent or full-coherent UE, UE can report: 2-port {2-bit bitmap} and one4-port non-coherent group from {G0~G3} and one 4-port partial-coherent group from {G0~G6} For 4 port non-coherent UE, UE can report: 2-port {2-bit bitmap} and 4-port non-coherent {G0~G3} For 2 port UE, UE can report: 2-port {2-bit bitmap} |

  - UE features for V2X

There are still a few remaining issues in the UE features for V2X after the email discussion [101-e-Post-NR-UE-Features-08] and [101-e-Post-NR-UE-Features-09]. As the LTE features can be updated by applying the conclusion of NR features straightforwardly, we focus on the remaining NR UE feature issues for V2X in the following:

    - [15-2] Transmitting NR sidelink mode 1 scheduled by NR Uu

On mandating this FG for a UE operating SL in a licensed band, we think that the operator needs to have a choice between mode 1 and mode 2 and mandating only mode 2 may not be a good solution for the market. We note that a similar discussion in Rel-12 LTE D2D led to a conclusion that both modes are mandated for a UE supporting sidelink (all the spectrum was under operator management at that time as no V2X was considered).

On mandating an SCS in a band “indicated with the PC5 interface in 38.101-1 Table 5.2E-1,” we observe that Component 12 of FG 15-1 always mandates 30 kHz and 120 kHz with normal CP in FR1 and FR2 respectively for the SL reception in such a band. We think it is reasonable to mandate the same implementation in this FG.

On mandating Component 9 of DL pathloss based power control in a band “indicated with the PC5 interface in 38.101-1 Table 5.2E-1,” we think the note should be aligned with a similar one in FG 15-3 “Note: Component 11 (DL pathloss based open loop power control when mode 2 is configured by NR Uu) is not required to be supported in a band indicated with only the PC5 interface in 38.101-1 Table 5.2E-1.”

    - [15-3] Transmitting NR sidelink mode 2

This FG should be the basic FG in in ITS spectrum where gNB is not defined. In a licensed spectrum where gNB is defined, this FG still can be the basic FG by reusing the implementation for the ITS spectrum.

    - [15-5] Sidelink congestion control

Congestion control is one of the fundamental building block both in mode 1 and mode2, so we think this FG should be the basic FG.

    - [15-6] Short-term time-scale TDM for in-device coexistence

The issue of “2) FFS: Maximum time required for the inter-RAT conflict resolution is X” can be resolved by simply deleting this component from the UE feature list. The value of X can be directly specified by changing the following part of Section 16.2.4.1 of TS 38.213:

If a UE - would transmit a first channel/signal using E-UTRA radio access and a second channel/signal using NR radio access, and - a transmission of the first channel/signal would overlap in time with a transmission of the second channel/signal, and - the priorities of the two channels/signals are known to the UE T msec prior to the start of the earlier of the two transmissions the UE transmits only the channel/signal with the higher priority as determined by the SCI formats scheduling the transmissions or, in case of a S-SS/PSBCH block or a sidelink synchronization signal using E-UTRA radio access, as indicated by higher layers or, in case of PSFCH, equal to the priority of the corresponding PSSCH.

A UE can perform transmission and reception simultaneously when they take place in different band, but still this FG may be implemented in such a band combination in order to make use of the specified prioritization rule. We think the issue of different frequency bands can be resolved by writing the consequence of not supporting this feature as follows: “UE only supports the long-term time-scale TDM for the coexistence with LTE V2X sidelink in the same band. UE only supports FDM for the coexistence with LTE V2X sidelink in a different band.” With this update, the absence of this FG will lead to the use of long-term time-scale TDM and FDM in the same band case and different band case, respectively.

    - [15-11] PSFCH format 0

It was discussed whether gNB or another UE need to know the capability of PSFCH TX/RX of a UE. As this FG is a basic FG, if this FG is signaled to gNB or another UE, its usage would be to adapt its operation based on the number of PSFCH TX and RX supported by the signaling UE. If reported to gNB, the gNB can configure the resource pool and/or schedule the sidelink grant in consideration of the reports. For example, gNB may select more frequent PSFCH slot to avoid too many FDM of PSFCH transmissions in the resource pool if many UEs reported that they have relative lower capability in terms of the simultaneous PSFCH reception. For a similar reason, gNB may decide to schedule sidelink grants with HARQ feedback enabled in the time slots separated enough if a problem is expected due to the excessive PSFCH transmissions to an RX UE in a slot otherwise.

We cannot find such a usage of reporting this capability to another UE. A UE can transmit only one PSSCH in a slot and this means that a UE is not required to send more than 4 PSFCHs to a specific UE. As a result, the minimum capability is already enough from the viewpoint of a unicast session.

    - [15-14] Sidelink CSI report

The usage of the sidelink CSI report is limited to enhance the performance of sidelink unicast in some limited operation scenarios such as the interference level is relatively stable, e.g., by gNB scheduling or configuration of the resources. We think this does not need to be a basic FG.

    - [15-18] Support of rank 2 transmission

The need of signaling this FG to another UE remains as an open issue. From the receiver viewpoint, the transmitter capability on rank 2 transmission is not necessary as the receiver needs to receive whatever the transmitter configures. In other words, there is no action the receiver can take after knowing the transmitter’s capability.

    - [15-19] Support of rank 2 reception

Similarly to FG 15-14, we think that this FG does not need to be a basic FG.

    - [15-22] Support of fewer than 14 consecutive sidelink symbols in a slot

This FG is not necessary for the ITS dedicated spectrum. It may be useful in some licensed spectrum operations but considering the possibility of reusing the implementation of the out-of-coverage operation, this FG does not need to be mandated even for the licensed spectrum.

    - [15-23] Support of open loop SL power control and RSRP report

The use of SL pathloss based sidelink power control is limited to unicast and it may cause adverse impact on some other scenarios; for example, the change of TX power may make it difficult to assume the same level of interference on a resource reserved by an SCI, so the overall mode 2 performance might be degraded. Our preference is not to mandate this FG in any case (including what was already included in the working assumption), but considering that the additional implementation burden is limited for this FG, if the underlying assumption in the working assumption is that gNB can avoid such potential adverse impact, confirming the working assumption without mandating this in any other case is acceptable.

Proposal 3) Adopt the RAN1 UE feature list for 5G V2X below.

| 15-2 | Transmitting NR sidelink mode 1 scheduled by NR Uu | 1) UE can transmit PSCCH/PSSCH using dynamic scheduling or configured grant type 1 and 2 in NR sidelink mode 1 scheduled by NR Uu. Up to 8 configured grants can be configured for a UE. Up to C sidelink HARQ processes are supported including those for configured grants 2) UE can transmit PSSCH according to the normal 64QAM MCS OFDM table. 3) UE supports PT-RS transmission in FR2. 4) UE can monitor DCI format 3_0 for NR sidelink dynamic scheduling and configured grant type 2. 6) UE can transmit using the subcarrier spacing and CP length it reports. 8) Supports 14-symbol SL slot with all DMRS patterns corresponding to {#PSSCH symbols} = {12, 9} for slots w/wo PSFCH. If UE signals support of ECP, support 12-symbol SL slot with all DMRS patterns corresponding to {#PSSCH symbols} = {10,7} for slots w/wo PSFCH. 9) Support downlink pathloss based open loop power control 11) UE can report sidelink HARQ-ACK to gNB via PUCCH and PUSCH when it is operating in NR sidelink mode 1 12) UE can transmit using 30 kHz and normal CP subcarrier spacing in FR1, 120 kHz subcarrier spacing with normal CP FR2 |  | Yes | No |  | Per band | N.A. | N.A. | N.A. | Note: Random selection in the exceptional pool is supported. This is the basic FG for sidelink in licensed spectrum where gNB is operating on or managing that spectrum and optional FG otherwise Candidate values for C are {8,16} Note: the UE supports up max(B, C) as the total number of sidelink HARQ processes across both Mode 1 and Mode 2 Component-6 candidate value set in FR1: {{15 kHz}, {30 kHz}, {60 kHz}, {15, 30 kHz}, {30, 60 kHz}, {15, 60 kHz}, {15, 30, 60 kHz}} Component-6 candidate value set in FR2: {{60 kHz}, {120 kHz}, {60, 120 kHz}} Component-6 candidate value set for CP length: {NCP,NCP and ECP} (ECP only applies to SCS of 60 kHz) Note: For Component 6, if a band is not indicated with only the PC5 interface in 38.101-1 Table 5.2E-1, the reported numerology shall be the same for sidelink and uplink. Component (9) is not required to be supported in a band indicated with only the PC5 interface in 38.101-1 Table 5.2E-1 Note: Component 11 is not required to be supported in a band indicated with the PC5 interface in 38.101-1 Table 5.2E-1 Note: Component 12 is only required in a band indicated with only the PC5 interface in 38.101-1 Table 5.2E-1 | Optional with capability signalling For UE supports NR sidelink in licensed spectrum where gNB is defined, UE must indicate this FG is supported. |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 15-3 | Transmitting NR sidelink mode 2 | 1) UE can transmit PSCCH/PSSCH using NR sidelink mode 2 configured by NR Uu or preconfiguration. Up to B sidelink processes are supported. 2) UE can transmit PSSCH according to the normal 64QAM MCS table. 3) UE supports PT-RS transmission in FR2. 4) UE can perform mode 2 sensing and resource allocation operations 6) UE can transmit using the subcarrier spacing and CP length it reports for FG 15-1 8) Supports 14-symbol SL slot with all DMRS patterns corresponding to {#PSSCH symbols} = {12, 9} for slots w/wo PSFCH. If UE signals support of ECP, support 12-symbol SL slot with all DMRS patterns corresponding to {#PSSCH symbols} = {10,7} for slots w/wo PSFCH. 10) UE can transmit using 30 kHz and normal CP subcarrier spacing in FR1, 120 kHz subcarrier spacing with normal CP FR2 11) DL pathloss based open loop power control when mode 2 is configured by NR Uu | 15-1 | Yes | No |  | Per band | N.A. | N.A. | N.A. | Note: Random selection in the exceptional pool is supported. Note: configuration by NR Uu is not required to be supported in a band indicated with only the PC5 interface in 38.101-1 Table 5.2E-1 This is the basic FG for sidelink Candidate values for B are {8,16} Note: the UE supports up max(B, C) as the total number of sidelink HARQ processes across both Mode 1 and Mode 2 Note: Component 6 is not required to be signalled in a band indicated with only the PC5 interface in 38.101-1 Table 5.2E-1 Note: Component 10 is only required in a band indicated with only the PC5 interface in 38.101-1 Table 5.2E-1 Note: Component 11 is not required to be supported in a band indicated with only the PC5 interface in 38.101-1 Table 5.2E-1 | Optional with capability signalling For UE supports NR sidelink, UE must indicate this FG is supported. |
| 15-5 | Sidelink congestion control | 1) UE can report CBR measurement to gNB when operating in Mode 1 and mode 2 2) UE can adjust its radio parameters based on CBR measurement and CRlimit. 3) UE can process CBR and CR within the time it indicates | 15-1 and at least one of 15-2 and 15-3 | Yes | No |  | Per band | N.A. | N.A. | N.A. | This is the basic FG for NR sidelink Note: component 1 is not required to be supported in a band indicated with only the PC5 interface in 38.101-1 Table 5.2E-1 Component-3 candidate value set {Congestion process time 1, Congestion process time 2} where Congestion process time 1: 2, 2, 4, 8 slots for 15, 30, 60, 120 kHz subcarrier spacing. Congestion process time 2: 2, 4, 8, 16 slots for 15, 30, 60, 120 kHz subcarrier spacing | Optional with capability signalling For UE supports NR sidelink, UE must indicate this FG is supported. |
| 15-6 | Short-term time-scale TDM for in-device coexistence | - Support prioritization between LTE sidelink transmission/reception and NR sidelink transmission/reception | At least one of 15-1, 15-2, 15-3 UE supports LTE V2X sidelink | No | No | UE only supports the long-term time-scale TDM for the coexistence with LTE V2X sidelink in the same band. UE only supports FDM for the coexistence with LTE V2X sidelink in a different band. | per band combination | N.A. | N.A. | N.A. |  | Optional with capability signalling |
| 15-11 | PSFCH format 0 | 1) UE can transmit and receive NR PSFCH format 0 2) UE can receive up to N PSFCH(s) resources in a slot. 3) UE can transmit up to M PSFCH(s) resources in a slot | At least one of 15-1, 15-3 | Yes | No |  | Per band | N.A. | N.A. | N.A. | This is the basic FG for sidelink. Note: configuration by NR Uu is not required to be supported in a band indicated with only the PC5 interface in 38.101-1 Table 5.2E-1 Candidate values for N are {5, 15, 25, 32, 35, 45, 50, 64} Candidate values for M are {4, 8, 16} | Optional with capability signalling For UE supports NR sidelink, UE must indicate this FG is supported. |
| 15-14 | Sidelink CSI report | 1) UE can transmit and receive sidelink CSI-RS with up to P antenna port(s). 2) UE supports RI and CQI feedback on sidelink. | 15-1 and at least one of 15-2 and 15-3 | No | Yes |  | Per band | N.A. | N.A. | N.A. | Note: Component 1 candidate values are P = {1,2} | Optional with capability signalling. |
| 15-18 | Support of rank 2 transmission | 1) UE additionally supports rank 2 PSSCH transmission | 15-14 with P=2 | No | No | UE supports rank 1 PSSCH transmission only. | Per band | N.A. | N.A. | N.A. | RAN1 does not see a need for the gNB to know if the feature is supported but would like to leave final decision to RAN2 | Optional with capability signalling |
| 15-19 | Support of rank 2 reception | 1) UE additionally supports rank 2 PSSCH reception | 15-1 | No | Yes | UE supports rank 1 PSSCH reception only. | Per band | N.A. | N.A. | N.A. | RAN1 does not see a need for the gNB to know if the feature is supported but would like to leave final decision to RAN2 | Optional with capability signalling |
| 15-22 | Support of fewer than 14 consecutive sidelink symbols in a slot | - UE additionally supports transmission/reception of SL slot configured with 7, 8, 9, 10, 11, 12, 13 consecutive symbols and all the corresponding DMRS patterns | At least one of 15-1, 15-2, 15-3 | Yes | No | UE supports SL only in a SL slot configured with 14 consecutive symbols. | Per band | N.A. | N/A | N.A. |  | Optional with capability signalling |
| 15-23 | Support of open loop SL power control and RSRP report | - Support sidelink pathloss based open loop power control and RSRP report in case of unicast | 15-1 and at least one of 15-2 and 15-3 | Yes | Yes |  | Per band | N.A. | N/A | N.A. |  | Optional with capability signalling |

- Reference

RAN1#101e chairman’s note

R1-2005097, “RAN1 UE features list for Rel-16 NR after RAN1#101-e,” Moderators (AT&T, NTT DOCOMO, INC.)

R1-2001484, RAN1 UE feature list for Rel-16 NR after RAN1#101-E