3GPP TSG RAN WG1 #102-e										R1-2006653

e-Meeting, August 17th – 28th, 2020

Agenda Item:	7.2.5.2

Source:		ITRI

Title:			Remaining issue on the HARQ-ACK/PUSCH priority

Document for:		Discussion and Decision

- Introduction

After RAN1#101-e meeting, we have two remaining issues cannot reach consensus:

Issue 1:	PHY priority when both DCI format 0_1/1_1 and DCI format 0_2/1_2 are configured to be monitored per BWP

Issue 2:	Default priority if priority field in DCI is not configured

In this contribution we provide our views on these issues.

- Discussion

  - PHY priority when both DCI format 0_1/1_1 and DCI format 0_2/1_2 are configured to be monitored per BWP

When a UE is not capable of supporting dynamic switching of PUSCH/HARQ-ACK priority via both DCI format 0_1/1_1 and 0_2/1_2, according to the discussion after RAN1#101-e meeting, we have following proposals to define the UE behavior:

- Interpretation 1: A UE not indicating the support of [FG11-4b] which is configured for monitoring of DCI format 0_1 / 1_1 and 0_2/1_2 in a BWP of a serving cell, is not expected to be configured with PriorityIndicator-ForDCIFormat1_1 and PriorityIndicator-ForDCIFormat1_2 for that BWP of a serving cell.

  - Alt. 1: DCI format 1_2 schedules PDSCH with HARQ-Ack of priority index 1. DCI format 1_1 follows current 38.213 specifications.

  - Alt. 1b: Follow the current 38.213 (both are of priority 0, as not dynamically indicated)

- Interpretation 1 / Alt. 2: A UE not indicating the support of [FG11-4b] and being configured for monitoring of DCI format 0_1 / 1_1 and 0_2/1_2 in a BWP is not expected to be configured with PriorityIndicator-ForDCIFormat1_1 for that BWP of a serving cell.

Wherein the [FG11-4b] is defined as following:

| 11-4b | Support of priority indicator field configured in DCI formats 1_1 and 1_2 in a BWP when configured to monitor both DCI formats 1_1 and 1_2 in the BWP |
| --- | --- |

This issue is related to the interpretation of the RAN1#99 agreement:

Agreements:

When both DCI format 0_1/1_1 and DCI format 0_2/1_2 are configured to be monitored per BWP, a DCI format (from the formats 0_1/1_1/0_2/1_2) can be used to schedule PDSCH with different HARQ-ACK priorities or PUSCH with different priorities.

- This feature is UE optional

However, different companies have different understanding on this agreement. According to above agreement and [FG11-4b] defined in UE feature, our understanding is that the UE cannot dynamically determine the priority of PUSCH/HARQ-ACK according to the priority field in DCI. Therefore, it is reasonable that the priority could be determined according to a pre-determined rule since we have agreed that a DCI format can be used to schedule PDSCH with different HARQ-ACK priorities or PUSCH with different priorities. Moreover, since the DCI format 0_2/1_2 is intended for URLLC, it’s reasonable to assume it corresponds to high priority in case UE is not capable of scheduling different priorities by a DCI format while both DCI format 0_2/1_2 and DCI format 0_1/1_1 are configured to be monitored for a BWP. Therefore, we suggest to adopt Alt. 1 of representation 1 to handle the priority issue.

Proposal 1:

- Adopt Alt. 1 of representation 1 to handle the priority when a UE is not indicate the support of [FG11-4b] and both DCI format 0_1/1_1 and DCI format 0_2/1_2 are configured to be monitored per BWP.

  - Default priority if priority field in DCI is not configured

According to this issue, we have following options to define the UE behaviour after RAN1#101-e meeting:

Option 1:

- If DCI format 0_1 does not include a priority indicator field, DCI format 0_1 triggers transmissions of priority 0.

- If DCI format 1_1 does not include a priority indicator field, DCI format 1_1 triggers receptions of priority 0

- If DCI format 0_2 does not include a priority indicator field

  - if the UE is not configured USS for DCI format 0_1, DCI format 0_2 triggers transmissions of priority 0

  - if the UE is configured USS for DCI format 0_1, DCI format 0_2 triggers transmissions of priority 1

- If DCI format 1_2 does not include a priority indicator field

  - if the UE is not configured USS for DCI format 1_1, DCI format 1_2 triggers receptions of priority 0

  - if the UE is configured USS for DCI format 1_1, DCI format 1_2 triggers receptions of priority 1

Option 2:

- If DCI format 0_1/1_1 does not include a priority indicator field, priority of PUSCH/HARQ-ACK associated with DCI format 0_1/1_1 is low

- If DCI format 0_2/1_2 does not include a priority indicator field, priority of PUSCH/HARQ-ACK associated with DCI format 0_2/1_2 is high.

Option 3:

- If priority field in a DCI format is not configured, the corresponding transmissions or receptions are of low priority

When a DCI format can be configured with a priority indicator to differentiate the priority of corresponding HARQ-ACK/PUSCH, it is reasonable that the gNB should configure it if the priority differentiation is needed. Therefore, if the priority indicator is not existed in a DCI format, it implies that the differentiation is not needed. Therefore, the option 3 is much preferable than other options.

Proposal 2:

- Adopt option 3 to handle the default priority if priority field in DCI is not configured.

- Conclusion

Based on our discussions, we have the following proposals:

Proposal 1:

- Adopt Alt. 1 of representation 1 to handle the priority when a UE is not indicate the support of [FG11-4b] and both DCI format 0_1/1_1 and DCI format 0_2/1_2 are configured to be monitored per BWP.

Proposal 2:

- Adopt option 3 to handle the default priority if priority field in DCI is not configured.

- References

RAN WG1#99 chairman’s note.

RAN WG1#100bis-e chairman’s note.

R1-2002784, “Summary of email thread [100b-e-NR-L1enh_URLLC-UCI_Enh-02]”, OPPO.

RAN WG1#101-e chairman’s note.

R1-2005097, “RAN1 UE features list for Rel-16 NR after RAN1#101-e”, AT&T, NTT DOCOMO

R1-2004919, “Summary#2 of email thread [101-e-NR-L1enh_URLLC-UCI_Enh-02]”, OPPO.