3GPP TSG RAN WG1 #102-e	Tdoc R1-2006669

e-Meeting, August 17th – August 28th, 2020

Agenda Item:	8.7.3

Source:	Ericsson

Title:	Evaluation of UE power saving schemes

Document for:	Discussion and Decision

# 1		Introduction

In this paper, performance of UE power saving features from Rel-15/16 is presented.

# 2	Discussion

## 2. 1	Long DRX with different On durations

In current typical setting, only long-DRX (with a notable on-duration length) and a relatively large inactivity timer (IAT) are used in the CDRX configurations. Thus, straightforwardly, UE power consumption can be reduced by reducing the length of on-duration and/or inactivity timer. In Table 1 performance in terms of user perceived throughput (UPT) and delay by reducing the on-duration. In the simulation, FTP3 traffic type is used with a packet size of  0.5 MB and a mean inter-arrival time of 200ms. Reducing the on-duration gives limited power savings and has less UPT impact.

Table 1. UPT loss and power saving by shortening the On-duration

| Methods | IAT | Long-DRX Cycle | Short-DRX Cycle | On-duration | UPT (Mbps) | UPT Loss (%) | Power | Power saving (%) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Long-DRX only | 100 ms | 160 ms |  | 8 ms | 31.94 |  | 46.25 |  |
|  |  |  |  | 4 ms | 31.27 | 2.1% | 44.91 | 2.9% |
|  |  |  |  | 2 ms | 31.01 | 2.9% | 44.30 | 4.2% |

## 2. 2	Long and short-DRX

Short-DRX can be used in conjunction with a shorter IAT. For example, rather than using 100ms of inactivity timer and only long-DRX, a 20ms of inactivity timer can be used with 4 cycles of short DRX (20ms each) and long-DRX. In this simulation, FTP3 traffic type is also used with a packet size of  0.5 MB and a mean inter-arrival time of 200ms. Reducing the IAT along with short-DRX could significantly reduce the power consumption with some UPT loss.

Table 2. UPT loss and power saving using short DRX and smaller IAT.

| Methods | IAT | Long-DRX Cycle | Short-DRX Cycle | On-duration | UPT (Mbps) | UPT Loss (%) | Power | Power saving (%) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Long-DRX with Short-DRX | 20 ms | 160 ms | 20 ms 4 cycles | 8 ms | 31.24 | 2.2% | 39.00 | 15.7% |
|  |  |  |  | 4 ms | 29.75 | 6.9% | 34.80 | 24.8% |
|  |  |  |  | 2 ms | 28.90 | 9.5% | 32.73 | 29.2% |

## 2. 3	WUS with long DRX

For Rel. 16, WUS was introduced for the long-DRX cycle. In below table, the performance of WUS for the case of only long-DRX configuration is shown. FTP3 traffic type is used with a packet size of  0.5 MB and a mean inter-arrival time of 200ms. In addition, the WUS offset is set to 3 slots.

Table 3. UPT loss and power saving obtained from WUS.

| Methods | IAT | Long-DRX Cycle | Short-DRX Cycle | On-duration | UPT (Mbps) | UPT Loss (%) | Power | Power saving (%) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Long-DRX+WUS | 100 ms | 160 ms | #N/A | 8ms | 30.14 | 5.6% | 43.93 | 5.0% |

# Conclusion

In this document, we provide performance evaluation for Rel-16 power savings from the prespective of UE power savings vs system impact and make the following observation.

Observation Rel-15/16 power savings schemes provide a flexible framework for trade-off between UE power savings and system impact by appropriate choice of scheduling parameters as shown in the figure below.

Figure 1. UE power savings vs UPT loss for Rel-15/16 power savings schemes.