3GPP TSG RAN1 Meeting #102-e 	R1-2006676

e-Meeting, August 17th – 28th, 2020

Title:	NR NTN Reference scenarios definition for Rel-17 normative phase

Source: 	Thales

Type:	Discussion

Document for:	Decision

Agenda Item:	8.4

# Abstract

The objective of this paper is to down select the reference scenarios that will be considered for the normative phase on NR supports NTN. Three scenarios are proposed.

# Introduction

NR NTN WI description [1] focus the work for NR solutions to support NTN as follows:

“LEO and GEO with implicit compatibility to support HAPS (high altitude platform station) and ATG (air to ground) scenarios according to the following principles:

- FDD is assumed for core specification work for NR-NTN

  - NOTE: This does not imply that TDD cannot be used for relevant scenario, e.g. HAPS, ATG

- Earth fixed tracking area is assumed with Earth fixed and moving cells

- UEs with GNSS capabilities are assumed

- Transparent payload is assumed”

According to TR 38.821, a Transparent payload refers to payload that changes the frequency carrier of the uplink RF signal, filters and amplifies it before transmitting it on the downlink. In other word, it doesn’t embark any function or network entity defined in the 3GPP architecture.

This TDOC intends to define NTN reference scenarios for Rel.17 normative work and all related assumptions for all the RAN WGs involved in the normative work.

The objective is to support LEO, GEO as well as MEO space segments. LEO features the largest Doppler while GEO features the largest Latency.

They correspond to scenarios A, C1 and C2 in Table 4.2-1: “Reference scenarios” of 3GPP TR 38.821.

# Non-Terrestrial Network scenarios: Overview

3.1 Main assumptions

Based on the reference scenarios identified in TR 38.821, the following scenarios are selected

Six transparent payload based satellite reference scenarios are considered for the Rel-17 work item “NR_NTN_solutions” characterised in the table below:

Table 2-1 Reference satellite scenarios for Rel-17 work item “NR_NTN_solutions”

| Scenarios | C1.1 | C1.2 | C2.1 | C2.2 | A1 | A2 |
| --- | --- | --- | --- | --- | --- | --- |
| Orbit | LEO @ 600 km altitude | LEO @ 600 km altitude | LEO @ 1200 km altitude | LEO @ 1200 km altitude | GEO @ 35,786 km altitude | GEO @ 35,786 km altitude |
| Frequency band | Sub 6GHz | Above 6GHz | Sub 6GHz | Above 6GHz | Sub 6 GHz | Above 6 GHz |
| Beams generation | Earth fixed beams (Note 1) | Earth fixed beams (Note 1) | Earth moving beams | Earth moving beams | Earth fixed beams | Earth fixed beams |

NOTE 1: Each satellite has the capability to steer beams towards fixed points on earth using beam-forming techniques. This is applicable for a period of time corresponding to the visibility time of the satellite

While at low altitude, Earth moving beams are not realistic due to excessive Hand-over rate, it may be envisaged at higher altitude.

3.2 Key parameters

The key reference scenario parameters can be found in table 4.2-2 of this document. It corresponds to the table 4.2-2 of [2] in which the scenarios referring to the regenerative payload option have been removed.

Table 4.2-2: Reference scenario parameters

| Scenarios | GEO based non-terrestrial access network (Scenario A) | LEO based non-terrestrial access network (Scenario C) |
| --- | --- | --- |
| Orbit type | notional station keeping position fixed in terms of elevation/azimuth with respect to a given Earth point | circular orbiting around the Earth |
| Altitude | 35,786 km | 600 km 1,200 km |
| Spectrum (service link) | <6 GHz (e.g. 2 GHz) >6 GHz (e.g. DL 20 GHz, UL 30 GHz) |  |
| Max channel bandwidth capability (service link) | 30 MHz for band < 6 GHz 400 MHz for band > 6 GHz |  |
| Payload | Scenario A: Transparent (including radio frequency function only) | Scenario C: Transparent (including radio frequency function only) |
| Inter-Satellite link | No | No |
| Earth-fixed beams | Yes | Scenario C1: Yes (steerable beams), see note 1 Scenario C2: No (the beams move with the satellite) |
| Max beam foot print size (edge to edge) regardless of the elevation angle | 3500 km (Note 5) | 1000 km |
| Min Elevation angle for both sat-gateway and user equipment | 10° for service link and 10° for feeder link | 10° for service link and 10° for feeder link |
| Max distance between satellite and user equipment at min elevation angle | 40,581 km | 1,932 km (600 km altitude) 3,132 km (1,200 km altitude) |
| Max Round Trip Delay (propagation delay only) | Scenario A: 541.46 ms (service and feeder links) | Scenario C: (transparent payload: service and feeder links) 25.77 ms (600km) 41.77 ms (1200km) |
| Max differential delay within a cell (Note 6) | 10.3 ms | 3.12 ms and 3.18 ms for respectively 600km and 1200km |
| Max Doppler shift (Earth fixed user equipment) | 0.93 ppm | 24 ppm (600km) 21 ppm(1200km) |
| Max Doppler shift variation (earth fixed user equipment) | 0.000 045 ppm/s | 0.27 ppm/s (600km) 0.13 ppm/s(1200km) |
| Maximum Delay variation as seen by the UE (Note 7) | Negligible | Up to +/- 48 µs/sec (600 km) Up to +/- 42 µs/sec (1200 km) |
| Service link | 3GPP defined New Radio |  |
| Feeder link | 3GPP defined Radio interface | 3GPP defined Radio interface |
| NOTE 1: Each satellite has the capability to steer beams towards fixed points on Earth using beamforming techniques. This is applicable for a period of time corresponding to the visibility time of the satellite, from the point of view of the ground cell. NOTE 2: Max delay variation within a beam (Earth fixed user equipment) is calculated based on minimum elevation angle for both gateway and user equipment NOTE 3: Max differential delay within a beam is calculated based on maxium beam foot print diameter at nadir NOTE 4: Speed of light used for delay calculation is 299792458 m/s. NOTE 5: The Maximum beam foot print size for GEO is based on current state of the art GEO High Throughput Systems (HTS), assuming low elevation NOTE 6: The maximum differential delay at cell level has been computed considering the one at beam level for largest beam size. It does not preclude that cell may include more than one beam when beam size are small or medium size. However the cumulated differential delay of all beams within a cell will not exceed the maximum differential delay at cell level in the table above. NOTE 7: The delay variation measures how fast the round trip delay (function of UE-satellite-NTN gateway distance) varies over time when the satellite moves towards/away from the UE. It is expressed in µs/s and is negligible for GEO scenario |  |  |

The NTN study results apply to GEO scenarios as well as all NGSO scenarios with circular orbit at altitude greater than or equal to 600 km.

3.3 User Equipment

The User equipment considered for the key reference scenario parameters can be found in table 4.3-1 of this document.

Table 4.3-1 Reference satellite scenarios: User equipment types

| User equipment characteristics | Handheld | VSAT (Note 1) |
| --- | --- | --- |
| Antenna type | Omnidirectional antenna | Directive antenna |
| Motion on the earth | 500 km/h (e.g. on board a high speed train) | Up to 1200 km/h (e.g. aircraft mounted) |
| antenna types | Omnidirectional antenna | Directional antenna (up to 60 cm equivalent aperture diameter) |
| Antenna polarisation | Linear: +/-45°X-pol | circular |
| Max transmit power | up to 200 mW (power class 3) | up to 20 W |
| Note 1 : VSAT terminal characteristics could be implemented with phased array antenna. It may be mounted on Moving platforms (e.g., aircrafts, vessels) or building |  |  |

As per [1], UEs with GNSS capabilities are assumed and hence have the capability to determine their own position.

However, both UE with and without the capability for pre-compensation of timing and frequency offset shall be supported.

The reasons are that processing capabilities (such as NR UE processing) and/or service availability (such as 5GC available services) may differ or may be decoupled.

UEs with capability on timing and frequency pre-compensation using their GNSS capabilities are assumed. However the support of UEs without capability on timing and frequency pre-compensation is not precluded in the subsequent release.

# Non-Terrestrial Network scenarios: Physical layer assumptions (RAN1)

4.1 Satellite parameters

The following tables are considered as the baseline for system level simulations:

The list of study cases for the system level simulations during the Work item phase are listed in the table 5.1-1 of this document.

Table 5.1-1: List of study cases for SLS in WI phase

| Case | Satellite orbit | Satellite parameter set | Central beam elevation | Terminal | Frequency Band | Frequency/ Polarization Reuse |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | GEO | Set 1 | 45 deg | VSAT | Ka-band | Option 2 |
| 2* | GEO | Set 1 | 45 deg | Handheld | S-band | Option 2 |
| 3 | LEO-600 | Set 1 | 90 deg | VSAT | Ka-band | Option 2 |
| 4 | LEO-600 | Set 1 | 90 deg | Handheld | S-band | Option 2 |
| 5* | LEO-1200 | Set 2 | 90 deg | VSAT | Ka-band | Option 2 |
| 6 | LEO-1200 | Set 2 | 90 deg | Handheld | S-band | Option 2 |
| NOTE 1: no star = 1st priority, * = second priority scenario |  |  |  |  |  |  |

4.2 UE parameters

The parameters of the UE for the system level simulations during the Work item phase are listed in the table 5.2-1 of this document.

Table 5. 2-1: UE parameters for SLS in WI phase

| Characteristics | Handheld | VSAT (Note 1) |
| --- | --- | --- |
| Frequency band | Sub 6 GHz e.g S band (i.e. 2 GHz) | Above 6 GHz e.g. Ka band (i.e. 30 GHz UL and 20 GHz DL) |
| Antenna type and configuration | (1, 1, 2) with omni-directional antenna element | Directional Section 6.4.1 of [2] with 60 cm equivalent aperture diameter |
| Polarisation | Linear: +/-45°X-pol | circular |
| Rx Antenna gain | 0 dBi per element | 39.7 dBi |
| Antenna temperature | 290 K | 150 K |
| Noise figure | 7 dB | 1.2 dB |
| Tx max transmit power | 200 mW (23 dBm) corresponding to UE power class 3 | 2 W (33 dBm) |
| Tx antenna gain | 0 dBi per element | 43.2 dBi |
| Note 1 : VSAT terminal characteristics could be implemented with phased array antenna. It may be mounted on Moving platforms (e.g., aircrafts, vessels) or building |  |  |

4.3 Others

FDD is assumed for the considered scenarios.

FDD is assumed for all the reference scenarios

For HARQ process evaluation altitude of NGSO satellite of up to 1500 km (upper boundary limit of LEO satellite altitude) could be considered.

For HARQ process evaluation altitude of NGSO satellite of up to 1500 km (upper boundary limit of LEO satellite altitude) could be considered.

# Conclusion

Proposal 1: Six transparent payload based satellite reference scenarios are considered for the Rel-17 work item “NR_NTN_solutions” characterised in the table below:

Table 2-1 Reference satellite scenarios for Rel-17 work item “NR_NTN_solutions”

| Scenarios | C1.1 | C1.2 | C2.1 | C2.2 | A1 | A2 |
| --- | --- | --- | --- | --- | --- | --- |
| Orbit | LEO @ 600 km altitude | LEO @ 600 km altitude | LEO @ 1200 km altitude | LEO @ 1200 km altitude | GEO @ 35,786 km altitude | GEO @ 35,786 km altitude |
| Frequency band | Sub 6GHz | Above 6GHz | Sub 6GHz | Above 6GHz | Sub 6 GHz | Above 6 GHz |
| Beams generation | Earth fixed beams (Note 1) | Earth fixed beams (Note 1) | Earth moving beams | Earth moving beams | Earth fixed beams | Earth fixed beams |

NOTE 1: Each satellite has the capability to steer beams towards fixed points on earth using beam-forming techniques. This is applicable for a period of time corresponding to the visibility time of the satellite

Proposal 2: The key reference scenario parameters can be found in table 4.2-2 of this document. It corresponds to the table 4.2-2 of [2] in which the scenarios referring to the regenerative payload option have been removed.

Proposal 3: The User equipment considered for the key reference scenario parameters can be found in table 4.3-1 of this document.

Proposal 4: UEs with capability on timing and frequency pre-compensation using their GNSS capabilities are assumed. However the support of UEs without capability on timing and frequency pre-compensation is not precluded in the subsequent release.

Proposal 5: The list of study cases for the system level simulations during the Work item phase are listed in the table 5.1-1 of this document.

Proposal 6: The parameters of the UE for the system level simulations during the Work item phase are listed in the table 5.2-1 of this document.

Proposal 7: FDD is assumed for all the reference scenarios

Proposal 8: For HARQ process evaluation altitude of NGSO satellite of up to 1500 km (upper boundary limit of LEO satellite altitude) could be considered.

# References

[1]	3GPP RP-201256 Work Item "Solutions for NR to support non-terrestrial networks (NTN)"

[2]	3GPP TR 38.821 “Solutions for NR to support non-terrestrial networks (NTN) (Release 16)”

END