3GPP TSG-RAN WG1 Meeting #102-e	R1-2006689

e-Meeting, August 17th – 28th, 2020

Agenda Item:	7.2.6

Source:	Ericsson

Title:	Remaining issues on multi-DCI based Multi-TRP

Document for:	Discussion

# Introduction

In this paper we address open issues related to multi-DCI based multi-TRP PDSCH transmission.

# Discussion

## PDCCH monitoring priority based on QCL-TypeD for multi-DCI based M-TRP

Offline proposal a-11: For the PDCCH monitoring priority rule based QCL-TypeD in multi-DCI based M-TRP, down-select from:

- Alt1: do not support simultaneous reception of PDCCHs with different QCL-TypeDs and no Spec change

- Alt2: for UE capable of two simultaneous QCL-TypeD, apply the priority rule within CORESETs with same CORESETPoolIndex.

In the FL summary from RAN1#101-e, the issue of PDCCH monitoring priority based on QCL-TypeD for multi-DCI based M-TRP was discussed.  According to the current specification in TS 38.213 (see Section 10.1), simultaneous reception of PDCCHs using different QCL-TypeD is not possible.  The following alternatives were proposed in the FL summary [1]:

Since simultaneous reception of PDSCH will be supported for multi-DCI based multi-TRP as a UE capability, we think it is beneficial if the multi-DCI UEs with this capability can also simultaneously receive PDCCH.  Hence, we make the following proposal:

For the PDCCH monitoring priority rule based QCL-TypeD in multi-DCI based M-TRP, apply the priority rule within CORESETs with same CORESETPoolIndex for a UE capable of two simultaneous QCL-TypeD.

# Conclusion

Based on the discussion in the previous sections we propose the following:

Proposal 1	For the PDCCH monitoring priority rule based QCL-TypeD in multi-DCI based M-TRP, apply the priority rule within CORESETs with same CORESETPoolIndex for a UE capable of two simultaneous QCL-TypeD.

# References

[1]	R1-2004719, FL summary #2 for Multi-TRP/Panel Transmission, RAN1 #101-e, May 25th – June 5th, 2020.