3GPP TSG RAN WG1 Meeting #102-e	R1- 2006770

Aug. 17th – 28th, 2020

Agenda item:	7.2.4.2.2

Source: 	Qualcomm Incorporated

Title: 	Sidelink Resource Allocation Mode 2

Document for:	Discussion/Decision

# Introduction

In this contribution, we discuss open issues for NR V2X specifications regarding Mode 2 resource allocation.

# Configuration of T2 min Parameters by Upper Layer

A candidate single-slot resource for transmission Rx,y is defined as a set of LsubCH contiguous sub-channels with sub-channel x+j in slot tySL where j=0,...,LsubCH-1. The UE shall assume that any set of LsubCH contiguous sub-channels included in the corresponding resource pool within the time interval [n+T1,n+T2] correspond to one candidate single-slot resource, where

-	selection of T1 is up to UE implementation under 0 ≤ T1≤ Tproc,1SL  , where Tproc,1SL  is defined in slots in Table 8.1.4-2 where μSL is the SCS configuration of the SL BWP;

-	if  T2min is shorter than the remaining packet delay budget (in slots) then T2 is up to UE implementation subject to  T2min ≤ T2 ≤ remaining packet budget (in slots); otherwise T2 is set to the remaining packet delay budget (in slots).

The total number of candidate single-slot resources is denoted by Mtotal.

Currently, the resource selection window is defined in Section 8.1.4 of TS 38.214 as

Where  T2min is defined as

t2min_SelectionWindow: internal parameter T2min is set to the corresponding value from higher layer parameter t2min_SelectionWindow for the given value of prioTX.

However, the corresponding RRC parameter is defined in TS 38.331 as

SL-SelectionWindowConfig-r16 ::=       SEQUENCE {

sl-Priority-r16                        INTEGER (1..8),

sl-SelectionWindow-r16                 ENUMERATED {n1, n5, n10, n20}

}

As we can see, there is a mismatch in the parameter naming.

Observation 1: There is a naming mismatch between TS 38.331 and TS 38.214 for the higher layer parameter used to define T2min

Proposal 1: Adopt the following TP to correct the higher layer parameter used to define T2min in TS 36.214.

SL-SelectionWindow: internal parameter T2min is set to the corresponding value from higher layer parameter SL-SelectionWindow for the given value of prioTX.

# Configuration of Reservation Periodicity by Upper Layer

The current RAN1 procedure to set "Resource reservation period" field in SCI1-A is captured as follow in TS 36.213

-	"Resource reservation period" as an index in sl-ResourceReservePeriod-r16 corresponding to a reservation period provided by higher layers [11, TS 38.321], if the UE is provided sl-MultiReserveResource-r16

This is similar to the LTE V2X procedure in TS 36.213

the UE shall set the Resource reservation field according to table 14.2.1-2 based on indicated value X, where X is equal to the Resource reservation interval provided by higher layers divided by 100.

However, in LTE V2X Resource reservation interval value is properly set in TS 36.321 as follow.

-	if SL_RESOURCE_RESELECTION_COUNTER = 1 for the Sidelink process associated with the configured sidelink grant and the MAC entity randomly selected, with equal probability, a value in the interval [0, 1] which is above the probability configured by upper layers in probResourceKeep:

-	set the resource reservation interval for the configured sidelink grant equal to 0;

For NR V2X, currently there is no provision in TS 38.321 to set the value of "Resource reservation period". Furthermore, it was agreed in RAN1-101e meeting that

Agreements:

- A UE is expected to be (pre-)configured with a set sl-ResourceReservePeriod containing value of 0 ms

Agreements:

- A UE sets “Resource reservation period” in SCI 1-A to correspond to value of the period provided by higher layers from (pre-)configured set sl-ResourceReservePeriod

  - RAN1 assumes that at least in cases if higher layer decides not to keep the resource for the transmission in the next period or there is no associated period, then higher layer provides 0 ms periodicity

Send LS to RAN2 to inform this decision.

-	If the UE is provided sl-MultiReserveResource-r16, "Resource reservation period" as an index in sl-ResourceReservePeriod-r16 corresponding to

- 	a reservation period provided by higher layers [11, TS 38.321] if the resource in the next period will be used to transmit the next TB

- 	0 otherwise.

The expected outcome of this agreement is that RAN2 will capture in TS 38.321 procedure to set “Resource reservation period” value for each TB. Furthermore, if the UE decides not to use the selected resource for the next TB due to any reason, the value needs to be set to 0. Another possibility is to capture this procedure directly in TS 38.213, section 16.4 as follow.

Observation 2: The procedure to set reservation period value by higher layer has not been captured

Proposal 2: Adopt the following TP to set "Resource reservation period" field in SCI-1A in TS 36.213, section 16.4.

-	If the UE is provided sl-MultiReserveResource-r16, "Resource reservation period" as an index in sl-ResourceReservePeriod-r16 corresponding to

- 	a reservation period provided by higher layers [11, TS 38.321] if the resource in the next period will be used to transmit the next TB

- 	0 otherwise.

# Resource Selection

The resource selection procedure in TS 38.214 is triggered independently for each transport block without any consideration for whether a slot already contains resource selected for the transmission of another transport. This could cause two transmissions to overlap in time, which is not supported by the UE.

Proposal 3: Adopt the following TP to prevent overlap in time between two selected resources

----------------------------------------------------begin text proposal for 38.214----------------------------------------------------

8.1.4	UE procedure for determining the subset of resources to be reported to higher layers in PSSCH resource selection in sidelink resource allocation mode 2

<<<unchanged text omitted>>>

7)	The UE shall exclude any candidate single-slot resource Rx,y from the set SA if slot tySL contains a resource selected for transmission of another transport block.

8)	If the number of candidate single-slot resources remaining in the set SA is smaller than X⋅Mtotal, then Th(pi) is increased by 3 dB for each priority value Th(pi) and the procedure continues with step 4.

-----------------------------------------------------end text proposal for 38.214-----------------------------------------------------

# Conclusion

Observation 1: There is a naming mismatch between TS 38.331 and TS 38.214 for the higher layer parameter used to define T2min

Observation 2: The procedure to set reservation period value by higher layer has not been captured

Proposal 1: Adopt the following TP to correct the higher layer parameter used to define T2min in TS 36.214.

SL-SelectionWindow: internal parameter T2min is set to the corresponding value from higher layer parameter SL-SelectionWindow for the given value of prioTX.

Proposal 2: Adopt the following TP to set "Resource reservation period" field in SCI-1A in TS 36.213, section 16.4.

-	If the UE is provided sl-MultiReserveResource-r16, "Resource reservation period" as an index in sl-ResourceReservePeriod-r16 corresponding to

- 	a reservation period provided by higher layers [11, TS 38.321] if the resource in the next period will be used to transmit the next TB

- 	0 otherwise.

Proposal 3: Adopt the following TP to prevent overlap in time between two selected resources

----------------------------------------------------begin text proposal for 38.214----------------------------------------------------

8.1.4	UE procedure for determining the subset of resources to be reported to higher layers in PSSCH resource selection in sidelink resource allocation mode 2

<<<unchanged text omitted>>>

7)	The UE shall exclude any candidate single-slot resource Rx,y from the set SA if slot tySL contains a resource selected for transmission of another transport block.

8)	If the number of candidate single-slot resources remaining in the set SA is smaller than X⋅Mtotal, then Th(pi) is increased by 3 dB for each priority value Th(pi) and the procedure continues with step 4.

-----------------------------------------------------end text proposal for 38.214-----------------------------------------------------

# References

- 3GPP TS 38.211 V16.2.0, NR, Physical Channels and Modulation (Release 16).

- 3GPP TS 38.212 V16.2.0, NR, Multiplexing and Channel Coding (Release 16).

- 3GPP TS 38.213 V16.2.0, NR, Physical Layer Procedures for Control (Release 16).

- 3GPP TS 38.214 V16.2.0, NR, Physical Layer Procedures for Data (Release 16).