| 3GPP TSG-RAN WG1 Meeting #102 e-Meeting, August 17th – 28th, 2020 | R1-2006818 |
| --- | --- |

Agenda item:	8.8.1.1

Source: 	Qualcomm Incorporated

Title: 	Baseline FR1 coverage performance

Document for:	Discussion/Decision

# Introduction

In this contribution, we first discuss remaining aspects of link-level simulation assumption for baseline FR1 coverage evaluation. Based on the proposed assumption, we present our results on coverage of 5G NR using link level simulations, followed by a detailed link budget analysis. A thorough link budget analysis is presented for two distinct scenarios -- a dense urban deployment and a rural deployment. The basic parameters assumed in this study are in line with the agreements made in the previous RAN1 meeting.

# Remaining Aspects on Baseline Coverage Assumption

In this section we address some of our assumptions on certain key aspects on coverage analysis that were identified in the previous RAN1 meeting. Three key aspects include (a) assumptions on gNB modelling and antenna gain, (b) link budget template and (c) methodology to identify performance bottlenecks. We address each of the above in the discussion below.

## gNB Modelling and antenna gain assumptions

We made the following two agreements in R1-101e:

____________________________________

Agreement:

Down selection on the following options for antenna array gain for LLS based methodology for FR1 in next meeting.

- Option 1: Antenna array gain is included in the link budget template.

- FFS: array gain = 10 * 1og10 (number of antenna elements/number of TxRUs)

- FFS: For TDL channel model

- FFS: Values reflective of realistic implementation and network operation.

- Option 2: Antenna array gain is included in LLS.

- FFS: For CDL channel model

Agreements:

- For link level simulation, adopt the following table for eMBB data or VoIP on PUSCH and for PUCCH for FR1.

| Parameters | Values |
| --- | --- |
| Number of receive antenna elements for BS | Urban: 192 antenna elements for 4GHz and 2.6GHz, (M,N,P,Mg,Ng) = (12,8,2,1,1) (optional) 128 antenna elements for 4GHz, (M,N,P,Mg,Ng) = (8,8,2,1,1) Rural: 64 antenna elements for 4GHz and 2.6GHz (M,N,P,Mg,Ng) = (8,4,2,1,1) 32 antenna elements for 2GHz (M,N,P,Mg,Ng) = (8,2,2,1,1) 16 antenna elements for 700MHz (M,N,P,Mg,Ng) = (4,2,2,1,1) |
| Number of receive TxRUs for BS | TBD gNB architectures to study for TDL: - 2 or 4 TXRUs for 2GHz, 700 MHz - 64TxRUs for 2.6 and 4 GHz. - Optional: 32 TXRUs at 2 GHz [gNB modeling in LLS for TDL: - Option 1: 2 or 4 gNB receive chains in LLS (as starting point). FFS: correlation - Option 2: Number of gNB receive chains = number of TXRUs in LLS. FFS: correlation.] [gNB architectures to study for CDL: - Urban: 64 receive chains for 2.6 and 4 GHz in LLS - Rural: 8 receive chains for 4GHz and 2.6GHz in LLS - 4 receive chains for 2GHz and 700MHz in LLS.] [gNB modeling in LLS for CDL: Number of gNB receive chains = number of TXRUs in LLS.] |
| Delay spread | Urban: 300ns Rural: 300ns Rural with long distance: 30ns |
| PRBs/TBS/MCS for eMBB for PUSCH | Any value of PRBs, and corresponding MCS index, reported by companies will be considered in the discussion. Companies are encouraged to use 30 PRBs for 1Mbps, 4 PRBs for 100kbps, 1 PRB for 30kbps as a starting point. TBS can be calculated based on e.g. the number of PRBs, target data rate, frame structure and overhead. |
| PRBs/MCS for VoIP for PUSCH | [4 PRBs] for VoIP as starting point. Other values of PRBs can be reported by companies. QPSK, pi/2 BPSK (optional) |

Note: For TDL models, companies report whether antenna array gain, obtained from mapping antenna elements to TXRU, is included in LLS or link budget template. Array gain calculation method and how channel estimation is accounted for is reported by companies

____________________________________

We prefer Option 2 in the first agreement and Option 2 in the second agreement. We believe it is important that the LLS capture accurately the number of gNB TXRUs. Simulating gNBs with only 1, 2 or 4 TXRUs is an over-simplification and may not accurately reflect the performance of a real gNB with a lot more TXRUs. We therefore make the following proposal:

Proposal 1: The number of gNB receive chains is equal to the number of TXRUs in LLS. Antenna array gains due to digital beamforming or combining are included in LLS.

## Link budget template

With regard to which the link budget template should be used for coverage analysis, we made the following agreement in R1-101e:

---------------------------

Agreement:

- Down selection on the following options for the link budget template for FR1 in next meeting.

- Option 1: Adopt single link budget template based on IMT-2020 self-evaluation with necessary revisions, including adding/removing/revising some parameters.

  - FFS: The template provided by FL in Tdoc R1-2005005.

- Option 2: Adopt both templates, i.e. link budget template in IMT-2020 self-evaluation and link budget template in TR 36.824.

- Option 3: Adopt single link budget template in TR 36.824 with necessary revisions, including adding/revising some parameters.

---------------------------

We primarily focus on MCL as a metric of choice to understand the current status of coverage in 5G NR. MCL is a straightforward and clear metric that is not dependent on specific choice of deployment density. It provides a clear view of the PHY channels are likely to be a bottleneck in a real deployment and can be used reliably to draw additional insight on the main areas to focus on for coverage enhancement. In particular, we propose to reuse MCL template (Table 5-1) in TR 36.824 for MCL computation.

Proposal 2: Reuse MCL template (Table 5-1) in TR 36.824 for MCL analysis.

## Methodology to identify performance bottleneck

In R1-101e, we made the following agreement on the overall evaluation methodology to be adopted for this study.

--------

Agreements:

- The basic evaluation methodology is based on link-level simulation for FR1.

- Step 1: Obtain the required SINR for the physical channels under target scenarios and service/reliability requirements.

- Step 2: Obtain the baseline performance based on required SINR and link budget template.

- Note: aspects related to identifying target performance and coverage bottlenecks based on target performance metric is to be handled separately

- The evaluation methodology based on system-level simulation is optional for FR1.

- Note: The simulation assumptions for SLS are up to companies’ reports.

--------

We first note that any coverage analysis has two key components, the first focused on coverage of control channels that are devoid of any minimum service requirements and are likely to determine some of the fundamental aspects of a network such as the requirements for initial access procedure. The second component focuses on the data channels and the discussion here depends on the various levels of service requirement. While a true cell-edge UE may be able to stay in RRC connected mode, the data rates delivered to such a UE may be too low to be useful to support any application. Data rates specified for minimum service requirements act as benchmarks that further qualify the coverage offered in a cell. With these two-pronged approach in mind, we make the following proposal:

Proposal 3: Use relative MCL as a measure of coverage in different scenarios. Aim to address any gap in uplink and downlink control channel coverage. Aim to then close the gap between control and data channel coverage to meet minimum service requirements.

# Parameters for Link Budget Analysis

In R1-101e, we made several agreements on the general parameters and channel-specific parameters for link-level simulations. All results presented here are in line with these agreements. We list some of our assumptions for clarity.

Table 1 Common link-level parameters

| Parameter | Rural | Urban |
| --- | --- | --- |
| Carrier Frequency | 700 MHz | 4GHz |
| Multiplexing | FDD | TDD (DDDSU) |
| System BW | 20MHz | 100MHz |
| gNB Tx power | 49 dBm for 20 MHz bandwidth (ITU document [2] specifies 41 dBm for 10 MHz bandwidth for urban scenarios ) | 51 dBm for 100 MHz |
| UE power class | 23 dBm | 23 dBm |
| Numerology | 15 kHz SCS | 30 kHz SCS |
| BS Noise Figure | 5 dB | 5 dB |
| UE Noise Figure | 7 dB | 7 dB |
| BS antenna gain (dB) | 8 dB | 8 dB |
| UE antenna gain (dB) | 0 dB | 0 dB |
| Thermal noise level | -174 dBm/Hz | -174 dBm/Hz |
| Channel | TDL-C 300ns | TDL-C 300ns |
| UE speed | 3km/h & 120 km/h | 3km/h |
| # of gNB TxRUs | 4 | 64 |
| # of Tx at UE | 1 | 1/2 |
| # of Rx at UE | 2 | 4 |
| # of cells | 1 | 1 |
| TDD pattern | N/A | DDDSU |
| UL data rate target | 100kbps | 1Mbps |
| DL data rate target | 1Mbps | 10Mbps |

Table 2 PUCCH Parameters for link-level simulations and link budget analysis

| PUCCH format | PUCCH format 1 | PUCCH format 3 |
| --- | --- | --- |
| Waveform | Low PAPR sequence | DFT-s-OFDM |
| UCI payload (bit) | 2 bits | 11, 22 bits |
| PUCCH duration (symbols) | 14 | 14 |
| # RBs for long PUCCH | 1 RB | 1 RB |
| Repetition | disabled | disabled |
| Frequency hopping | Enabled | Enabled |
| Performance | 1% false alarm and 1% mis-detection |  |

Table 3 PUSCH (EMBB, VoNR)  Parameters for link-level simulations and link budget analysis

| PUSCH Parameter | Value |
| --- | --- |
| UE antenna configuration | 1 Tx |
| UE power class | Power class 3 |
| Slot structure | 12 data symbols and 2 DMRS symbols |
| Waveform | DFT-S-OFDM |
| PUSCH rank | 1 |
| PUSCH RBs | Variable 1-32 RBs |
| PUSCH DMRS | Type 1 with no data on DMRS symbols |
| HARQ | Enabled |
| Performance metric | 10% iBLER after 1st tx for eMBB and VoNR |

Table 4 VoNR parameters

| TDD Config (DDDSU) |  |
| --- | --- |
| Payload segments per 20 ms | 1 |
| Payload/TB size (no segmentation) | 320 bits (256 bits from voice codec; 3 byte RoHC; 2 byte PDCP hdr; 1 byte RLC hdr ; 2 byte MAC hdr) |
| RB, MCS | 4 RB, MCS4 (64QAM table) |
| HARQ | enabled; max retransmissions = 7 |
| FDD config |  |
| Payload segments per 20ms | 4 |
| Payload/TB size(with 4 segments) | (296)/4 + 24 = 98 bits |
| RB, MCS | 4 RB, MCS 0 (64QAM table) |
| HARQ | enabled; max retransmissions = 4 |

Table 5 Parameters related to RACH procedure

| RACH |  |
| --- | --- |
| Format | Format 0 for rural, B4 for urban |
| Msg 2 PDSCH |  |
| Payload | 9 bytes |
| RB, MCS | MCS 0, TB scale factor 0.25, 12 PRBs |
| HARQ | N/A |
| Slot structure | 9 data symbols, 3 DMRS symbols |
| Precoding | Precoder cycling |
| Performance target | 10% iBLER |
| Msg 3 PUSCH |  |
| Payload | 56 bits |
| RB, MCS | 2 RBs, MCS 0 (64QAM table) |
| Slot structure | 11 data symbols, 3 DMRS symbols |
| HARQ | disabled |
| Performance target | 10% iBLER |
| Msg 4 PDSCH |  |
| Payload | 130 bytes |
| Slot structure | 10 data symbols, 2 DMRS symbols |
| RB, MCS | 36 RB, MCS 0 (64QAM table) |
| HARQ | enabled; max retransmissions = 4 |
| Precoding | Precoder cycling |
| Performance target | 10% rBLER |

Table 6 Broadcast PDCCH Parameters

| Parameter | Value |
| --- | --- |
| BW | 48 RBs |
| # of symbols | 2 |
| PDCCH aggregation level | 16 |
| DCI size | 40 (+ 24 bits CRC) |
| REG bundle size | 6 |
| Beam forming | Precoder cycling |

Table 7 PDSCH Parameters

| Parameter | Value |
| --- | --- |
| Slot structure | 10 PDSCH data symbols, 2 DMRS symbols |
| Precoding | Closed-loop precoding |
| PDSCH rank | 1 |
| Frequency Allocation | Full band allocation |

# Baseline eMBB Coverage

In the following, with a focus on eMBB data traffic we present our results on the link budget analysis driven by the results obtained in our link level simulations.

## Rural Scenario

Result of the link budget analysis for downlink and uplink are presented in Table 8 and Table 9 for downlink and uplink channels in rural scenarios, respectively.

Table 8 Link budget analysis for downlink in rural scenarios

| Rural Pedestrian (700MHz FDD, 20MHz, 4T2R, TDL-C 300ns, 3kmph) |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Channel / Format |  | PBCH | Broadcast PDCCH (40-bit, AL16) | Broadcast PDSCH msg2 (no HARQ) | Broadcast PDSCH msg4 (wHARQ) | Unicast PDSCH 1Mbps (wHARQ) |
| gNB Tx Power (dBm) | A | 49.0 | 49.0 | 49.0 | 49.0 | 49.0 |
| gNB Tx Bandwidth (Hz) | B | 2.00E+07 | 2.00E+07 | 2.00E+07 | 2.00E+07 | 2.00E+07 |
| Occupied channel BW (Hz) | C | 3.60E+06 | 8.64E+06 | 2.16E+06 | 6.48E+06 | 1.91E+07 |
| Occupied channel Tx Power (dBm) Uniform PSD | D=10*LOG10(C/B)+A | 41.6 | 45.4 | 39.3 | 44.1 | 48.8 |
| Thermal noise density (dBm/Hz) | E | -174 | -174 | -174 | -174 | -174 |
| UE Receiver noise figure (dB) | F | 7 | 7 | 7 | 7 | 7 |
| Interference margin (dB) | G | 0 | 0 | 0 | 0 | 0 |
| Effective noise power(dBm) | H=E+10*LOG10(C)+F | -101.44 | -97.63 | -103.66 | -98.88 | -94.19 |
| required SINR (dB) per RxAnt | I | -12.80 | -9 | -10 | -10.5 | -11 |
| Receiver sensitivity (dBm) | J=I+H | -114.24 | -106.63 | -113.66 | -109.38 | -105.19 |
| MCL | MCL=D-J | 155.79 | 151.99 | 152.99 | 153.49 | 153.99 |

| Rural Vehicular (700MHz FDD, 20MHz, 4T2R, TDL-C 300ns, 120kmh) |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Channel / Format |  | PBCH | Broadcast PDCCH (40-bit, AL16) | Broadcast PDSCH msg2 (no HARQ) | Broadcast PDSCH msg4 (wHARQ) | Unicast PDSCH 1Mbps (wHARQ) |
| gNB Tx Power (dBm) | A | 49.0 | 49.0 | 49.0 | 49.0 | 49.0 |
| gNB Tx Bandwidth (Hz) | B | 2.00E+07 | 2.00E+07 | 2.00E+07 | 2.00E+07 | 2.00E+07 |
| Occupied channel BW (Hz) | C | 3.60E+06 | 8.64E+06 | 2.16E+06 | 6.48E+06 | 1.91E+07 |
| Occupied channel Tx Power (dBm) Uniform PSD | D=10*LOG10(C/B)+A | 41.6 | 45.4 | 39.3 | 44.1 | 48.8 |
| Thermal noise density (dBm/Hz) | E | -174 | -174 | -174 | -174 | -174 |
| UE Receiver noise figure (dB) | F | 7 | 7 | 7 | 7 | 7 |
| Interference margin (dB) | G | 0 | 0 | 0 | 0 | 0 |
| Effective noise power(dBm) | H=E+10*LOG10(C)+F | -101.44 | -97.63 | -103.66 | -98.88 | -94.19 |
| required SINR (dB) per RxAnt | I | -12.80 | -9 | -10 | -10.5 | -11 |
| Receiver sensitivity (dBm) | J=I+H | -114.24 | -106.63 | -113.66 | -109.38 | -105.19 |
| MCL | MCL=D-J | 155.79 | 151.99 | 152.99 | 153.49 | 153.99 |

Table 9 Link budget analysis for uplink in rural scenarios

|  |  | Rural pedestrian (1Tx4Rx, TDL-C 300ns, 700MHz FDD, 3kmph) |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Channel / Format |  | PUCCH |  |  | PUSCH (14 OS) |  |  | PRACH |
|  |  | PUCCH PF1 2 bits | PUCCH PF3 11 bits | PUCCH PF3 22 bits | 100kbps Unicast (w HARQ) | 56 bits Msg3 (no HARQ) | VoNR (w HARQ 2% BLER) | Format 0 |
| UE Tx Power (dBm) | A | 23 | 23 | 23 | 23 | 23 | 23 | 23 |
| Thermal noise density (dBm/Hz) | B | -174 | -174 | -174 | -174 | -174 | -174 | -174 |
| gNB Receiver noise figure (dB) | C | 5 | 5 | 5 | 5 | 5 | 5 | 5 |
| Interference margin (dB) | D | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| Occupied channel bandwidth (Hz) | E | 1.80E+05 | 1.80E+05 | 1.80E+05 | 7.20E+05 | 3.60E+05 | 7.20E+05 | 1.05E+06 |
| Effective noise power(dBm) | F=10*LOG10(E) +B+C+D | -116 | -116 | -116 | -110 | -113 | -110 | -109 |
| required SINR (dB) per RxAnt | G | -10.5 | -7.0 | -4.5 | -10.0 | -6.0 | -9.0 | -17.6 |
| Receiver sensitivity (dBm) | H=F+G | -126.95 | -123.45 | -120.95 | -120.43 | -119.44 | -119.43 | -126.39 |
| Max coupling loss (dB) | MCL=A-H | 149.95 | 146.45 | 143.95 | 143.43 | 142.44 | 142.43 | 149.39 |

|  |  | Rural Vehicular (1Tx4Rx, TDL-C 300ns, 700MHz FDD, 120kmph) |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Channel / Format |  | PUCCH |  |  | PUSCH (14 OS) |  |  | PRACH |
|  |  | PUCCH PF1 2 bits | PUCCH PF3 11 bits | PUCCH PF3 22 bits | 100kbps Unicast (w HARQ) | 56 bits Msg3 (no HARQ) | VoNR (w HARQ 2% BLER) | Format 0 |
| UE Tx Power (dBm) | A | 23 | 23 | 23 | 23 | 23 | 23 | 23 |
| Thermal noise density (dBm/Hz) | B | -174 | -174 | -174 | -174 | -174 | -174 | -174 |
| gNB Receiver noise figure (dB) | C | 5 | 5 | 5 | 5 | 5 | 5 | 5 |
| Interference margin (dB) | D | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| Occupied channel bandwidth (Hz) | E | 1.80E+05 | 1.80E+05 | 1.80E+05 | 7.20E+05 | 3.60E+05 | 7.20E+05 | 1.05E+06 |
| Effective noise power(dBm) | F=10*LOG10(E) +B+C+D | -116 | -116 | -116 | -110 | -113 | -110 | -109 |
| required SINR (dB) per RxAnt | G | -10.5 | -7.0 | -4.5 | -10.0 | -6.0 | -9.0 | -17.6 |
| Receiver sensitivity (dBm) | H=F+G | -126.95 | -123.45 | -120.95 | -120.43 | -119.44 | -119.43 | -126.39 |
| Max coupling loss (dB) | MCL=A-H | 149.95 | 146.45 | 143.95 | 143.43 | 142.44 | 142.43 | 149.39 |

Using the above analysis, we summarize the coverage for each channel in the following table.

| Physical Layer Channel | MCL (dB) |
| --- | --- |
| PUCCH (2 bits, PF1) | 150 |
| PUCCH (11 bits, PF3) | 146.5 |
| PUCCH (22 bits, PF3) | 144 |
| PUSCH (VoNR) | 142.5 |
| PUSCH (100 kbps) | 143.5 |
| Msg3 PUSCH (56 bits) | 142.5 |
| PRACH | 149.4 |
| Broadcast Msg2 PDSCH | 153 |
| Broadcast Msg4 PDSCH | 153.5 |
| Broadcast PDCCH (64 bits incl. CRC, AL16) | 152 |
| PDSCH (1 Mbps) | 154 |
| PBCH, 4 SSBs combining | 156 |

Based on the above analysis, we make the following observations and proposals:

Observation 1: Large coverage asymmetry is observed between uplink and downlink control channel coverage in the rural scenario.

Observation 2: PUSCH coverage is also a bottleneck in ensuring eMBB minimum service requirements.

Proposal 4: Consider enhancement to PUCCH performance to reduce the uplink-downlink control channel coverage imbalance in rural scenarios. Also consider enhancements to PUSCH performance to ensure minimum service requirements.

## Urban Scenario

Result of the link budget analysis for downlink and uplink are presented in Table 10 and Table 11 for the downlink and uplink channels in urban scenarios, respectively.

Table 10 Link budget analysis for downlink in urban scenarios

| Urban (100MHz, TDD 4G, DDDSU, 64T4R, TDL-C 300ns, 3kmh) |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Channel / Format |  | PBCH | Broadcast PDCCH (40-bit, AL16) | Broadcast PDSCH msg2 (no HARQ) | Broadcast PDSCH msg4 (wHARQ) | Unicast PDSCH 10Mbps (wHARQ) |
| gNB Tx Power (dBm) | A | 51.0 | 51.0 | 51.0 | 51.0 | 51.0 |
| gNB Tx Bandwidth (Hz) | B | 1.00E+08 | 1.00E+08 | 1.00E+08 | 1.00E+08 | 1.00E+08 |
| Occupied channel BW (Hz) | C | 7.20E+06 | 1.73E+07 | 9.79E+07 | 7.20E+06 | 7.20E+06 |
| Occupied channel Tx Power (dBm) Uniform PSD | D=10*LOG10(C/B)+A | 39.6 | 43.4 | 50.9 | 39.6 | 39.6 |
| Thermal noise density (dBm/Hz) | E | -174 | -174 | -174 | -174 | -174 |
| UE Receiver noise figure (dB) | F | 7 | 7 | 7 | 7 | 7 |
| Interference margin (dB) | G | 0 | 0 | 0 | 0 | 0 |
| Effective noise power(dBm) | H=E+10*LOG10(C)+F | -98.43 | -94.62 | -87.09 | -98.43 | -98.43 |
| required SINR (dB) per RxAnt | I | -15.4 | -11.5 | -13 | -12.00 | -10.00 |
| Receiver sensitivity (dBm) | J=I+H | -113.83 | -106.12 | -100.09 | -110.43 | -108.43 |
| MCL | MCL=D-J | 153.39 | 149.49 | 150.99 | 149.99 | 147.99 |

Table 11 Link budget analysis for uplink in urban scenarios

| Urban Pedestrian (TDD, 4GHz, DDDSU, 1Tx64Rx, TDL-C 300ns, 3kmph) |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Channel / Format |  | PUCCH |  |  | PUSCH (14 OS) |  |  | PRACH |
|  |  | PUCCH PF1 2 bits | PUCCH PF3 11 bits | PUCCH PF3 22 bits | 1Mbps Unicast (wHARQ) | 56 bits Msg3 (no HARQ) | VoNR (wHARQ) | Format 0 |
| UE Tx Power (dBm) | A | 23 | 23 | 23 | 23 | 23 | 23 | 23 |
| Thermal noise density (dBm/Hz) | B | -174 | -174 | -174 | -174 | -174 | -174 | -174 |
| gNB Receiver noise figure (dB) | C | 5 | 5 | 5 | 5 | 5 | 5 | 5 |
| Interference margin (dB) | D | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| Occupied channel bandwidth (Hz) | E | 3.60E+05 | 3.60E+05 | 3.60E+05 | 1.15E+07 | 7.20E+05 | 1.44E+06 | 4.17E+06 |
| Effective noise power(dBm) | F=10*LOG10(E) +B+C+D | -113.4 | -113.4 | -113.4 | -98.4 | -110.4 | -107.4 | -102.8 |
| required SINR (dB) per RxAnt | G | -20.5 | -18.0 | -16.5 | -13.5 | -16.5 | -18.0 | -29.0 |
| Receiver sensitivity (dBm) | H=F+G | -133.94 | -131.44 | -129.94 | -111.89 | -126.93 | -125.42 | -131.80 |
| Max coupling loss (dB) | MCL=A-H | 156.94 | 154.44 | 152.94 | 134.89 | 149.93 | 148.42 | 154.80 |

The above results are summarized in the table below.

| Physical Layer Channel | MCL (dB) |
| --- | --- |
| PUCCH (2 bits, PF1) | 157 |
| PUCCH (11 bits, PF3) | 154.5 |
| PUCCH (22 bits, PF3) | 153 |
| PUSCH (VoNR) | 148.5 |
| PUSCH (1 Mbps) | 135 |
| Msg3 PUSCH (56 bits) | 150 |
| PRACH | 154.8 |
| Broadcast Msg2 PDSCH | 151 |
| Broadcast Msg4 PDSCH | 150 |
| Broadcast PDCCH (64 bits incl. CRC, AL16) | 149.5 |
| PDSCH (10 Mbps) | 148 |
| PBCH, 4 SSBs combining | 153 |

Based on the above analysis, we make the following observations and proposals:

Observation 3: PUSCH coverage to meet minimum service requirements is a significant bottleneck.

Observation 4: Large coverage asymmetry observed between uplink control coverage (PUCCH) and downlink control coverage (broadcast PDCCH). Broadcast PDCCH emerges as a bottleneck in downlink.

Proposal 5: Consider enhancements aimed at improving the coverage of PUSCH and downlink broadcast channels.

Observation 5: VoIP PUSCH is a bottleneck in rural scenarios.

Proposal 6: Consider enhancements to improve coverage of VoIP PUSCH in rural scenarios.

# Conclusion

This contribution presents our views on remaining aspects of simulation assumptions and our baseline coverage analysis for FR1. In particular, the following observations and proposals have been made:

On remaining aspects of simulation assumption

Proposal 1: The number of gNB receive chains is equal to the number of TXRUs in LLS. Antenna array gains due to digital beamforming or combining are included in LLS.

Proposal 2: Reuse MCL template (Table 5-1) in TR 36.824 for MCL analysis.

Proposal 3: Use relative MCL as a measure of coverage in different scenarios. Aim to address any gap in uplink and downlink control channel coverage. Aim to then close the gap between control and data channel coverage to meet minimum service requirements.

On baseline coverage analysis for FR1

Observation 1: Large coverage asymmetry is observed between uplink and downlink control channel coverage in the rural scenario.

Observation 2: PUSCH coverage is also a bottleneck in ensuring eMBB minimum service requirements.

Observation 3: PUSCH coverage to meet minimum service requirements is a significant bottleneck.

Observation 4: Large coverage asymmetry observed between uplink control coverage (PUCCH) and downlink control coverage (broadcast PDCCH). Broadcast PDCCH emerges as a bottleneck in downlink.

Observation 5: VoIP PUSCH is a bottleneck in rural scenarios.

Proposal 4: Consider enhancement to PUCCH performance to reduce the uplink-downlink control channel coverage imbalance in rural scenarios. Also consider enhancements to PUSCH performance to ensure minimum service requirements.

Proposal 5: Consider enhancements aimed at improving the coverage of PUSCH and downlink broadcast channels.

Proposal 6: Consider enhancements to improve coverage of VoIP PUSCH in rural scenarios.