3GPP TSG RAN WG1 Meeting #102-e	R1- 2006828

Aug. 17th – 28th, 2020

Agenda item:	8.11.2.1

Source: 	Qualcomm Incorporated

Title: 	Power Savings for Sidelink

Document for:	Discussion/Decision

# Introduction

The work item for Release-17 NR sidelink enhancement includes multiple objectives [1]:

1. Sidelink evaluation methodology update: Define evaluation assumption and performance metric for power saving by reusing TR 36.843 and/or TR 38.840 (to be completed by RAN#89) [RAN1]

- Note: TR 37.885 is reused for the other evaluation assumption and performance metric. Vehicle dropping model B and antenna option 2 shall be a more realistic baseline for highway and urban grid scenarios.

2. Resource allocation enhancement:

- Specify resource allocation to reduce power consumption of the UEs [RAN1, RAN2]

  - Baseline is to introduce the principle of Rel-14 LTE sidelink random resource selection and partial sensing to Rel-16 NR sidelink resource allocation mode 2.

  - Note: Taking Rel-14 as the baseline does not preclude introducing a new solution to reduce power consumption for the cases where the baseline cannot work properly.

- Study the feasibility and benefit of the enhancement(s) in mode 2 for enhanced reliability and reduced latency in consideration of both PRR and PIR defined in TR37.885 (by RAN#91), and specify the identified solution if deemed feasible and beneficial [RAN1, RAN2]

  - Inter-UE coordination with the following until RAN#90.

    - A set of resources is determined at UE-A. This set is sent to UE-B in mode 2, and UE-B takes this into account in the resource selection for its own transmission.

  - Note: The study scope after RAN#90 is to be decided in RAN#90.

  - Note: The solution should be able to operate in-coverage, partial coverage, and out-of-coverage and to address consecutive packet loss in all coverage scenarios.

  - Note: RAN2 work will start after [RAN#89].

In this coordination, we discuss enhancements to the resource allocation procedure to reduce power consumption of UEs.

# Power Savings in Sensing Procedure

In Rel. 14 LTE V2X, two power saving based resource selection schemes were adopted: (1) resource selection with partial sensing, and (2) random resource selection. Since LTE V2X only supports periodic traffic, these resource selection methods were designed for such traffic patterns only. However, NR SL should allow for resource selection methods that not only provide power saving benefits for the UEs, but also support both periodic and aperiodic traffic.

Observation 1: The partial sensing and random selection procedures in LTE sidelink were defined for periodic reservations; whereas NR sidelink supports both periodic and aperiodic reservations.

Proposal 1: Introduce random selection without sensing for aperiodic and periodic reservations in NR sidelink.

Proposal 2: Reuse the partial sensing and random selection procedure from LTE sidelink in NR sidelink as a starting point for periodic reservations.

For UEs that transmit infrequently, sensing continuously consumes a significant amount of power. As discussed in Section 3, partial sensing from LTE V2X was not designed for aperiodic traffic since such traffic was not a design target of LTE V2X. To enable reduced sensing for aperiodic traffic, the UE could be allowed to perform sensing after a resource selection trigger is received. This enables the UE to only perform sensing when necessary to benefit from the performance improvement brought by sensing while still conserving energy. Figure 1 illustrates this process, which compatible with both periodic and aperiodic traffic, especially for latency tolerant transmissions.

Figure 1: Performing sensing after resource selection trigger

Observation 2: Performing sensing after the resource selection trigger enables the UE to only sense when needed and reduces power consumption.

Proposal 3: Introduce sensing only after resource selection trigger to save power for both aperiodic and periodic reservations.

Proposal 4: Introduce partial sensing for UEs with aperiodic traffic.

# Communication Window Alignment for Partial Sensing

This section focuses on the design of resource selection with partial sensing for NR SL. We first briefly review the LTE partial sensing solution and highlight some of its shortcomings.

Since LTE V2X only supports periodic traffic, a resource selection event for each UE can only be triggered at specific points of time. As an example, for a given UE, a resource selection procedure is triggered every 1ms; the time interval in-between every two consecutive resource selection triggering events is split into sub-intervals with duration of 100ms. A UE is given a 10-bit bitmap, where each bit maps to one sub-interval and indicates whether a UE should perform sensing or not. In the sub-intervals indicated for sensing, a UE can select a number of slots on its own and perform sensing. This is illustrated in Figure 1 below.

Figure 2: An illustration of resource selection with partial sensing for periodic traffic.

The sub-intervals indicated as “no sensing” should be selected carefully; more specifically, the selection of these sub-intervals is dependent on the supported periodicities in the system. As an example, consider a gap of Xms between one of the “no sensing” sub-intervals and the selection window as shown in the figure above. If a system supports data transmission with a periodicity of Xms, no resource within the selection window can be chosen by a UE given that it does not perform sensing in the no-sensing interval. This explains why the LTE partial sensing approach cannot support aperiodic traffic without modifications. To clarify, let us consider the example shown in Figure 1 again; let us also assume that the resource selection trigger is moved to the point shown in Figure 2 to support an aperiodic transmission.

Figure 3: An illustration of LTE’s partial sensing solution for supporting aperiodic transmissions

Since the pattern of the sensing and no-sensing sub-intervals are fixed, if a resource selection trigger moves, a UE may not be able to select any resource in its selection window. For example, in Figure 2, if the system supports traffics with periodicity of 100ms, given that the UE has not performed sensing in the no-sensing sub-interval before its resource selection window, it cannot pick any resources.

Based on the discussions above, the first modification to the LTE’s partial sensing solution for supporting aperiodic transmissions is to remove the bitmap indicating a fixed set of sensing/no-sensing sub-intervals.

Observation 3: For supporting aperiodic data transmission, configuring a UE with a bitmap indicating a fixed set of sensing/no-sensing sub-intervals should be avoided.

By removing the fixed sensing sub-intervals, each partial sensing UE should perform sensing/transmission/reception on a subset of resources. In LTE, since the partial sensing UEs only transmit, and do not receive, their sensing intervals need not be known by other UEs. In NR SL, however, it is desirable to allow the partial sensing UEs to transmit and receive, e.g., a pedestrian UEs may receive warnings from the vehicles or a public safety UEs may need to communicate with each other in both directions. As a result, to communicate with partial sensing UEs, other UEs needs to know their activity intervals, i.e., the duration of the time that they perform sensing/transmission/reception.

Observation 4: Other UEs need to know when a partial sensing UE can receive in order to communicate with it.

Consider an example illustrated in Figure 3 where UE1, UE2, UE3 and UE4 are all partial sensing UEs; UE1 and UE2 are communicating with each other and UE3 and UE4 are communicating with each other.

Figure 4: Two different pairs of partial sensing UEs communicating with each other; the activity durations are aligned within each pair of UEs.

As can be seen in Figure 3, the activity durations are aligned within each pair of UEs; hence, UE1 and UE2 can transmit to/receive from each other. Similarly, UE3 and UE4 can transmit to/receive from each other. However, the activity durations are neither completely aligned nor completely orthogonal across the two different pairs of UEs. As a result, some reservations made by UE1 and UE2 may not be visible by UE3 and UE4; this, in turn, would increase the collision probability.

To address this issue, a set of disjoint resource set can be defined on a resource pool, where each resource set comprises a set of consecutive SL slots and repeats with a given periodicity as illustrated in Figure 4.

Figure 5: An illustration of a set of 3 disjoint resource sets defined on a given resource pool.

A given partial sensing UE is active, i.e., transmit and/or receive, during one or several resource sets. As an example, a group of partial sensing UEs may choose the resource set A or A and B to communicate with each other. To ensure that the reservations made by a set of UEs are visible by other set of UEs, both periodic and aperiodic reservations are limited to the instances of the same resource pool. For example, a UE can reserve resources from one instance of resource set B to the same instance or a different instance of the resource set B. As a result, all the other UEs that might be active on resource set B can potentially receive the reservations information.

Proposal 5: For supporting resource selection with partial sensing, introduce a disjoint set of resources in a given resource pool, where each resource set comprises a set of slots, and repeats with a given periodicity.

Proposal 6: Both periodic and aperiodic reservations are limited to instances of the same resource set.

To facilitate communication among power-sensitive UEs and also between power-sensitive UE and an always-on UE, a power-sensitive UE can inform other UEs of its communication window.

Proposal 7: Support a UE informing other UEs of its reception and transmission availability.

# Conclusion

Observation 1: The partial sensing and random selection procedures in LTE sidelink were defined for periodic reservations; whereas NR sidelink supports both periodic and aperiodic reservations.

Observation 2: Performing sensing after the resource selection trigger enables the UE to only sense when needed and reduces power consumption.

Observation 3: For supporting aperiodic data transmission, configuring a UE with a bitmap indicating a fixed set of sensing/no-sensing sub-intervals should be avoided.

Observation 4: Other UEs need to know when a partial sensing UE can receive in order to communicate with it.

Proposal 1: Introduce random selection without sensing for aperiodic reservations in NR sidelink.

Proposal 2: Reuse the partial sensing and random selection procedure from LTE sidelink in NR sidelink as a starting point for periodic reservations.

Proposal 3: Introduce sensing only after resource selection trigger to save power for both aperiodic and periodic reservations.

Proposal 4: Introduce partial sensing for UEs with aperiodic traffic.

Proposal 5: For supporting a resource selection with partial sensing, introduce a disjoint set of resources on a given resource pool, where each resource set comprises a set of slots, and repeats with a given periodicity.

Proposal 6: Both the periodic and aperiodic reservations are limited to the instances of the same resource set.

Proposal 7: Support a UE informing other UEs of its reception and transmission availability.

# References

- RP-201385, “WID revision: NR sidelink enhancement,” LG Electronics, RAN 88-e.