3GPP TSG RAN WG1 Meeting #102-e		R1-2006841

e-Meeting, August 17th – 29th, 2020

Agenda item:       7.2.6

Source:	Nokia, Nokia Shanghai Bell

Title:	Maintenance of Rel-16 CSI enhancement

Document for:     Discussion and Decision

# 1	Introduction

In this contribution we briefly discuss a correction to TS 38.214 to align the description of Group 1 in the priority reporting levels for Part 2 CSI, in case the FD basis indicator is not reported.

# 2	Correction to Group 1 content when Mν=1

In RAN1#100bis-e, the text proposal in [1] was agreed, which adds the condition Mν>1 for reporting the FD basis indicator, i1,6,l, and its converse for not reporting. Because this indicator is part of Group 1 in the priority reporting levels for Part 2 CSI, the description of Group 1 in Sec. 5.2.3 of TS 38.214 [2] needs to be updated to include the possibility that the FD basis indicator is not reported.

To make the content of Group 1 consistent when i1,6,l is not reported, we suggest the following simple text addition.

TP to TS 38.214, Sec. 5.2.3: ## <Unchanged text is omitted> When CSI reporting on PUSCH comprises two parts, the UE may omit a portion of the Part 2 CSI. Omission of Part 2 CSI is according to the priority order shown in Table 5.2.3-1, where is the number of CSI reports configured to be carried on the PUSCH. Priority 0 is the highest priority and priority is the lowest priority and the CSI report n corresponds to the CSI report with the nth smallest Prii,CSI(y,k,c,s) value among the CSI reports as defined in Clause 5.2.5. The subbands for a given CSI report n indicated by the higher layer parameter csi-ReportingBand are numbered continuously in increasing order with the lowest subband of csi-ReportingBand as subband 0. When omitting Part 2 CSI information for a particular priority level, the UE shall omit all of the information at that priority level. - For Enhanced Type II reports, for a given CSI report n, each reported element of indices i2,4,l i2,5,l and i1,7,l, indexed by l,i and f, is associated with a priority value Pril,i,f=2⋅L⋅υ⋅πf+υ⋅i+l, with π(f)=min⁡(2⋅n3,lf,2⋅N3-n3,l(f)-1) with l=1,2,…,υ, i=0,1,…,2L-1, and f=0,1,…,Mυ-1, and where n3,l(f) is defined in Clause 5.2.2.2.5. The element with the highest priority has the lowest associated value Pril,i,f. Omission of Part 2 CSI is according to the priority order shown in Table 5.2.3-1, where - Group 0 includes indices i1,1, i1,2 and i1,8,l (l=1,…,υ). - Group 1 includes indices i1,5 (if reported), i1,6,l (if reported), the υ2LMυ-KNZ/2 highest priority elements of i1,7,l, i2,3,l, the KNZ/2-υ highest priority elements of i2,4,l and the KNZ/2-υ highest priority elements of i2,5,l (l=1,…,υ). - Group 2 includes the KNZ/2 lowest priority elements of i1,7,l, the KNZ/2 lowest priority elements of i2,4,l and the KNZ/2 lowest priority elements of i2,5,l (l=1,…,υ). ## <Unchanged text is omitted>

# 3	Conclusion

In this contribution, we propose the following modification to TS38.214:

Text Proposal 1.	Add the text “(if reported)” after the FD basis indicator, i1,6,l, in the description of Group 1 in Sec. 5.2.3 to include the case in which this indicator is not reported, for Mν=1.

# References

- R1-2002757, Samsung, “TP for eMIMO MU-CSI topic #2: correction on PMI subband size and basis subset selection,” RAN1#100bis-e, Apr. 2020

- 3GPP TS 38.214 v16.2.0: "NR; Physical layer procedures for data".