# 3GPP TSG RAN WG1 #102-e		                                    R1-2006869

# e-Meeting, August 17th – 28th, 2020

Agenda Item:	7.2.4.4

Source: 	ASUSTeK

Title:  	Discussion on processing time restriction for in-device coexistence

Document for:	Discussion and Decision

- Introduction

RAN1#96bis Working assumption: - For Tx/Tx overlap, - If packet priorities of both LTE and NR sidelink transmissions are known to both RATs prior to time of transmission subject to processing time restriction, then the packet with a higher relative priority is transmitted - In case the priorities of LTE and NR SL transmissions are the same, then it is up to UE implementation as to which transmission is chosen (e.g., taking into account congestion, etc.) - If packet priorities of both LTE and NR sidelink transmissions are not known to both RATs prior to time of transmission subject to processing time restriction, then it is up to UE implementation to manage Tx/Tx overlaps (e.g., LTE transmissions are always prioritized, etc.) - RAN1 does not assume any impact to LTE physical layer specifications RAN1#97 Agreements: - For Tx/Tx overlap, - Confirm the working assumption made in RAN1#96bis - UE capability is defined for short-term time-scale TDM for in-device coexistence RAN1#98 Agreements: - RAN1 understand that NR V2X priority field and PPPP are directly comparable i.e. the same numerical value has the same meaning in both the RATs. Agreements: Unless packet priorities of both LTE and NR sidelink are known to both RATs prior to time of collision (subject to processing time restriction), then - It is up to UE implementation to handle LTE Tx/NR Rx overlap. - It is up to UE implementation to handle NR Tx and LTE Rx overlap.

In this contribution, we discuss processing time restriction for in-device coexistence.

- Discussion

According to previous agreements, priority comparison for handling in-device collision between NR and E-UTRAN sidelink transmission/reception is captured in standard TS 38.213 [1]. One condition for priority comparison is that both priority of E-UTRAN and NR is known to the UE prior to sidelink transmission with a processing time restriction. The intention is to guarantee processing time for the UE. However, this processing time restriction seems not defined in previous agreement. Consequently, current text in TS 38.213 may leave with an unknown processing time T msec without any referred value. In our view, since RAN1 97 meeting confirms former working assumption said there is no LTE spec impact, we think inter-RAT/inter-modulo signaling delay is considered in NR side, and processing time restriction T msec may refer to NR SL process time. For example, one example could be illustrated in Fig. 1. Thus, one possible value of T msec could refer to Tproc,1SL (in unit of slots) in section 8.1.4 in TS 38.214 [2] since it’s similar to guarantee process time for resource selection and generate NR SL transmission. Alternatively, for NR mode-1, another possible value of T msec could refer to Tproc in section 8.6 in TS 38.214 [2] since Tproc in section 8.6 is considered for a minimum process time requirement from sidelink grant (DCI) to NR sidelink transmission. In the last, an simplest way for clarification is align agreement’s wording referred to Text proposal 3.

Fig. 1

Proposal: 	Adopt text proposal 1 or text proposal 2 or text proposal 3 in updating of TS 38.213 section 16.2.4.1

<Text proposal 1>

### 16.2.4 Prioritization of transmissions/receptions #### 16.2.4.1 Simultaneous NR and E-UTRA transmission/reception If a UE - would transmit a first channel/signal using E-UTRA radio access and a second channel/signal using NR radio access, and - a transmission of the first channel/signal would overlap in time with a transmission of the second channel/signal, and - the priorities of the two channels/signals are known to the UE T msec prior to the start of the earlier of the two transmissions, where T msec is obtained from Tproc,1SL slots [6, TS 38.214] the UE transmits only the channel/signal with the higher priority as determined by the SCI formats scheduling the transmissions or, in case of a S-SS/PSBCH block or a sidelink synchronization signal using E-UTRA radio access, as indicated by higher layers or, in case of PSFCH, equal to the priority of the corresponding PSSCH. If a UE - would respectively transmit or receive a first channel/signal using E-UTRA radio access and receive or transmit a second channel/signal using NR radio access, and - a transmission or reception of the first channel/signal would respectively overlap in time with a reception or transmission of the second channel/signal, and - the priorities of the two channels/signals are known to the UE T msec prior to the start of the earlier transmission or reception, where T msec is obtained from Tproc,1SL slots [6, TS 38.214] the UE transmits or receives only the channel/signal with the higher priority as determined by the SCI formats scheduling the transmissions or, in case of a S-SS/PSBCH block or a sidelink synchronization signal using E-UTRA radio access, as indicated by higher layers or, in case of PSFCH, equal to the priority of the corresponding PSSCH.

<Text proposal 2>

### 16.2.4 Prioritization of transmissions/receptions #### 16.2.4.1 Simultaneous NR and E-UTRA transmission/reception If a UE - would transmit a first channel/signal using E-UTRA radio access and a second channel/signal using NR radio access, and - a transmission of the first channel/signal would overlap in time with a transmission of the second channel/signal, and - the priorities of the two channels/signals are known to the UE T msec prior to the start of the earlier of the two transmissions, where T msec is obtained from Tproc [6, TS 38.214] the UE transmits only the channel/signal with the higher priority as determined by the SCI formats scheduling the transmissions or, in case of a S-SS/PSBCH block or a sidelink synchronization signal using E-UTRA radio access, as indicated by higher layers or, in case of PSFCH, equal to the priority of the corresponding PSSCH. If a UE - would respectively transmit or receive a first channel/signal using E-UTRA radio access and receive or transmit a second channel/signal using NR radio access, and - a transmission or reception of the first channel/signal would respectively overlap in time with a reception or transmission of the second channel/signal, and - the priorities of the two channels/signals are known to the UE T msec prior to the start of the earlier transmission or reception, where T msec is obtained from Tproc [6, TS 38.214] the UE transmits or receives only the channel/signal with the higher priority as determined by the SCI formats scheduling the transmissions or, in case of a S-SS/PSBCH block or a sidelink synchronization signal using E-UTRA radio access, as indicated by higher layers or, in case of PSFCH, equal to the priority of the corresponding PSSCH.

<Text proposal 3>

### 16.2.4 Prioritization of transmissions/receptions #### 16.2.4.1 Simultaneous NR and E-UTRA transmission/reception If a UE - would transmit a first channel/signal using E-UTRA radio access and a second channel/signal using NR radio access, and - a transmission of the first channel/signal would overlap in time with a transmission of the second channel/signal, and - the priorities of the two channels/signals are known to the UET prior to the start of the earlier of the two transmissions subject to processing time restriction the UE transmits only the channel/signal with the higher priority as determined by the SCI formats scheduling the transmissions or, in case of a S-SS/PSBCH block or a sidelink synchronization signal using E-UTRA radio access, as indicated by higher layers or, in case of PSFCH, equal to the priority of the corresponding PSSCH. If a UE - would respectively transmit or receive a first channel/signal using E-UTRA radio access and receive or transmit a second channel/signal using NR radio access, and - a transmission or reception of the first channel/signal would respectively overlap in time with a reception or transmission of the second channel/signal, and - the priorities of the two channels/signals are known to the UET prior to the start of the earlier transmission or reception subject to processing time restriction the UE transmits or receives only the channel/signal with the higher priority as determined by the SCI formats scheduling the transmissions or, in case of a S-SS/PSBCH block or a sidelink synchronization signal using E-UTRA radio access, as indicated by higher layers or, in case of PSFCH, equal to the priority of the corresponding PSSCH.

- Conclusion

In this contribution, we propose the following:

Proposal: 	Adopt text proposal 1 or text proposal 2 or text proposal 3 in updating of TS 38.213 section 16.2.4.1

<Text proposal 1>

### 16.2.4 Prioritization of transmissions/receptions #### 16.2.4.1 Simultaneous NR and E-UTRA transmission/reception If a UE - would transmit a first channel/signal using E-UTRA radio access and a second channel/signal using NR radio access, and - a transmission of the first channel/signal would overlap in time with a transmission of the second channel/signal, and - the priorities of the two channels/signals are known to the UE T msec prior to the start of the earlier of the two transmissions, where T msec is obtained from Tproc,1SL slots [6, TS 38.214] the UE transmits only the channel/signal with the higher priority as determined by the SCI formats scheduling the transmissions or, in case of a S-SS/PSBCH block or a sidelink synchronization signal using E-UTRA radio access, as indicated by higher layers or, in case of PSFCH, equal to the priority of the corresponding PSSCH. If a UE - would respectively transmit or receive a first channel/signal using E-UTRA radio access and receive or transmit a second channel/signal using NR radio access, and - a transmission or reception of the first channel/signal would respectively overlap in time with a reception or transmission of the second channel/signal, and - the priorities of the two channels/signals are known to the UE T msec prior to the start of the earlier transmission or reception, where T msec is obtained from Tproc,1SL slots [6, TS 38.214] the UE transmits or receives only the channel/signal with the higher priority as determined by the SCI formats scheduling the transmissions or, in case of a S-SS/PSBCH block or a sidelink synchronization signal using E-UTRA radio access, as indicated by higher layers or, in case of PSFCH, equal to the priority of the corresponding PSSCH.

<Text proposal 2>

### 16.2.4 Prioritization of transmissions/receptions #### 16.2.4.1 Simultaneous NR and E-UTRA transmission/reception If a UE - would transmit a first channel/signal using E-UTRA radio access and a second channel/signal using NR radio access, and - a transmission of the first channel/signal would overlap in time with a transmission of the second channel/signal, and - the priorities of the two channels/signals are known to the UE T msec prior to the start of the earlier of the two transmissions, where T msec is obtained from Tproc [6, TS 38.214] the UE transmits only the channel/signal with the higher priority as determined by the SCI formats scheduling the transmissions or, in case of a S-SS/PSBCH block or a sidelink synchronization signal using E-UTRA radio access, as indicated by higher layers or, in case of PSFCH, equal to the priority of the corresponding PSSCH. If a UE - would respectively transmit or receive a first channel/signal using E-UTRA radio access and receive or transmit a second channel/signal using NR radio access, and - a transmission or reception of the first channel/signal would respectively overlap in time with a reception or transmission of the second channel/signal, and - the priorities of the two channels/signals are known to the UE T msec prior to the start of the earlier transmission or reception, where T msec is obtained from Tproc [6, TS 38.214] the UE transmits or receives only the channel/signal with the higher priority as determined by the SCI formats scheduling the transmissions or, in case of a S-SS/PSBCH block or a sidelink synchronization signal using E-UTRA radio access, as indicated by higher layers or, in case of PSFCH, equal to the priority of the corresponding PSSCH.

<Text proposal 3>

### 16.2.4 Prioritization of transmissions/receptions #### 16.2.4.1 Simultaneous NR and E-UTRA transmission/reception If a UE - would transmit a first channel/signal using E-UTRA radio access and a second channel/signal using NR radio access, and - a transmission of the first channel/signal would overlap in time with a transmission of the second channel/signal, and - the priorities of the two channels/signals are known to the UET prior to the start of the earlier of the two transmissions subject to processing time restriction the UE transmits only the channel/signal with the higher priority as determined by the SCI formats scheduling the transmissions or, in case of a S-SS/PSBCH block or a sidelink synchronization signal using E-UTRA radio access, as indicated by higher layers or, in case of PSFCH, equal to the priority of the corresponding PSSCH. If a UE - would respectively transmit or receive a first channel/signal using E-UTRA radio access and receive or transmit a second channel/signal using NR radio access, and - a transmission or reception of the first channel/signal would respectively overlap in time with a reception or transmission of the second channel/signal, and - the priorities of the two channels/signals are known to the UET prior to the start of the earlier transmission or reception subject to processing time restriction the UE transmits or receives only the channel/signal with the higher priority as determined by the SCI formats scheduling the transmissions or, in case of a S-SS/PSBCH block or a sidelink synchronization signal using E-UTRA radio access, as indicated by higher layers or, in case of PSFCH, equal to the priority of the corresponding PSSCH.

- Reference

- T3 38.213, v16.2.0

- T3 38.214, v16.2.0