3GPP TSG RAN WG1 #102-e	R1-2006909

e-Meeting, August 17th – August 29th, 2020

Agenda item:		8.2.3

Source:	Nokia, Nokia Shanghai Bell

Title:		Simulation Results for NR from 52.6 GHz to 71 GHz

Document for:		Discussion and Decision

# Introduction

The study item on supporting NR from 52.6 GHz to 71 GHz [1] developed a set of simulation criteria [2] in RAN1#101-e.   This contribution provides simulation results consistent with that criteria.  Specifically, the results are provided for the following deployment scenarios:

- Scenario Indoor-A) InH open office model

These results show the performance of spatial selectivity is sufficient for coexistence having a reasonable degradation in mean and cell edge UE throughput for the two-operator scenario as compared to the one operator scenario

# Simulation Methodology

This contribution provides simulation results for the indoor office environment as depicted in Figure 1 with simulation parameters as captured in Table 1.   The simulation assumes a full Rel 16 NR compliant interface as would typically be configured for FR2.  No additional co-existence mechanisms were implemented.

Both a single operator and dual operator scenario were run.  The arrival rates were swept for the single operator to deterimine a bursty traffic arrival rate associated with a 60% resource utilization considered a full load.   A half load point was assumed to be half the arrival rate of the full load giving a 30% resource utilization.

Figure 1 Scenario Indoor-A) InH open office model

Table 1 Simulation Parameters

|  | Base Station - BS | User Equipment - UE |
| --- | --- | --- |
| Carrier Frequency [GHz] | 60 GHz |  |
| Subcarrier Spacing [kHz] | 960 kHz |  |
| Bandwidth [MHz] | 2000 MHz |  |
| Number of RB | 160 |  |
| Deployment Scenario | Scenario indoor-A |  |
| UE distribution | 100% indoor |  |
| Channel Model | - gNB-to-gNB and gNB-to-UE links: InH – office channel & PL model from TR38.901 - UE-to-UE links: [InH – office channel & PL model from TR38.901] |  |
| Mobility | 3 km/hr |  |
| Antenna Configuration | (Mg,Ng,M,N,P) = (1,1,4,8,2) with (0.5 dv, 0.5 dH) | (Mg,Ng,M,N,P) = (1,2,2,2,2) with (0.5 dv, 0.5 dH) |
| Antenna Pattern | Table A.2.1-7 of TR38.802 for ceiling mount | Table A.2.1-8 of TR38.802 |
| Antenna Element Gain | 5 dBi | 5 dBi |
| Power Limitation | 40 dBm EIRP | 25 dBm EIRP |
| Noise figure | 7 dB | 10 dB |
| Transmission Rank | Rank adaptative transmission between Rank 1 and 2 |  |
| TDD DL/UL Ratio | Dynamic TDD |  |
| CSI Feedback | Ideal feedback |  |
| Traffic Model | FTP Model 1 (27 Mbyte file) |  |
| UE Receiver | MMSE-IRC |  |
| Cell selection criteria | Random select from strongest RSRP with 1 dB HO Margin |  |
| DL/UL Traffic Ratio | 50% DL, 50% UL |  |

# Simulation Results & Discussion

The downlink and uplink performance for the single operator and two operator scenarios for both the full half load are provided Figure 2 and Figure 3.  The cell throughput provides the total data rate delivered by the cell.  In full load cases, we see 15% reduction in cell throughput for both uplink and downlink going from 1 operator to 2 operator scenarios.  For the half load cases, we see even smaller drop in cell throughput dropping by 5% or less going from 1 operator to 2 operator scenarios.  This is as expected given the cell throughput in bursty traffic is largely governed by the offered load when the cell is not overloaded.  This indicates that the 2 operator scenario has good performance serving nearly the same cell throughput even though it must co-exist.

Observation 1: The 2 operator co-existence has good performance and is stable with just beamforming and without LBT

The UE statistics are also provided Figure 2 and Figure 3 showing mean and 5 percentile (i.e. cell edge or CE) of the UE perceived throughput (i.e. packet call throughput).  In this case, we see there is a higher degradation in UE mean and CE throughput for the full load case than for the half load cases.    Moreover, the mean and CE UE throughput is greater for the uplink than downlink.  The coexistence penalty is tabulated for the full and half load cases in Table 2 and Table 3, respectively, showing the percent reduction for the 2-operator case compared to the 1 operator case.

Observation 2: Cell edge performance degradation for the 2 operator co-existence is approximately 50% and the degradation is consistently greater in the uplink than the downlink

The coexistence performance with beamforming alone is surprising good when one considers that the area offered load is double in the 2 operator scenario.  In other words, the 2 operator scenario is serving twice the number of UE as the 1 operator case.  Therefore, one could argue that is more appropriate to compare the 1-operator full load case to the 2-operator half load case as they would have the same number of UEs per unit area.  In that case, all the UE throughput numbers are improved with the 2-operator scenario with the one exception of UL cell edge which remains the same.

Observation 3:  For a constant UE load per unit area, the performance of 2 operator scenario performs better than the 1 operator scenario with the exception of UL cell edge which remains the same

Figure 2 Downlink Performance Results

Figure 3 Uplink Performance Results

Table 2 Full Load Co-existence Penalty

|  | Sector | Mean | CE |
| --- | --- | --- | --- |
| DL | 16% | 27% | 50% |
| UL | 15% | 39% | 60% |

Table 3 Half Load Co-existence Penalty

|  | Sector | Mean | CE |
| --- | --- | --- | --- |
| DL | 5% | 19% | 33% |
| UL | 3% | 25% | 56% |

# Conclusion

This contribution provides simulation results for the indoor office environment consistent with the criteria agreed in RAN1@101-e.  The simulations compared the performance of 1 operator and 2 operator co-existence using beamforming alone with the following observations:

Observation 1: The 2 operator co-existence has good performance and is stable with just beamforming and without LBT

Observation 2: Cell edge performance degradation for the 2 operator co-existence is approximately 50% and the degradation is constistently greater in the uplink than the downlink

Observation 3:  For a constant UE load per unit area, the performance of 2 operator scenario performs better than the 1 operator scenario with the exception of UL cell edge which remains the same

# References

- RP-193259, “New SID: Study on supporting NR from 52.6GHz to 71 GHz”, Intel Corporation

- R1-2005193, “Summary #3 of email discussions for [101-e-Post-NR-52_71_GHz]”, Moderator (Intel Corporation)