3GPP TSG RAN WG1 Meeting #102-e 	R1-2006923

# E-meeting, August 17-28, 2020

| CR-Form-v11.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| [DRAFT] CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.213 | CR | <CR#> | rev | - | Current version: | 14.15.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction on PDSCH starting position for BL/CE UEs and starting symbol for MPDCCH of an MBSFN subframe |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R1 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_feMTC-Core |  |  |  |  |  |  | Date: |  | 2020-08-07 |
| Category: | F |  |  |  |  |  |  | Release: |  | Rel-14 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Both the starting OFDM symbol index lDataStart for PDSCH and the starting OFDM symbol index lPMCHStart for MPDCCH are larger than zero. However, if the subframe is an MBSFN subframe with zero-size non-MBSFN region, then there is no symbol for PDSCH and MPDCCH transmission. |  |  |  |  |  |  |  |  |
| Summary of change: |  | A BL/CE UE is not required to monitor PDSCH nor MPDCCH in an MBSFN subframe with zero-size non-MBSFN region. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The BL/CE UE is required to monitor MBSFN subframes which cannot contain MPDCCH nor PDSCH. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.1.6.4A, 9.1.5.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

### < Unchanged parts are omitted >

#### 7.1.6.4A	PDSCH starting position for BL/CE UEs

A BL/CE UE is not required to monitor PDSCH in an MBSFN subframe with zero-size non-MBSFN region.

The starting OFDM symbol for PDSCH is given by index  in the first slot in a subframe  and is determined as follows

-	for reception of SIB1-BR

-	if for the cell on which PDSCH is received

-	if for the cell on which PDSCH is received

-	else

-	is given by the higher layer parameter startSymbolBR

-	if subframe  is a special subframe or configured as an MBSFN subframe, and if the BL/CE UE is configured in CEModeA

-

-	else

-	.

### < Unchanged parts are omitted >

#### 9.1.5.1	MPDCCH starting position

A BL/CE UE is not required to monitor MPDCCH in an MBSFN subframe with zero-size non-MBSFN region.

The starting OFDM symbol for MPDCCH given by index  in the first slot in a subframe  and is determined as follows

-	is given by the higher layer parameter startSymbolBR

-	if subframe  is a special subframe or configured as an MBSFN subframe, and if the BL/CE UE is configured in CEModeA

-

-	else

-	.

### < Unchanged parts are omitted >