3GPP TSG RAN WG1 Meeting #102   	  		                                               R1- 2006970

e-meeting, 2020, August 17th – 28th, 2020

Source:	CATT

Title:	Discussion of evaluation of NR positioning performance

Agenda item:	8.5.2

Document for:	Discussion and Decision

Introduction

At RAN1#101e meeting, following agreements on NR positioning enhancements were approved [1].

Agreement (Proposal 4.1-1, Revision #2, in Section 4.1 of R1-2004868):

- Adopt the parameters defined in Table below as the baseline parameters for all scenarios in the evaluation of the positioning performance in Rel-17.

- Note: Individual companies may consider additional parameter values or different parameter settings in their simulation investigation

- Note: Optional scenarios and assumptions will be discussed separately and can be included

Table: Common scenario parameters applicable for all scenarios

|  | FR1 Specific Values | FR2 Specific Values |
| --- | --- | --- |
| Carrier frequency, GHz | 3.5GHz | 28GHz |
| Bandwidth, MHz | 100MHz | 400MHz |
| Subcarrier spacing, kHz | 30kHz for 100MHz | 120kHz |
| gNB model parameters |  |  |
| gNB noise figure, dB | 5dB | 7dB |
| UE model parameters |  |  |
| UE noise figure, dB | 9dB – Note 1 | 13dB – Note 1 |
| UE max. TX power, dBm | 23dBm – Note 1 | 23dBm – Note 1 EIRP should not exceed 43 dBm. |
| UE antenna configuration | Panel model 1 – Note 1 Mg = 1, Ng = 1, P = 2, dH = 0.5λ, (M, N, P, Mg, Ng) = (1, 2, 2, 1, 1) | Baseline: Multi-panel Configuration 1 and Panel Configuration a – Note 1 - Multi-panel Configuration 1: (Mg, Ng) = (1, 2); Θmg,ng=90°; Ω0,1=Ω0,0+180°; (dg,H, dg,V)=(0,0) - Panel Configuration a: - Each antenna array has shape dH=dV=0.5λ - Config a: (M, N, P) = (2, 4, 2), - the polarization angles are 0° and 90° - The antenna elements of the same polarization of the same panel is virtualized into one TXRU Optional: FFS |
| UE antenna radiation pattern | Omni, 0dBi | Antenna model according to Table 6.1.1-2 in TR 38.855 |
| PHY/link level abstraction | Explicit simulation of all links, individual parameters estimation is applied. Companies to provide description of applied algorithms for estimation of signal location parameters. |  |
| Network synchronization | The network synchronization error, per UE dropping, is defined as a truncated Gaussian distribution of (T1 ns) rms values between an eNB and a timing reference source which is assumed to have perfect timing, subject to a largest timing difference of T2 ns, where T2 = 2*T1 – That is, the range of timing errors is [-T2, T2] – T1: 0ns (perfectly synchronized), 50ns (Optional) |  |
| Note 1: According to 3GPP TR 38.802 Note 2: According to 3GPP TR 38.901 |  |  |

Agreement:

Optional: The following UE antenna configuration can be considered

- 4 UE panels:

  - The antenna elements of the same polarization of the same panel is virtualized into one TXRU

- FFS: Other details

Agreement:

Absolute-time-of arrival model defined in TR 38.901 without modification is considered in the evaluation of all scenarios.

Agreement:

Blockage model is not considered in the evaluation of all scenarios

Agreement: (Proposal 5.1-4, Revision 3, in Section 5.1 of R1-2004961)

- Adopt the parameters defined in the Table below as the baseline parameters for all InF scenarios in the evaluation of positioning performance in Rel-17.

- Note: Individual companies may consider additional parameter values or different parameter settings in their simulation investigation

Table: Parameters common to InF scenario(s)

|  |  | FR1 Specific Values |  | FR2 Specific Values |
| --- | --- | --- | --- | --- |
| Channel model |  | InF-SH, InF-DH |  | InF-SH, InF-DH |
| Layout | Hall size | InF-SH: (baseline) 300x150 m (optional) 120x60 m InF-DH: (baseline) 120x60 m (optional) 300x150 m |  |  |
|  | BS locations | 18 BSs on a square lattice with spacing D, located D/2 from the walls. - for the small hall (L=120m x W=60m): D=20m - for the big hall (L=300m x W=150m): D=50m |  |  |
|  | Room height | 10m |  |  |
| Total gNB TX power, dBm |  | 24dBm | 24dBm EIRP should not exceed 58 dBm |  |
| gNB antenna configuration |  | (M, N, P, Mg, Ng) = (4, 4, 2, 1, 1), dH=dV=0.5λ – Note 1 | (M, N, P, Mg, Ng) = (4, 8, 2, 1, 1), dH=dV=0.5λ – Note 1 One TXRU per polarization per panel is assumed |  |
| gNB antenna radiation pattern |  | Single sector – Note 1 | 3-sector antenna configuration – Note 1 |  |
| Peneteration loss |  | 0dB |  |  |
| Number of floors |  | 1 |  |  |
| UE horizontal drop procedure |  | Uniformly distributed over the horizontal evaluation area for obtaining the CDF values for positioning accuracy, The evaluation area should be at least the convex hull of the horizontal BS deployment. It can also be the whole hall area if the CDF values for positioning accuracy is obtained from whole hall area. |  |  |
| UE antenna height |  | Baseline: 1.5m (Optional): FFS |  |  |
| UE mobility |  | 3km/h (Optional): FFS |  |  |
| Min gNB-UE distance (2D), m |  | 0m |  |  |
| gNB antenna height |  | Baseline: 8m (Optional): FFS |  |  |
| Clutter parameters: {density , height ,size } |  | Low clutter density: {20%, 2m, 10m} High clutter density: See Proposal 5.1-7 |  |  |
| Note 1: According to Table A.2.1-7 in 3GPP TR 38.802 |  |  |  |  |

Agreement:

Optional: For evaluating vertical positioning performance, UE antenna height can be uniformly distributed within [0.5, X2]m, where X2 = 2m for InF-SH and X2= for InF-DH defined in TR 38.901.

Agreement:

Clutter parameters {density , height ,size } for high clutter density are set as follows:

- (Baseline): {40%, 2m, 2m} for fixed UE antenna height and gNB antenna height

- (Optional): {40%, 3m, 5m}

- (Optional): {60%, 6m, 2m}

Agreement:

It will be left to companies to define the configurations for DL PRS and UL SRS for the evaluation of positioning performance.

- Note: Configurations of DL PRS and UL SRS supported by Rel-16 specifications are used for evaluation of the achievable performance based on Rel-16 positioning technologies.

Agreement:

CDFs of positioning errors are used as performance metrics in NR positioning evaluation with at least the following percentiles 50%, 67%, 80%, 90%.

- Note: In addition to overall positioning accuracy performance, companies are encouraged to report the estimation accuracy of UE/gNB measurements (e.g., RSTD) for performance comparison.

Agreement:

CDF values for positioning accuracy for IIoT scenarios are derived based on:

- Case 1 (Required): The UEs inside the convex hull of the horizontal BS deployment area.

- Case 2 (Optional): All the UEs

Agreement:

Optional: For evaluating vertical positioning performance, gNB antenna height can also be set to two fixed heights, which is either {4, 8} m, or {max(4,), 8}.

This contribution discusses the evaluation of NR positioning performance with initial simulation results in InF scenarios.

Initial simulation evaluation of NR positioning performance in InF scenarios

This section discusses initial simulation evaluation of NR positioning performance in InF scenarios. Simulation assumptions are based on the agreements on baseline InF scenarios in RAN1#101-e with the additional parameters given in Table 1.

Table 1: Simulation parameters common to InF scenario(s)

| Parameter | DL PRS | UL SRS-Pos |
| --- | --- | --- |
| Channel model (baseline, otherwise state any modifications) | InF-SH, InF-DH | InF-SH, InF-DH |
| Reference Signal Physical Structure and Resource Allocation (RE pattern) | DL PRS, Comb-size = 6 | UL SRS-Pos, Comb-size = 8 |
| Reference signal (type of sequence, number of ports, …) | Gold sequence, Port-1 | ZC sequence, Port-1 |
| Number of sites | 6 | 6 |
| Number of symbols used per slot per positioning estimate | 1 for FR1; 12 for FR2 | 1 for FR1; 12 for FR2 |
| Number of slots per positioning estimate | 1 | 1 |
| Power-boosting level | 10*log10(6) | 10*log10(8) |
| Uplink power control (applied/not applied) | —— | No |
| interference modelling (ideal muting, or other) | Ideal muting | Ideal muting |
| Description of Measurement Algorithm (e.g. super resolution, interference cancellation, ….) | MUSIC algorithm | MUSIC algorithm |
| Description of positioning technique / applied positioning algorithm (e.g. Least square, taylor series, etc) | Chan algorithm [3] | Chan algorithm [3] |
| Network synchronization assumptions | Perfect | Perfect |
| Beam-related assumption (beam sweeping / alignment assumptions at the tx and rx sides) | No | No |
| Precoding assumptions (codebook, nrof antenna elements used, etc) | No | No |
| Additional notes, if any | No | No |

Four positioning methods (DL-TDOA, UL-TDOA, UL-TDOA + UL-AOA, and Multi-RTT) are evaluated. The simulation cases are shown from Table 2 to Table 5 with the simulation results presented from section 2.1 to section 2.4, respectively.

Table 2: Simulation cases for DL-TDOA

| CASE No. | Scenarios | gNB & UE antenna height and clutter parameter | FR1 /FR2 |
| --- | --- | --- | --- |
| CASE 1.1 | InF-SH-2D: (baseline) 300x150 m | Horizontal: gNB antenna height (8m), UE antenna height (1.5m) , low clutter{20%, 2m, 10m} | FR1 |
| CASE 1.2 | InF-SH-2D: (baseline) 300x150 m | Horizontal: gNB antenna height (8m), UE antenna height (1.5m), low clutter{20%, 2m, 10m} | FR2 |
| CASE 1.3 | InF-SH-3D: (baseline) 300x150 m | Horizontal and vertical: gNB antenna height (4m+8m), UE antenna height uniformly distributed [0.5m, 2m],low clutter{20%, 2m, 10m} | FR1 |
| CASE 1.4 | InF-SH-3D: (baseline) 300x150 m | Horizontal and vertical gNB antenna height (4m+8m), UE antenna height uniformly distributed [0.5m, 2m],low clutter{20%, 2m, 10m} | FR2 |
| CASE 1.5 | InF-DH-2D: (baseline) 120x60 m | Horizontal: gNB antenna height (8m), UE antenna height (1.5m), high clutter#1{40%, 2m, 2m} | FR1 |
| CASE 1.6 | InF-DH-2D: (baseline) 120x60 m | Horizontal: gNB antenna height (8m), UE antenna height (1.5m), high clutter#1{40%, 2m, 2m} | FR2 |
| CASE 1.7 | InF-DH-3D: (baseline) 120x60 m | Horizontal and vertical: gNB antenna height (4m+8m), UE antenna height uniformly distributed [0.5m, 3m], high clutter#2{40%, 3m, 5m} | FR1 |
| CASE 1.8 | InF-DH-3D: (baseline) 120x60 m | Horizontal and vertical: gNB antenna height (4m+8m), UE antenna height uniformly distributed [0.5m, 3m], high clutter#2{40%, 3m, 5m} | FR2 |

Table 3: Simulation cases for UL-TDOA

| CASE No. | Scenarios | gNB & UE antenna height and clutter parameter | FR1 /FR2 |
| --- | --- | --- | --- |
| CASE 2.1 | InF-SH-2D: (baseline) 300x150 m | Horizontal: gNB antenna height (8m), UE antenna height (1.5m), low clutter{20%, 2m, 10m} | FR1 |
| CASE 2.2 | InF-SH-2D: (baseline) 300x150 m | Horizontal: gNB antenna height (8m), UE antenna height (1.5m), low clutter{20%, 2m, 10m} | FR2 |
| CASE 2.3 | InF-SH-3D: (baseline) 300x150 m | Horizontal and vertical: gNB antenna height (4m+8m), UE antenna height uniformly distributed [0.5m, 2m], low clutter{20%, 2m, 10m} | FR1 |
| CASE 2.4 | InF-SH-3D: (baseline) 300x150 m | Horizontal and vertical gNB antenna height (4m+8m), UE antenna height uniformly distributed [0.5m, 2m], low clutter{20%, 2m, 10m} | FR2 |
| CASE 2.5 | InF-DH-2D: (baseline) 120x60 m | Horizontal: gNB antenna height (8m), UE antenna height (1.5m), high clutter#1{40%, 2m, 2m} | FR1 |
| CASE 2.6 | InF-DH-2D: (baseline) 120x60 m | Horizontal: gNB antenna height (8m), UE antenna height (1.5m), high clutter#1{40%, 2m, 2m} | FR2 |
| CASE 2.7 | InF-DH-3D: (baseline) 120x60 m | Horizontal and vertical: gNB antenna height (4m+8m), UE antenna height uniformly distributed [0.5m, 3m], high clutter#2{40%, 3m, 5m} | FR1 |
| CASE 2.8 | InF-DH-3D: (baseline) 120x60 m | Horizontal and vertical: gNB antenna height (4m+8m), UE antenna height uniformly distributed [0.5m, 3m], high clutter#2{40%, 3m, 5m} | FR2 |

Table 4: Simulation cases for UL-TDOA + UL-AOA

| CASE No. | Scenarios | gNB & UE antenna height and clutter parameter | FR1 /FR2 |
| --- | --- | --- | --- |
| CASE 3.1 | InF-SH-3D: (baseline) 300x150 m | Horizontal and vertical: gNB antenna height (4m+8m), UE antenna height uniformly distributed [0.5m, 2m], low clutter{20%, 2m, 10m} | FR1 |
| CASE 3.2 | InF-DH-3D: (baseline) 120x60 m | Horizontal and vertical: gNB antenna height (4m+8m), UE antenna height uniformly distributed [0.5m, 3m], high clutter#2{40%, 3m, 5m} | FR1 |

Table 5: Simulation cases for Multi-RTT

| CASE No. | Scenarios | gNB & UE antenna height and clutter parameter | FR1 /FR2 |
| --- | --- | --- | --- |
| CASE 4.1 | InF-SH-2D: (baseline) 300x150 m | Horizontal: gNB antenna height (8m), UE antenna height (1.5m), low clutter{20%, 2m, 10m} | FR1 |
| CASE 4.2 | InF-SH-3D: (baseline) 300x150 m | Horizontal and vertical: gNB antenna height (4m+8m), UE antenna height uniformly distributed [0.5m, 2m], low clutter{20%, 2m, 10m} | FR1 |
| CASE 4.3 | InF-DH-2D: (baseline) 120x60 m | Horizontal: gNB antenna height (8m), UE antenna height (1.5m), high clutter#1{40%, 2m, 2m} | FR1 |
| CASE 4.4 | InF-DH-3D: (baseline) 120x60 m | Horizontal and vertical: gNB antenna height (4m+8m), UE antenna height uniformly distributed [0.5m, 3m], high clutter#2{40%, 3m, 5m} | FR1 |

DL-TDOA

Positioning performance of UEs within Convex Hull

The horizontal and vertical accuracy for DL-TDOA positioning method for the UEs located within convex hull are given in Table 6 and Table 7, respectively. The CDF curves of positioning errors are given in Section 5.1.1.

Observation 1: For DL-TDOA positioning method from CASE 1.1 to CASE 1.8, the range of horizontal accuracy is from 0.0372 meter to 1.5033 meter at 90% CDF point, while the range of vertical accuracy is from 0.6283 meter to 3.0578 meter at 90% CDF point.

Table 6: Horizontal accuracy obtained from UEs within convex hull (DL-TDOA)

| CASE No. | FR1/FR2 | 50% | 67% | 80% | 90% |
| --- | --- | --- | --- | --- | --- |
| CASE 1.1 | FR1 | 0.0386 | 0.0503 | 0.0662 | 0.1127 |
| CASE 1.2 | FR2 | 0.0136 | 0.0223 | 0.028 | 0.0372 |
| CASE 1.3 | FR1 | 0.079 | 0.1083 | 0.1379 | 0.2045 |
| CASE 1.4 | FR2 | 0.025 | 0.0324 | 0.046 | 0.0789 |
| CASE 1.5 | FR1 | 0.0602 | 0.0891 | 0.1114 | 0.1378 |
| CASE 1.6 | FR2 | 0.0178 | 0.023 | 0.0277 | 0.0388 |
| CASE 1.7 | FR1 | 0.1201 | 0.2499 | 0.4116 | 0.7089 |
| CASE 1.8 | FR2 | 0.0314 | 0.089 | 0.2996 | 1.5033 |

Table 7: Vertical accuracy obtained from UEs within convex hull (DL-TDOA)

| CASE No. | FR1/FR2 | 50% | 67% | 80% | 90% |
| --- | --- | --- | --- | --- | --- |
| CASE 1.3 | FR1 | 0.608 | 0.9636 | 1.3383 | 1.8954 |
| CASE 1.4 | FR2 | 0.1581 | 0.277 | 0.3961 | 0.6283 |
| CASE 1.7 | FR1 | 0.461 | 0.8138 | 1.3474 | 2.9917 |
| CASE 1.8 | FR2 | 0.1337 | 0.2703 | 0.6779 | 3.0578 |

Positioning performance of all UEs

For comparison, the horizontal and vertical accuracy for DL-TDOA positioning method for all UEs (within and out of the convex hull) are given in Table 8 and Table 9, respectively. The CDF curves of positioning errors are given in Section 5.1.2.

Observation 2: For DL-TDOA positioning method from CASE1.3 and CASE 1.7, the horizontal accuracy obtained from the UEs within Convex Hull performs better than that of all UEs (0.2045 meter vs 0.4159 meter at CDF 90% point), while the vertical accuracy obtained from the UEs within Convex Hull and that of all UEs are nearly the same.

Table 8: Horizontal accuracy obtained from all UEs (DL-TDOA)

| CASE No. | FR1/FR2 | UE Type | 50% | 67% | 80% | 90% |
| --- | --- | --- | --- | --- | --- | --- |
| CASE 1.3 | FR1 | Convex Hull UE | 0.079 | 0.1083 | 0.1379 | 0.2045 |
| CASE 1.3 | FR1 | All UE | 0.0996 | 0.14 | 0.2196 | 0.4159 |
| CASE 1.7 | FR1 | Convex Hull UE | 0.1201 | 0.2499 | 0.4116 | 0.7089 |
| CASE 1.7 | FR1 | All UE | 0.2007 | 0.4052 | 0.6782 | 2.1255 |

Table 9: Vertical accuracy obtained from all UEs (DL-TDOA)

| CASE No. | FR1/FR2 | UE Type | 50% | 67% | 80% | 90% |
| --- | --- | --- | --- | --- | --- | --- |
| CASE 1.3 | FR1 | Convex Hull UE | 0.608 | 0.9636 | 1.3383 | 1.8954 |
| CASE 1.3 | FR1 | All UE | 0.6392 | 0.9905 | 1.3333 | 1.8943 |
| CASE 1.7 | FR1 | Convex Hull UE | 0.461 | 0.8138 | 1.3474 | 2.9917 |
| CASE 1.7 | FR1 | All UE | 0.5661 | 0.9039 | 1.5702 | 3.4177 |

UL-TDOA

Positioning performance of UEs within Convex Hull

The horizontal and vertical accuracy for UL-TDOA positioning method for the UEs located within convex hull are given in  Table 10 and Table 11, respectively. The CDF curves of positioning errors are given in Section 5.1.3.

Observation 3: For UL-TDOA positioning method from CASE 2.1 to CASE 2.8, the range of horizontal accuracy is from 0.0538 meter to 1.9448 meter at 90% CDF point, while the range of vertical accuracy is from 0.8304 meters to 3.1267 meter at 90% CDF point.

Table 10: Horizontal accuracy obtained from UEs within convex hull (UL-TDOA)

| CASE No. | FR1/FR2 | 50% | 67% | 80% | 90% |
| --- | --- | --- | --- | --- | --- |
| CASE 2.1 | FR1 | 0.0525 | 0.0712 | 0.0922 | 0.1176 |
| CASE 2.2 | FR2 | 0.022 | 0.0296 | 0.041 | 0.0538 |
| CASE 2.3 | FR1 | 0.0754 | 0.1143 | 0.1728 | 0.2574 |
| CASE 2.4 | FR2 | 0.0223 | 0.0276 | 0.0383 | 0.0817 |
| CASE 2.5 | FR1 | 0.0723 | 0.098 | 0.1284 | 0.1893 |
| CASE 2.6 | FR2 | 0.0204 | 0.0296 | 0.0416 | 0.0553 |
| CASE 2.7 | FR1 | 0.1075 | 0.0155 | 0.3692 | 0.9437 |
| CASE 2.8 | FR2 | 0.0282 | 0.0561 | 0.6127 | 1.9448 |

Table 11: Vertical accuracy obtained from UEs within convex hull (UL-TDOA)

| CASE No. | FR1/FR2 | 50% | 67% | 80% | 90% |
| --- | --- | --- | --- | --- | --- |
| CASE 2.3 | FR1 | 0.608 | 1.036 | 1.3383 | 2.0599 |
| CASE 2.4 | FR2 | 0.2018 | 0.3084 | 0.4611 | 0.8304 |
| CASE 2.7 | FR1 | 0.4167 | 0.6408 | 1.0091 | 2.049 |
| CASE 2.8 | FR2 | 0.116 | 0.2358 | 0.5051 | 3.1267 |

Positioning performance of all UEs

For comparison, the horizontal and vertical accuracy for UL-TDOA positioning method for all UEs (within and out of the convex hull) are given in Table 12 and Table 13, respectively. The CDF curves of positioning errors are given in Section 5.1.4.

Observation 4: For UL-TDOA positioning method from CASE 2.3 and CASE 2.7, the horizontal accuracy obtained from the UEs within Convex Hull performs better than that of all UEs (0.2574 meter vs 0.5174 meter at CDF 90% point), while the vertical accuracy obtained from the UEs within Convex Hull and that of all UEs are nearly the same.

Table 12: Horizontal obtained from all UEs (UL-TDOA)

| CASE No. | FR1/FR2 | UE Type | 50% | 67% | 80% | 90% |
| --- | --- | --- | --- | --- | --- | --- |
| CASE 2.3 | FR1 | Convex Hull UE | 0.0754 | 0.1143 | 0.1728 | 0.2574 |
| CASE 2.3 | FR1 | All UE | 0.1083 | 0.1717 | 0.2574 | 0.5174 |
| CASE 2.7 | FR1 | Convex Hull UE | 0.1075 | 0.155 | 0.3692 | 0.8386 |
| CASE 2.7 | FR1 | All UE | 0.4672 | 0.7594 | 1.261 | 2.9364 |

Table 13: Vertical accuracy obtained from all UEs (UL-TDOA)

| CASE No. | FR1/FR2 | UE Type | 50% | 67% | 80% | 90% |
| --- | --- | --- | --- | --- | --- | --- |
| CASE 2.3 | FR1 | Convex Hull UE | 0.608 | 1.036 | 1.3383 | 2.0599 |
| CASE 2.3 | FR1 | All UE | 0.6392 | 1.0359 | 1.3411 | 2.1666 |
| CASE 2.7 | FR1 | Convex Hull UE | 0.4167 | 0.6408 | 1.0091 | 2.049 |
| CASE 2.7 | FR1 | All UE | 0.4672 | 0.7594 | 1.261 | 2.9364 |

UL-TDOA+UL-AOA

Positioning performance of UEs within Convex Hull

The horizontal and vertical accuracy for UL-TDOA+UL-AOA positioning method are given in Table 14 and Table 15 respectively. The CDF curves of positioning errors are given in Section 5.1.5.

Observation 5: For UL-TDOA+UL-AOA positioning method from CASE 3.1 and CASE 3.2, the range of horizontal accuracy is from 0.151 meter to 0.2677 meter at 90% CDF point, while the range of vertical accuracy is from 0.633 meter to 2.256 meter at 90% CDF point.

Table 14: Horizontal accuracy (UL-TDOA+UL-AOA)

| CASE No. | FR1/FR2 | 50% | 67% | 80% | 90% |
| --- | --- | --- | --- | --- | --- |
| CASE 3.1 | FR1 | 0.033 | 0.054 | 0.1428 | 0.2677 |
| CASE 3.2 | FR1 | 0.044 | 0.062 | 0.118 | 0.151 |

Table 15: Vertical accuracy (UL-TDOA+UL-AOA)

| CASE No. | FR1/FR2 | 50% | 67% | 80% | 90% |
| --- | --- | --- | --- | --- | --- |
| CASE 3.1 | FR1 | 0.8572 | 1.154 | 1.516 | 2.256 |
| CASE 3.2 | FR1 | 0.334 | 0.486 | 0.593 | 0.633 |

Multi-RTT

Positioning performance of UEs within Convex Hull

The horizontal and vertical accuracy for Multi-RTT positioning method are given in Table 16 and Table 17 respectively. The CDF curves of positioning errors are given in Section 5.1.6.

Observation 6: For Multi-RTT positioning method from CASE 4.1 to CASE 4.4, the range of horizontal accuracy is from 0.069 meter to 0.562 meter at 90% CDF point, while the range of vertical accuracy is from 2.18 meter to 2.82 meter at 90% CDF point.

Table 16: Horizontal accuracy (Multi-RTT)

| CASE No. | FR1/FR2 | 50% | 67% | 80% | 90% |
| --- | --- | --- | --- | --- | --- |
| CASE 4.1 | FR1 | 0.031 | 0.040 | 0.057 | 0.090 |
| CASE 4.2 | FR1 | 0.08 | 0.108 | 0.153 | 0.254 |
| CASE 4.3 | FR1 | 0.03 | 0.041 | 0.056 | 0.069 |
| CASE 4.4 | FR1 | 0.029 | 0.079 | 0.431 | 0.562 |

Table 17: Vertical accuracy (Multi-RTT)

| CASE No. | FR1/FR2 | 50% | 67% | 80% | 90% |
| --- | --- | --- | --- | --- | --- |
| CASE 4.2 | FR1 | 0.639 | 0.917 | 1.60 | 2.18 |
| CASE 4.4 | FR1 | 0.507 | 0.723 | 1.59 | 2.82 |

Conclusions

In this contribution, we discussed the evaluation of NR positioning performance. Horizontal and vertical accuracy for four positioning methods are summarized in Table 18 and Table 19, respectively. The following observations are given.

Observation 1: For DL-TDOA positioning method from CASE 1.1 to CASE 1.8, the range of horizontal accuracy is from 0.0372 meter to 1.5033 meter at 90% CDF point, while the range of vertical accuracy is from 0.6283 meter to 3.0578 meter at 90% CDF point.

Observation 2: For DL-TDOA positioning method from CASE1.3 and CASE 1.7, the horizontal accuracy obtained from the UEs within Convex Hull performs better than that of all UEs (0.2045 meter vs 0.4159 meter at CDF 90% point), while the vertical accuracy obtained from the UEs within Convex Hull and that of all UEs are nearly the same. Observation 3: For UL-TDOA positioning method from CASE 2.1 to CASE 2.8, the range of horizontal accuracy is from 0.0538 meter to 1.9448 meter at 90% CDF point, while the range of vertical accuracy is from 0.8304 meters to 3.1267 meter at 90% CDF point.

Observation 4: For UL-TDOA positioning method from CASE 2.3 and CASE 2.7, the horizontal accuracy obtained from the UEs within Convex Hull performs better than that of all UEs (0.2574 meter vs 0.5174 meter at CDF 90% point), while the vertical accuracy obtained from the UEs within Convex Hull and that of all UEs are nearly the same.

Observation 5: For UL-TDOA+UL-AOA positioning method from CASE 3.2 to CASE 3.4, the range of horizontal accuracy is from 0.151 meter to 0.2677 meter at 90% CDF point, while the range of vertical accuracy is from 0.633 meter to 2.256 meter at 90% CDF point.

Observation 6: For Multi-RTT positioning method from CASE 4.1 to CASE 4.4, the range of horizontal accuracy is from 0.069 meter to 0.562 meter at 90% CDF point, while the range of vertical accuracy is from 2.18 meter to 2.82 meter at 90% CDF point.

Table 18: Horizontal accuracy for four positioning method

| CASE No. | FR1/FR2 | 50% | 67% | 80% | 90% |
| --- | --- | --- | --- | --- | --- |
| CASE 1.1 | FR1 | 0.0386 | 0.0503 | 0.0662 | 0.1127 |
| CASE 1.2 | FR2 | 0.0136 | 0.0223 | 0.028 | 0.0372 |
| CASE 1.3 | FR1 | 0.079 | 0.1083 | 0.1379 | 0.2045 |
| CASE 1.4 | FR2 | 0.025 | 0.0324 | 0.046 | 0.0789 |
| CASE 1.5 | FR1 | 0.0602 | 0.0891 | 0.1114 | 0.1378 |
| CASE 1.6 | FR2 | 0.0178 | 0.023 | 0.0277 | 0.0388 |
| CASE 1.7 | FR1 | 0.1201 | 0.2499 | 0.4116 | 0.7089 |
| CASE 1.8 | FR2 | 0.0314 | 0.089 | 0.2996 | 1.5033 |
| CASE 2.1 | FR1 | 0.0525 | 0.0712 | 0.0922 | 0.1176 |
| CASE 2.2 | FR2 | 0.022 | 0.0296 | 0.041 | 0.0538 |
| CASE 2.3 | FR1 | 0.0754 | 0.1143 | 0.1728 | 0.2574 |
| CASE 2.4 | FR2 | 0.0223 | 0.0276 | 0.0383 | 0.0817 |
| CASE 2.5 | FR1 | 0.0723 | 0.098 | 0.1284 | 0.1893 |
| CASE 2.6 | FR2 | 0.0204 | 0.0296 | 0.0416 | 0.0553 |
| CASE 2.7 | FR1 | 0.1075 | 0.0155 | 0.3692 | 0.9437 |
| CASE 2.8 | FR2 | 0.0282 | 0.0561 | 0.6127 | 1.9448 |
| CASE 3.1 | FR1 | 0.033 | 0.054 | 0.1428 | 0.2677 |
| CASE 3.2 | FR1 | 0.044 | 0.062 | 0.118 | 0.151 |
| CASE 4.1 | FR1 | 0.031 | 0.040 | 0.057 | 0.090 |
| CASE 4.2 | FR1 | 0.08 | 0.108 | 0.153 | 0.254 |
| CASE 4.3 | FR1 | 0.03 | 0.041 | 0.056 | 0.069 |
| CASE 4.4 | FR1 | 0.029 | 0.079 | 0.431 | 0.562 |

Table 19: Vertical accuracy for four positioning method

| CASE No. | FR1/FR2 | 50% | 67% | 80% | 90% |
| --- | --- | --- | --- | --- | --- |
| CASE 1.3 | FR1 | 0.608 | 0.9636 | 1.3383 | 1.8954 |
| CASE 1.4 | FR2 | 0.1581 | 0.277 | 0.3961 | 0.6283 |
| CASE 1.7 | FR1 | 0.461 | 0.8138 | 1.3474 | 2.9917 |
| CASE 1.8 | FR2 | 0.1337 | 0.2703 | 0.6779 | 3.0578 |
| CASE 2.3 | FR1 | 0.608 | 1.036 | 1.3383 | 2.0599 |
| CASE 2.4 | FR2 | 0.2018 | 0.3084 | 0.4611 | 0.8304 |
| CASE 2.7 | FR1 | 0.4167 | 0.6408 | 1.0091 | 2.049 |
| CASE 2.8 | FR2 | 0.116 | 0.2358 | 0.5051 | 3.1267 |
| CASE 3.1 | FR1 | 0.8572 | 1.154 | 1.516 | 2.256 |
| CASE 3.2 | FR1 | 0.334 | 0.486 | 0.593 | 0.633 |
| CASE 4.2 | FR1 | 0.639 | 0.917 | 1.60 | 2.18 |
| CASE 4.4 | FR1 | 0.507 | 0.723 | 1.59 | 2.82 |

References

- RAN1 Chairman's Notes 3GPP TSG RAN WG1 Meeting #101-e, May 25th – June 5th, 2020.

- 3GPP TR 38.901, “Study on channel model for frequencies from 0.5 to 100 GHz”, V16.1.0 (2019-12)

- Chan Y T, Ho K C. A simple and efficient estimator for hyperbolic location[J]. IEEE Transactions on Signal Processing, 2002, 42(8):1905-1915.

Appendix

CDF curves of positioning errors

    - DL-TDOA

- : CASE 1.1 Vs  CASE 1.2 (InF-SH-2D)

- : CASE 1.3 Vs CASE 1.4 (InF-SH-3D-Horizontal)

- : CASE 1.3 Vs CASE 1.4 (InF-SH-3D-Vertical)

- : CASE 1.5 Vs CASE 1.6 (InF-DH-2D)

- : CASE 1.7 Vs CASE 1.8 (InF-DH-3D-Horizontal)

- : CASE 1.7 Vs CASE 1.8 (InF-DH-3D-Vertical)

    - DL-TDOA (Impact of Convex Hull UE and all UE)

- : CASE 1.3 (InF-SH-3D-FR1-Horizontal)

- : CASE 1.3 (InF-SH-3D-FR1-Vertical)

- : CASE 1.7 (InF-DH-3D-FR1-Horizontal)

- :  CASE 1.7 (InF-DH-3D-FR1-Vertical)

    - UL-TDOA

- : CASE 2.1 Vs  CASE2.2 (InF-SH-2D)

- : CASE 2.3 Vs  CASE2.4 (InF-SH-3D-Horizontal)

- : CASE 2.3 Vs  CASE2.4 (InF-SH-3D-Vertical)

- : CASE 2.5 Vs  CASE2.6 (InF-DH-2D)

- : CASE 2.7 Vs CASE2.8 (InF-DH-3D-Horizontal)

- : CASE 2.7 Vs CASE2.8 (InF-DH-3D-Vertical)

    - UL-TDOA (Impact of Convex Hull UE and all UE)

- : CASE2.3 (InF-SH-3D-FR1- Horizontal)

- : CASE2.3 (InF-SH-3D-FR1-Vertical)

- : CASE2.7 (InF-DH-3D-FR1-Horizontal)

- : CASE2.7 (InF-DH-3D-FR1-Vertical)

    - UL-TDOA+UL-AOA

Figure-21: CASE 3.1 (InF-SH-3D-FR1-Horizontal)

Figure-22: CASE 3.1 (InF-SH-3D-FR1-Vertical)

Figure-23: CASE 3.2 (InF-DH-3D-FR1-Horizontal)

Figure-24: CASE 3.2 (InF-DH-3D-FR1-Vertical)

    - Multi-RTT

Figure-25: CASE4.1 (InF-SH-2D-FR1)

Figure-26: CASE 4.2 (InF-SH-3D-FR1-Horizontal)

Figure-27: CASE 4.2 (InF-SH-3D-FR1-Vertical)

Figure-28: CASE4.3 (InF-DH-2D-FR1)

Figure-29: CASE 4.4 (InF-SH-3D-FR1-Horizontal)

Figure-30: CASE 4.4 (InF-SH-3D-FR1-Vertical)