3GPP TSG RAN WG1#102-e			R1-2007011

e-Meeting, 17th August – 28th August, 2020

Agenda item:	8.1.5

Source: 	Samsung

Title: 	Simulation results for multi-beam enhancements

Document for:	Discussion and Decision

# Introduction

This contribution provides simulation results for the beam-based inter-cell mobility and high speed train (HST) as proposed in the companion contribution [1].

# Simulation results for beam-based inter-cell mobility

A system-level evaluation with the agreed full buffer traffic model is carried out for Dense Urban (Macro only) scenario. SU-MIMO transmission is considered in the simulation. The relevant evaluation assumptions (EVMs) and parameters are listed in Table 1 in the Appendix, which closely resembles the EVM agreed offline and captured in [2]. To model UE mobility, UE locations on a linear trajectory are sampled an illustration of which is shown in Fig. 1. Note that since the simulation time is fixed, the number of samples modelled increases with UE speed. So, in this setup, the actual number of samples modelled for UEs at 30 km/h is less that for UEs at 120 km/h.

Fig. 1. An illustration of sampling on linear trajectory

The UE throughput performance comparing Rel.15/16-based mobility and proposed beam-based inter-cell mobility are provided in Fig. 2 (average and 5% UE throughput) and Fig. 3 (CDF of UE throughout) for two mobility scenarios: 30 and 120 km/h. In Fig. 2, the results are shown with Rel.15/16-based mobility with 30 km/h as reference. We can observe the following.

Observation 1: For the dense urban scenario,

- the performance of Rel. 15/16-based mobility degrades severely with high mobility, i.e., ~26% and ~41% loss in avg. and 5% UE throughput,

- the performance of proposed beam-based mobility is significantly better than Rel. 15/16-based mobility

  - 27% and 50% additional gain in avg. UE throughput at 30 and 120 km/h, respectively,

- the performance of proposed beam-based mobility remains the same on average for two UE speeds (30 and 120 km/h), and

- based on the CDF of UE throughput, the performance at 30 km/h is better in high percentile regime and that at 120 km/h is better in low percentile regime.

Fig. 2. Full buffer: avg. and 5% UE throughput.

Fig. 3. Full buffer: CDF of UE throughput

Simulation Results for High Speed Train (HST)

A system-level evaluation with the agreed full buffer traffic model is carried out for HST scenario. The relevant evaluation assumption (EVMs) and parameters are listed in Table 2 in the Appendix, which closely resembles the EVM agreed offline and captured in [2]. The simulation assumptions remain the same as in Table 2 in the Appendix except the following:

- The height of the base station and the UE are assumed to be same (2.5 meters).

- Each RRH has 16 beams in the azimuth direction, with a bearing of +/- 20o.

- The UE is assumed to be omnidirectional.

- The UE moves from RRH2 to RRH5, at a speed of 256 km/h, the trajectory is sampled every 1 millisecond (7.11 cm). Total number of simulated points is 8438 points (8.438 seconds).

- The simulation considers the impact of beam latency on the throughput. Three cases have been simulated, with 0 beam latency delay, with 25 millisecond beam latency delay and with 50 millisecond beam latency delay.

An illustration of the system layout and trajectory is shown in Fig.5. Fig. 6 illustrates the relative throughput along the track with different beam switching latencies. To get the average throughput, the throughput is averaged across N points (N milliseconds), and then divided by the maximum average throughput along the track. As expected, the throughput decreases as the UE moves further away from the RRH, then a sudden jump occurs as the UE starts communicating with the next RRH. It should be noted from the figures, that with a longer beam switching latency, the transition to the next RRH is delayed by an amount equivalent to the beam switching latency. Furthermore, as the UE switches to the new RRH and it is close to the new RRH, the rate of beam change is the fastest, increasing the beam switching latency results in a UE using a less optimal beam until the UE moves further away from the RRH, at which time the rate of beam change decreases. This is apparent from the fact that the sudden jump in relative throughput at the RRH is less abrupt in case of a longer beam switching latency. To better see the impact of beam switching latency on throughput, Fig. 7 is an illustration of the CDF distribution of the relative throughput. Increasing the beam switching latency moves the CDF further to the left.

Fig 5: An illustration of the HST scenario layout and UE trajectory.

Fig 6: Relative throughput along the track with different beam switching latencies.

Fig 7: CDF distribution of relative throughput.

Observation 2: For the high speed train scenario, the impact of beam switching latency is more profound as the UE moves past the RRH, where the rate of change of beams is fastest. Based on the CDF of the relative UE throughput, increasing the beam switching latency reduces throughput.

Conclusion

In this contribution, the following observations are made:

Observation 1: For the dense urban scenario,

- the performance of Rel. 15/16-based mobility degrades severely with high mobility, i.e., ~26% and ~41% loss in avg. and 5% UE throughput,

- the performance of proposed beam-based mobility is significantly better than Rel. 15/16-based mobility

  - 27% and 50% additional gain in avg. UE throughput at 30 and 120 km/h, respectively,

- the performance of proposed beam-based mobility remains the same on average for two UE speeds (30 and 120 km/h), and

- based on the CDF of UE throughput, the performance at 30 km/h is better in low percentile regime and that at 120 km/h is better in high percentile regime.

Observation 2: For the high speed train scenario, the impact of beam latency is more profound as the UE moves past the RRH, where the rate of change of beams is fastest. Based on the CDF of the relative UE throughput, increasing the beam switching latency reduces throughput.

# References

- R1-2006128, “Multi-Beam Enhancements,” Samsung.

- R1-2006126, “Moderator summary for Rel.17 FeMIMO EVM offline,” Moderator (Samsung).

Appendix

Table 1: Simulation assumptions for inter-cell mobility

| Parameters | Values |
| --- | --- |
| Scenarios | Dense urban (macro-layer only, TR 38.913) @FR2, 200m ISD, 2-tier model with wrap-around (7 sites, 3 sectors/cells per cell), 100% outdoor, number of dropped UEs per sector = 1 |
| Frequency Range | FR2 @ 30 GHz, SCS: 120 kHz, BW: 80 MHz |
| Mode | DL SU-MIMO |
| Channel Model | Following related assumption in TR 38.802/38.901 |
| TXRU mapping to antenna elements | 2D DFT based beam per polarization |
| TXRU mapping weights | gNB: 16 2D DFT beams (4 in azimuth and 4 in zenith) UE: 8 DFT DFT beams (4 in azimuth and 2 in zenith) |
| Criteria for selection for serving TRP | (Similar to sub-6GHz), based on 1 TXRU at gNB sweeping 16 beams and all TXRUs at UE sweeping 8 beams; metric = max sum received power |
| Criteria for beam selection for serving TRP | Based on RSRP or 38.215 CSI-SINR metrics |
| Constraints for the range of selective beams per TRP sector | Uniform in azimuth and zenith: azimuth within 65 degree, and zenith within [0,180] |
| Scheduling algorithm | PF based |
| Link adaptation | Based on CSI-RS |
| Traffic Model | Full buffer |
| BS antenna configurations | (M, N, P, Mg, Ng; Mp, Np) = (4, 8, 2, 1, 1; 1, 1). (dV, dH) = (0.5, 0.5) λ |
| BS antenna element radiation pattern | According to TR38.802 |
| UE antenna configurations | (M, N, P, Mg, Ng; Mp, Np) = (2, 4, 2, 1, 2; 1, 1); (dV, dH) = (0.5, 0.5) λ. (dg,V, dg,H) = (0, 0) λ. *Θmg,ng=90°; Ω0,1=Ω0,0+180° |
| UE antenna element radiation pattern | See Table A.2.1-8 in TR 38.802 |
| Inter-panel calibration for UE | Ideal |
| Beam correspondence | Ideal |
| Control and RS overhead | DMRS, CSI-RS, PDCCH |
| Control channel decoding | Ideal |
| UE receiver type | MMSE-IRC |
| BF scheme | 1 TXRU per polarization per panel |
| Transmission scheme | Codebook-based, rank 1 only |
| UE speed | 30 and 120 km/h |
| UE mobility and trajectory | Linear trajectory, inter-cell mobility, and linear trajectory with sampling period = 10 meter and a maximum of N = 11 samples are modelled |
| Performance metrics | Avg. and 5% UPT |
| UE dropping and number drops | 1 UE per site/sector, implying 21 UEs in total per drop, and 10 drops, implying 210 UE in total across 10 drops, |
| Simulation time | 2 seconds |

Table 2: Simulation assumptions for HST

| Parameters | Values |
| --- | --- |
| Scenarios | HST train scenario with 6 RRHs as illustrated in Fig. 5 |
| Frequency Range | FR2 @ 30 GHz, SCS: 120 kHz, BW: 80 MHz |
| Channel Model | Following related assumption in TR 38.802/38.901 |
| TXRU mapping to antenna elements | 2D DFT based beam per polarization |
| TXRU mapping weights | gNB: 16 1D DFT beams (16 in azimuth and 1 in zenith) UE: omnidirectional antenna |
| Criteria for selection for serving RRH | Within RRH based on 1 TXRU at RRH sweeping 16 beams and all TXRUs |
| Criteria for beam selection for serving RRH | Based on RSRP or 38.215 CSI-SINR metrics |
| Constraints for the range of selective beams per RRH sector | Uniform in azimuth and zenith: azimuth within 65 degree, and zenith within [0,180] |
| Scheduling algorithm | Single user |
| Link adaptation | Based on CSI-RS |
| Traffic Model | Full buffer |
| BS antenna configurations | (M, N, P, Mg, Ng; Mp, Np) = (4, 8, 2, 1, 1; 1, 1). (dV, dH) = (0.5, 0.5) λ |
| BS antenna element radiation pattern | According to TR38.802 |
| Inter-panel calibration for UE | Ideal |
| Beam correspondence | Ideal |
| Control and RS overhead | DMRS, CSI-RS, PDCCH |
| Control channel decoding | Ideal |
| UE receiver type | MMSE-IRC |
| BF scheme | 1 TXRU per polarization per panel |
| Transmission scheme | Codebook-based, rank 1 only |
| UE speed | 256 km/h |
| UE mobility and trajectory | Linear trajectory |
| Performance metrics | Relative Throughput |

Any difference or additions to the agreed assumption is described in this document.