3GPP TSG RAN WG1 #102-e   	 	  	                 R1-2007086

e-Meeting, August 17th – 28th, 2020

Agenda item: 7.2.6

Source: Moderator (LG Electronics)

Title: Outcome of email thread [102-e-NR-eMIMO-01]

Document for: Discussion and Decision

- Introduction

This contribution summaries discussion in email thread [102-e-NR-eMIMO-01]

- Background

Codebook based or non-codebook based PUSCH can be scheduled by either DCI format 0_1 or DCI format 0_2. However, in TS 38.213 the UE behavior on determining default pathloss RS for PUSCH transmission is only specified for the PUSCH scheduled by DCI format 0_1. For this reason, OPPO proposed following TP.

TP from OPPO for clause 7.1.1 of TS 38.213

### 7.1.1 UE behavior ## *** Unchanged text is omitted *** - If the PUSCH transmission is scheduled by a DCI format 0_0, and if the UE is provided a spatial setting by PUCCH-SpatialRelationInfo for a PUCCH resource with a lowest index for active UL BWP b of each carrier f and serving cell c, as described in Clause 9.2.2, the UE uses the same RS resource index qd as for a PUCCH transmission in the PUCCH resource with the lowest index - If the PUSCH transmission is scheduled by a DCI format 0_1 or a DCI format 0_2, and if the UE is provided enableDefaultBeamPlForSRS and is not provided PUSCH-PathlossReferenceRS and PUSCH-PathlossReferenceRS-r16, the UE uses the same RS resource index qd as for a SRS resource set with an SRS resource associated with the PUSCH transmission - If - the PUSCH transmission is scheduled by a DCI format 0_0 and the UE is not provided a spatial setting for a PUCCH transmission, or - the PUSCH transmission is scheduled by a DCI format 0_1 or a DCI format 0_2 that does not include an SRI field, or - SRI-PUSCH-PowerControl is not provided to the UE, the UE determines a RS resource index qd with a respective PUSCH-PathlossReferenceRS-Id value being equal to zero where the RS resource is either on serving cell c or, if provided, on a serving cell indicated by a value of pathlossReferenceLinking ## *** Unchanged text is omitted ***

- Discussion

Based on the discussion during preparation phase and due to the fact that DCI0_1 and DCI0_2 are quite similar in terms of power control, it seems reasonable to adopt OPPO’s TP.

FL’s proposal: Adopt the TP in section 2.

Companies’ view

| Company name | View |
| --- | --- |
| DOCOMO | Support. This TP enables to align default spatial relation and default PL-RS for PUSCH scheduled by DCI format 0_2, hence this proposal is beneficial when gNB uses DCI format 0_2. |
| Ericsson | Support. But note that we added default pathloss RS for PUSCH scheduled by DCI format 0_1 only in the previous meeting, which was a very late addition of new functionality. |
| MediaTek | Support. We also think it is good to extend this feature to DCI format 0_2 and this is a missing part in current spec. |
| Huawei, HiSilicon | Though this is a late addition, for completeness of Rel-16 and avoid fragmented discussions in the future, we can live with the FL proposal. With this change, the previously agreed UE feature16-1c in R1-2004970 is assumed to include the support of default spatial relation and PL-RS for PUSCH scheduled by DCI format 0_2. We think it is safer to explicitly capture such understanding as a conclusion, without changing FG 16-1c. |
| OPPO | Support FL’s proposal. In our view, this is a missing part in current spec as a part of the feature of enableDefaultBeamPlForSRS. |
| Apple | We are fine with FL proposal |
| Qualcomm | Fine with the proposal |
| LG | Support the FL’s proposal |
| Sony | Support FL’s proposal. Technically, we think it is good to treat DCI 0_1 and 0_2 equally. It seems late addition, but late better than never. |
| ZTE | Support the FL’s proposal |
| vivo | Fine with the FL’s proposal. Also sympathize with Huawei’s comment on UE feature clarification. |
| Spreadtrum | Support the FL’s proposal |
| Samsung | Support the FL’s proposal |
| Nokia/NSB | Support FL’s proposal |
| CATT | Support FL’s proposal. |

- Conclusion

Based on discussion in email thread [102-e-NR-eMIMO-01], the following conclusion and agreement were made.

Conclusion

- For PUSCH, UE feature 16-1c is applicable to DCI format 0_0, 0_1, and 0_2.

Agreement

- The 7 text proposals in R1-2007005 are endorsed for the editor’s CR on TS38.213, TS38.214

- The following TP in R1-2007092 is endorsed for the editor’s CR on TS38.213 (clause 7.1.1)

7.1.1 UE behavior *** Unchanged text is omitted *** - If the PUSCH transmission is scheduled by a DCI format 0_0, and if the UE is provided a spatial setting by PUCCH-SpatialRelationInfo for a PUCCH resource with a lowest index for active UL BWP b of each carrier f and serving cell c, as described in Clause 9.2.2, the UE uses the same RS resource index qd as for a PUCCH transmission in the PUCCH resource with the lowest index - If the PUSCH transmission is scheduled by a DCI format 0_1 or a DCI format 0_2, and if the UE is provided enableDefaultBeamPlForSRS and is not provided PUSCH-PathlossReferenceRS and PUSCH-PathlossReferenceRS-r16, the UE uses the same RS resource index qd as for a SRS resource set with an SRS resource associated with the PUSCH transmission - If - the PUSCH transmission is scheduled by a DCI format 0_0 and the UE is not provided a spatial setting for a PUCCH transmission, or - the PUSCH transmission is scheduled by a DCI format 0_1 or a DCI format 0_2 that does not include an SRI field, or - SRI-PUSCH-PowerControl is not provided to the UE, the UE determines a RS resource index qd with a respective PUSCH-PathlossReferenceRS-Id value being equal to zero where the RS resource is either on serving cell c or, if provided, on a serving cell indicated by a value of pathlossReferenceLinking *** Unchanged text is omitted ***

Reference

| TDoc | Title | Source |
| --- | --- | --- |
| R1-2005976 | Text proposals for Multi-beam Operation Enhancement | OPPO |