3GPP TSG RAN WG1 #102-e   	 	  	                 			          R1-2007092

e-Meeting, August 17th – 28th, 2020

Agenda item: 7.2.6

Source: Moderator (LG Electronics)

Title: Text proposal on default PL RS for Rel.16 NR_eMIMO Multi-Beam

Document for: Discussion and Decision

- Analysis

| # Reasons for change | # Codebook based or non-codebook based PUSCH can be scheduled by either DCI format 0_1 or DCI format 0_2. However, in TS 38.213 the UE behavior on determining default pathloss RS for PUSCH transmission is only specified for the PUSCH scheduled by DCI format 0_1. |
| --- | --- |
| # Summary of changes | # DCI format 0_2 is added to follow the same behaviour as DCI format 0_1 for default PL RS. |
| # Specs/Sections impacted | # TS 38.213, clause 7.1.1 |
| # Consequences if not approved | # Default PL RS cannot be applied for PUSCH scheduled by DCI format 0_2. |

- Text Proposal

TP for clause 7.1.1 of TS 38.213

### 7.1.1 UE behavior ## *** Unchanged text is omitted *** - If the PUSCH transmission is scheduled by a DCI format 0_0, and if the UE is provided a spatial setting by PUCCH-SpatialRelationInfo for a PUCCH resource with a lowest index for active UL BWP b of each carrier f and serving cell c, as described in Clause 9.2.2, the UE uses the same RS resource index qd as for a PUCCH transmission in the PUCCH resource with the lowest index - If the PUSCH transmission is scheduled by a DCI format 0_1 or a DCI format 0_2, and if the UE is provided enableDefaultBeamPlForSRS and is not provided PUSCH-PathlossReferenceRS and PUSCH-PathlossReferenceRS-r16, the UE uses the same RS resource index qd as for a SRS resource set with an SRS resource associated with the PUSCH transmission - If - the PUSCH transmission is scheduled by a DCI format 0_0 and the UE is not provided a spatial setting for a PUCCH transmission, or - the PUSCH transmission is scheduled by a DCI format 0_1 or a DCI format 0_2 that does not include an SRI field, or - SRI-PUSCH-PowerControl is not provided to the UE, the UE determines a RS resource index qd with a respective PUSCH-PathlossReferenceRS-Id value being equal to zero where the RS resource is either on serving cell c or, if provided, on a serving cell indicated by a value of pathlossReferenceLinking ## *** Unchanged text is omitted ***