3GPP TSG RAN WG1#102-e		R1-2007145

e-Meeting, August 17th – 28th, 2020

Agenda Item:	7.2.2.1.2

Source:	Moderator (Lenovo)

Title:	Summary of Email discussion [102-e-NR-unlic-NRU-DL_Signals_and_Channels-03] to capture earlier agreements and align specifications

Document for:	Discussion, Decision

# Scope and issues based on company submissions

According to the guidance by RAN1 (vice-)chairman, this email discussion to approve TPs is to be finalised by 20 August.

# Summary of Discussion and Suggestions

Regarding IE Names in DCI format 2_0, there is consensus to adopt ZTE's TP#3 in R1-2005598.

Regarding how to capture RRC parameter subcarrierSpacing-r16, there is consensus to adopt a description similar to the suggestion by Sharp's TP #3 in R1-2006553, however as a subbullet of CO-DurationPerCell-r16. A corresponding TP should be drafted and adopted.

Regarding the parameter correction for CSI-RS reception, there is consensus to adopt Sharp's TP#7 in R1-2006553.

Regarding the processing time for SSSG switching, there is no consensus whether ZTE's TP is required or not, even though some change/clarification is seen as necessary by several companies. Discussion could continue based on the following TP:

-------------------------------- < Start of TP for Clause 10.4 of 38.213> ---------------------------------- < Unchanged parts are omitted > A UE can be provided by searchSpaceSwitchingDelay-r16 a number of symbols Pswitch where a minimum value of Pswitch is provided in Table 10.4-1 for UE processing capability 1 and UE processing capability 2 and SCS configuration μ.PswitchPswitchUE processing capability 1 for SCS configuration μ applies unless a UE indicates support of UE processing capability 2. Table 10.4-1: Minimum value of Pswitch [symbols] μ; Minimum Pswitch value for UE processing capability 1 [symbols]; Minimum Pswitch value for UE processing capability 2 [symbols]. 0; 25; 10. 1; 25; 12. 2; 25; 22. < Unchanged parts are omitted >

focussing on a clarification that UE processing capability 1 applies unless capability 2 is indicated and configured.

# Prior Agreements and Discussion

## IE Names in DCI 2_0

ZTE includes the following proposal and TP in R1-2005598:

Proposal 5: The child IEs (availableRB-SetPerCell-r16, CO-DurationPerCell-r16 and searchSpaceSwitchTrigger-r16) quoted in the definition of DCI format 2_0 in TS 38.212 should be replaced by the parent IEs (xxxToAddModList-r16). The TP#3 can be adopted.

----------------------------- < Start of TP#3 for Clause 7.3.1.3.1of 38.212 [4]> ------------------------------- 7.3.1.3.1 Format 2_0 DCI format 2_0 is used for notifying the slot format, COT duration, available RB set, and search space set group switching. The following information is transmitted by means of the DCI format 2_0 with CRC scrambled by SFI-RNTI: - If the higher layer parameter slotFormatCombToAddModList is configured, - Slot format indicator 1, Slot format indicator 2, …, Slot format indicator N, - If the higher layer parameter availableRB-SetPerCell-r16availableRB-SetsToAddModList-r16 is configured, - Available RB set Indicator 1, Available RB set Indicator 2, …, Available RB set Indicator N1, - If the higher layer parameter CO-DurationPerCell-r16co-DurationsPerCellToAddModList-r16 is configured - COT duration indicator 1, COT duration indicator 2, …, COT duration indicator N2. - If the higher layer parameter searchSpaceSwitchTrigger-r16 searchSpaceSwitchTriggerToAddModList-r16 is configured - Search space set group switching flag 1, Search space set group switching flag 2, …, Search space set group switching flag M. The size of DCI format 2_0 is configurable by higher layers up to 128 bits, according to Clause 11.1.1 of [5, TS 38.213]. -------------------------------------------- < End of text proposal> ------------------------------------------------

| Q1: Do you agree with the proposal/TP by ZTE? |  |
| --- | --- |
| Company | Comment |
| Qualcomm | Agree |
| LG Electronics | Agree |
| Huawei, HiSilicon | Agree |
| Samsung | Agree |
| Sharp | Agree |
| Nokia, NSB | Agree |
| ZTE, Sanechips | Agree |
| Spreadtrum | Agree |
| vivo | Agree |

## Capturing RRC parameter subcarrierSpacing-r16

Sharp includes the following in R1-2006553:

Proposal 3:

- TS38.213 to implement subcarrierSpacing configuration for CO duration indication.

  - Adopt the following Text proposal #3.

Text proposal #3 --------- beginning of text proposal for TS 38.213 11.1.1 UE procedure for determining slot format <omitted> For each serving cell in the set of serving cells, the UE can be provided: - an identity of the serving cell by servingCellId - a location of a SFI-index field in DCI format 2_0 by positionInDCI - a set of slot format combinations by slotFormatCombinations, where each slot format combination in the set of slot format combinations includes - one or more slot formats indicated by a respective slotFormats for the slot format combination, and - a mapping for the slot format combination provided by slotFormats to a corresponding SFI-index field value in DCI format 2_0 provided by slotFormatCombinationId - for unpaired spectrum operation, a reference SCS configuration by subcarrierSpacing and, when a supplementary UL carrier is configured for the serving cell, a reference SCS configuration by subcarrierSpacing2 for the supplementary UL carrier - for paired spectrum operation, a reference SCS configuration for a DL BWP by subcarrierSpacing and a reference SCS configuration for an UL BWP by subcarrierSpacing2 - a location of an available RB set indicator field in DCI format 2_0 that is - one bit, if intraCellGuardBandDL-r16 for the serving cell indicates no intra-cell guard-bands are configured, where a value of '1' indicates that the serving cell is available for receptions, a value of '0' indicates that the serving cell is not available for receptions, by availableRB-SetPerCell-r16, and the serving cell remains available or unavailable for reception until the end of the indicated channel occupancy duration - a bitmap having a one-to-one mapping with the RB sets [6, TS 38.214] of the serving cell, if intraCellGuardBandDL-r16 for the serving cell indicates intra-cell guard-bands are configured, where the bitmap includes NRB,set,DL bits and NRB,set,DL is the number of RB sets in the serving cell, a value of '1' indicates that an RB set is available for receptions, a value of '0' indicates that an RB set is not available for receptions, by availableRB-SetPerCell-r16 and a RB set remains available or unavailable for receptions until the end of the indicated channel occupancy duration - a location of a channel occupancy duration field in DCI format 2_0, by CO-DurationPerCell-r16, that indicates a remaining channel occupancy duration for the serving cell starting from a first symbol of a slot where the UE detects the DCI format 2_0 by providing a value from CO-DurationList-r16. The channel occupancy duration field includes maxlog2COdurationListSize,1 bits, where COdurationListSize is the number of values provided by CO-DurationList-r16. If CO-DurationPerCell-r16 is not provided, the remaining channel occupancy duration for the serving cell is a number of slots, starting from the slot where the UE detects the DCI format 2_0, that the SFI-index field value provides corresponding slot formats - a reference SCS configuration for the list of Channel Occupancy durations, by subcarrierSpacing-r16 - a location of a search space set group switching flag field in DCI format 2_0, by SearchSpaceSwitchTrigger-r16, that indicates a group from two groups of search space sets for PDCCH monitoring for scheduling on the serving cell as described in Clause 10.4.

| Q2: Please provide your view on Sharp's proposal/TP. |  |
| --- | --- |
| Company | Comment |
| Qualcomm | Agree in general, but might be better to put this as a sub-bullet of CO-DurationPerCell-r16. |
| LG Electronics | Agree, but as Qualcomm pointed out, instead of having separate bullet, it can be merged with the above bullet describing CO-DurationPerCell-r16. |
| Huawei, HiSilicon | Agree with the intention. It should be differentiated from the existing reference SCS for SFI |
| Samsung | Agree with Qualcomm |
| Sharp | This change is necessary. We are fine to move it to a sub-bullet of CO-DurationPerCell-r16. |
| Nokia, NSB | OK with QC proposal |
| ZTE, Sanechips | Agree with Qualcomm’s proposal. |
| Spreadtrum | Agree with Qualcomm’s proposal. |
| vivo | Agree with Qualcomm’s proposal. |

## Parameter correction for CSI-RS reception

Sharp includes the following proposal and TP in R1-2006553:

Proposal 7:

- SlotFormatCombinationsPerCell should be referred to for determination of whether or not SFI is configured.

  - Adopt the following Text proposal #7.

| Text proposal #7 --------- beginning of text proposal for TS 38.213 11.1.1 UE procedure for determining slot format <omitted> For operation with shared spectrum channel access, if a UE is configured by higher layers to receive a CSI-RS and the UE is provided CO-DurationPerCell-r16 and is not provided SlotFormatCombinationsPerCell, for a set of symbols of a slot that are indicated as downlink or flexible by tdd-UL-DL-ConfigurationCommon or tdd-UL-DL-ConfigurationDedicated, or when tdd-UL-DL-ConfigurationCommon and tdd-UL-DL-ConfigurationDedicated are not provided, the UE cancels the CSI-RS reception in the set of symbols of the slot that are not within the indicated remaining channel occupancy duration. |  |
| --- | --- |
| Q3: Please provide your view on Sharp's proposal/TP. |  |
| Company | Comment |
| Qualcomm | Support |
| LG Electronics | Support |
| Huawei, HiSilicon | Agree |
| Samsung | Support |
| Sharp | This change is necessary. |
| Nokia, NSB | Support, since we had the same proposal as Sharp |
| ZTE, Sanechips | Support |
| Spreadtrum | Support |
| vivo | Support |

## Processing time for switching

ZTE includes the following proposal/TP in R1-2005598:

Proposal 4: The description for UE processing time of SSS group switching in TS 38.213 should be clarified. The following TP#2 can be considered.

-------------------------------- < Start of TP#2 for Clause 10.4 of 38.213 [1]> ---------------------------------- 10.4 Search space set group switching < Unchanged parts are omitted > A UE can be provided by searchSpaceSwitchingDelay-r16 a number of symbols Pswitch. The UE applies the indicated Pswitch and does not expect the indicated Pswitch to be smaller than a minimum value of Pswitch. If the UE indicates a corresponding capability, where a the minimum value of Pswitch is provided in Table 10.4-1 for UE processing capability 1 and UE processing capability 2 and SCS configuration μ applies. Otherwise, a minimum value of Pswitch provided in Table 10.4-1 for UE processing capability 1 and SCS configuration μ applies If the UE indicates a corresponding capability, the UE applies the Pswitch value for UE processing capability 2; otherwise, the UE applies the Pswitch value for UE processing capability 1 for SCS configuration μ. Table 10.4-1: Minimum value of Pswitch [symbols] μ; Minimum Pswitch value for UE processing capability 1 [symbols]; Minimum Pswitch value for UE processing capability 2 [symbols]. 0; 25; 10. 1; 25; 12. 2; 25; 22. < Unchanged parts are omitted > --------------------------------------------- < End of text proposal> ------------------------------------------------

| Q4: Please provide your view on ZTE's proposal/TP. |  |
| --- | --- |
| Company | Comment |
| Qualcomm | Agree some clean up of the language is needed. We proposed the following: “A UE can be provided by searchSpaceSwitchingDelay-r16 a number of symbols 𝑃𝑠𝑤𝑖𝑡𝑐ℎ where a minimum value of 𝑃𝑠𝑤𝑖𝑡𝑐ℎ is provided in Table 10.4-1 for UE processing capability 1 and UE processing capability 2 and SCS configuration 𝜇. UE can indicate if UE processing capability 2 is supported. If UE processing capability 2 is not supported, UE processing capability 1 is supported.” |
| LG Electronics | Support ZTE’s TP. |
| Huawei, HiSilicon | The existing TP is clear. |
| Samsung | Current specification seems clear. |
| Sharp | Clean-up on this paragraph was made in the last meeting. Not sure if further modification is necessary, as the current description is not incorrect. |
| Nokia, NSB | Same comments as last meeting - There is no need to clarify that UE shall not indicate value smaller than UE capability. This was also the specification editor’s comment previously. - It is important to clarify that capability 1 applies by default if capability 2 is not configured If CR follows the above principle, we are fine to support. Otherwise not. |
| ZTE, Sanechips | Support our TP, which is to reflect the previous agreement more accurately. |
| Spreadtrum | Current spec seems clear. |
| vivo | Support the TP |