3GPP TSG RAN WG1 #102-e	R1-2007195

e-Meeting, August 17th – 28th, 2020

Source:	moderator (vivo)

Title:	Feature lead summary#3 on NRU configured grant enhancement

Agenda Item:	7.2.2.2.4

Document for:	Discussion and Decision

Introduction

According to agreements following TPs are provided with side information as below. The relevant proposals can be found in [1].

Agreed TPs with side information

TP#1

TP for TS 38.214

Reason for change: when cg-RetransmissionTimer is provided the UE determines the redundancy version for uplink transmission, the text in square bracket was redundant.

Summary of change: the text in square bracket [, except for the redundancy version of the first repetition that is set to 0] is removed

Consequences if not approved: it is unclear how to interpret the text in square bracket in the spec

Clauses affected: 38.214, section 6.1.2.3.1

---------------------------------------------- TP for 38.214 6.1.2.3.1--------------------------------------------------

-------------------------------Unchanged Texts Omitted -----------------------------

The procedures described in this clause apply to PUSCH transmissions of PUSCH repetition Type A with a Type 1 or Type 2 configured grant.

The higher layer parameter repK-RV defines the redundancy version pattern to be applied to the repetitions. If cg-RetransmissionTimer is provided, the redundancy version for uplink transmission with a configured grant is determined by the UE[, except for the redundancy version of the first repetition that is set to 0] . If the parameter repK-RV is not provided in the configuredGrantConfig and cg-RetransmissionTimer is not provided, the redundancy version for uplink transmissions with a configured grant shall be set to 0. If the parameter repK-RV is provided in the configuredGrantConfig and cg-RetransmissionTimer is not provided, for the nth transmission occasion among K repetitions, n=1, 2, …, K, it is associated with (mod(n-1,4)+1)th value in the configured RV sequence. If a configured grant configuration is configured with Configuredgrantconfig-StartingfromRV0 set to ‘off’, the initial transmission of a transport block may only start at the first transmission occasion of the K repetitions.

-------------------------------Unchanged Texts Omitted -----------------------------

-----------------------------------------------End-------------------------------------------

TP#2

TP for TS 38.214

Reason for change: it was agreed in previous RAN1 meeting that for configured grant time domain resource allocation, the mechanisms in Rel-15 (both Type 1 and Type 2) are extended, however the description of PUSCH resource allocation in TS 38.214 may be interpreted as it is only allowed for Type-1 CG PUSCH, thus it is clarified that both Type-1 and Type-2 are supported

Summary of change: in section 6.1.2.3 a phrase is added to clarify that both Type 1 and Type 2 PUSCH transmission are supported

Consequences if not approved: spec is unclear whether both Type-1 and Type-2 PUSCH transmission are supported or not.

Clauses affected: 38.214, section 6.1.2.3

----------------------------------------- Start of TP for TS 38.214 --------------------------------------------

6.1.2.3	Resource allocation for uplink transmission with configured grant

-----------------------------------Unchanged Texts Omitted ---------------------------------

A set of allowed periodicities P are defined in [12, TS 38.331]. The higher layer parameter cg-nrofSlots-r16, provides the number of consecutive slots allocated within a configured grant period. The higher layer parameter cg-nrofPUSCH-InSlot-r16 provides the number of consecutive PUSCH allocations within a slot, where the first PUSCH allocation follows the higher layer parameter timeDomainAllocation for Type 1 PUSCH transmission or the higher layer configuration according to [10, TS 38.321], and UL grant received on the DCI for Type 2 PUSCH transmissions,, and the remaining PUSCH allocations have the same length and PUSCH mapping type, and are appended following the previous allocations without any gaps. The same combination of start symbol and length and PUSCH mapping type repeats over the consecutively allocated slots.

----------------------------- Unchanged Texts Omitted -------------------------------

----------------------------------------------- End -------------------------------------------------------

TP#3

TP for TS 38.214

Reason for change: description on how to validate for activation/release of a configured UL grant Type 2 PDCCH when DCI 0_1 scrambled by CS-RNTI includes NDI field for multiple PUSCHs is missing in spec

Summary of change: a column in table Table 6.1.2.1.1-1A added to support the case when pusch-Config includes pusch-TimeDomainAllocationList –ForMultiPUSCH

Consequences if not approved: spec is not clear how to validate for activation/release of a configured UL grant Type 2 PDCCH when DCI 0_1 scrambled by CS-RNTI includes NDI field for multiple PUSCHs

Clauses affected: 38.214, section 6.1.2.1.1

----------------------------------------- TP #2 for 38.214 6.1.2.1.1------------------------------------------------------

--------------------------------- Unchanged Texts Omitted ------------------------------

Table 6.1.2.1.1-1A Applicable PUSCH time domain resource allocation for DCI format 0_1 in UE specific search space scrambled with C-RNTI, MCS-C-RNTI, CS-RNTI or SP-CSI-RNTI

| pusch-ConfigCommon includes pusch-TimeDomainAllocationList | pusch-Config includes pusch-TimeDomainAllocationList | pusch-Config includes pusch-TimeDomainAllocationList -ForDCIformat0_1 | pusch-Config includes pusch-TimeDomainAllocationList –ForMultiPUSCH | PUSCH time domain resource allocation to apply |
| --- | --- | --- | --- | --- |
| No | No | No | No | Default A |
| Yes | No | No | No | pusch-TimeDomainAllocationList provided in pusch-ConfigCommon |
| No/Yes | Yes | No | No | pusch-TimeDomainAllocationList provided in pusch-Config |
| No/Yes | No/Yes | Yes | - | pusch-TimeDomainAllocationList-ForDCIformat0_1 provided in pusch-Config |
| No/Yes | No/Yes | - | Yes | pusch-TimeDomainAllocationList –ForMultiPUSCH provided in pusch-Config |

-------------------------------- Unchanged Texts Omitted -----------------------------------

-------------------------------------------------END --------------------------------------------------------------

References

[1] R1-2007112, “Feature lead summary on NRU configured grant enhancement#2”, vivo, RAN1#102-e, 17th – 28th Aug, 2020