3GPP TSG RAN WG1 Meeting #102-e	R1-2007275

E-Meeting, August 17 – 28, 2020

Agenda Item:	6.2.3

Source:	Moderator (Huawei)

Title:	TP to 36.211 on higher layer parameters for additional SRS

Document for:	Discussion and Decision

Introduction

This document provides the text proposal as outcomes of the following email discussion [1]:

[102-e-LTE-LTE_DL_MIMO_EE-02] Correction for power control, and alignment with higher layer terminologies – Yubo (Huawei)

- Issue #2 from FL prep phase summary on LTE DL MIMO efficiency enhancement

- Discussion and agreement by 8/19, TPs by 8/21

Discussion

TP on resource reservation to special subframes

Reason for changes:

The terms referring to higher layer parameters for additional SRS are not aligned with TS 36.331.

Summary of changes:

The terms referring to higher layer parameters for additional SRS are aligned with TS 36.331.

Specs/sections impacted:

36.211 sections 5.5.3.1.2, 5.5.3.2.2

Consequences if not approved:

There is ambiguity when referring to higher layer parameters for additional SRS.

-----------------------------------------------------Start of Text Proposal-----------------------------------

<unchanged parts are omitted>

- 5.5.3.1.2	Sequence generation for additional SRS

The sounding reference signal rSRS(p)n is defined by clause 5.5.3.1.1 with the following exceptions

-	nSRScs is given by the higher-layer parameter srs-CyclicShiftAdd

-	Nap is given by the higher-layer parameter srs-AntennaPortAdd

-	KTC is given by the higher-layer parameter srs-TransmissionCombNumAdd

<Unchanged parts are omitted>

- 5.5.3.2.2	Mapping to physical resources for additional SRS

An additional SRS spans one or more OFDM symbols in the time domain, where

-	the starting OFDM symbol l0 within the subframe is given by the higher-layer parameter srs-StartPosAdd;

-	the duration N in number of OFDM symbols, including potential guard symbols, is given by the higher-layer parameter srs-DurationAdd;

Mapping to physical resources shall be done according to clause 5.5.3.2.1 with the following exceptions:

-	frequency hopping between OFDM symbols is supported and if a UE is configured by higher layer parameter srs-GuardSymbolFH-Add, a guard symbol is added between every frequency hop;

-	antenna switching within a subframe is supported and if a UE is configured by higher layer parameter srs-GuardSymbolAS-Add, a guard symbol is added between every antenna switching;

-	nSRS=l'R where l' is the additional SRS transmission number not counting guard symbol(s) within the subframe with l'=0 corresponding to the starting OFDM symbol l0, and R∈1,2,3,4,6,7,8,9,12,13 is the repetition factor given by the higher-layer parameter srs-RepNumAdd;

-	BSRS is given by the higher-layer parameter srs-BandwidthAdd;

-	bhop is given by the higher-layer parameter srs-HoppingBandwidthAdd;

-	NFH is the number of frequency hops with the same antenna/antenna pair for additional SRS, derived from N=RNFH+NFH-1GFH if antenna switching is not configured for additional SRS, and from N=RNASNFH+NAS-1GAS+NFH-1NASGFH+NAS-1(1-GAS)GFH if antenna switching is configured for additional SRS, where R is the repetition factor given by the higher-layer parameter srs-RepNumAdd, NAS is the number of antenna switches for additional SRS defined in 8.2 of [4], GAS∈0, 1 is the guard-symbol configuration for antenna switching given by the higher-layer parameter srs-GuardSymbolAS, GFH∈0, 1 is the guard symbol configuration for frequency hopping given by the higher-layer parameter srs-GuardSymbolFH, and N is given by the higher-layer parameter srs-DurationAdd;

-	nRRC is given by the higher-layer parameter srs-FreqDomainPosAdd;

-	Nap is given by the higher-layer parameter srs-AntennaPortAdd;

-	nSRScs is given by the higher-layer parameter srs-CyclicShiftAdd;

-	KTC is given by the higher-layer parameter srs-TransmissionCombNumAdd;

-	kTC is given by the higher-layer parameter srs-TransmissionCombAdd.

<unchanged parts are omitted>

------------------------------------------------------End of Text Proposal------------------------------------

- References

- R1-2007274	Feature summary on [102-e-LTE-LTE_DL_MIMO_EE-02]		Moderator (Huawei)