3GPP TSG RAN WG1 Meeting #102-e	R1-2007276

E-Meeting, August 17 – 28, 2020

Agenda Item:	6.2.3

Source:	Moderator (Huawei)

Title:	TP to 36.213 on higher layer parameters for additional SRS

Document for:	Discussion and Decision

Introduction

This document provides the text proposal as outcomes of the following email discussion [1]:

[102-e-LTE-LTE_DL_MIMO_EE-02] Correction for power control, and alignment with higher layer terminologies – Yubo (Huawei)

- Issue #2 from FL prep phase summary on LTE DL MIMO efficiency enhancement

- Discussion and agreement by 8/19, TPs by 8/21

Discussion

TP on resource reservation to special subframes

Reason for changes:

The terms referring to higher layer parameters for additional SRS are not aligned with TS 36.331.

Summary of changes:

The terms referring to higher layer parameters for additional SRS are aligned with TS 36.331.

Specs/sections impacted:

36.213 sections 5.1.3, 5.1.3.1, 8.2

Consequences if not approved:

There is ambiguity when referring to higher layer parameters for additional SRS.

-----------------------------------------------------Start of Text Proposal-----------------------------------

### 5.1.3	Sounding Reference Symbol (SRS)

#### 5.1.3.1	UE behaviour

The setting of the UE Transmit power for the SRS transmitted on subframe i for serving cell  is defined by:

for SRS transmission given trigger type 2 or for serving cell  with frame structure type 2, and not configured for PUSCH/PUCCH transmission

[dBm]

otherwise

[dBm]

where

-	 is the configured UE transmit power defined in [6] in subframe i for serving cell .

-	 is semi-statically configured by higher layers for m=0 and m=1 for serving cell . For SRS transmission given trigger type 0 then m=0 and for SRS transmission given trigger type 1 then m=1.

-	 is the bandwidth of the SRS transmission in subframe i for serving cell  expressed in number of resource blocks.

-	 is the current PUSCH power control adjustment state for serving cell , see Subclause 5.1.1.1.

-	 and  are parameters as defined in Subclause 5.1.1.1 for subframe , where .

-	 is the higher layer parameter alpha-SRS for SRS transmission given trigger type 0, 1, or is the higher layer parameter alphaSRS-Add for SRS transmission given trigger type 2, configured by higher layers for serving cell .

- 	is a parameter composed of the sum of a component  which is p0-Nominal-PeriodicSRS, p0-Nominal-AperiodicSRS, or p0-NominalSRS-Add provided from higher layers for m=0, 1 or 2 respectively, and a component  which is p0-UE-PeriodicSRS, p0-UE-AperiodicSRS, or p0-UE-SRS-Add provided by higher layers for m=0, 1 or 2 respectively, for serving cell . For SRS transmission given trigger type 0 then m=0 and for SRS transmission given trigger type 1 then m=1 and for SRS transmission given trigger type 2 then m=2.

---- Unchanged parts are omitted ----

-	If higher layer parameter fieldTypeFormat3B or fieldTypeFormat3B-SRS-Add indicates 2-bit TPC command, the  dB values signalled on PDCCH with DCI format 3B are given in Table 5.1.1.1-2 by replacing  with , or if higher layer parameter fieldTypeFormat3B or fieldTypeFormat3B-SRS-Add indicates 1-bit TPC command, the  dB values signalled on PDCCH with DCI format 3B are given in Table 5.1.1.1-3 by replacing  with .

---- Unchanged parts are omitted ----

## 8.2	UE sounding procedure

---- Unchanged parts are omitted ----

For a TDD serving cell,

-	If the serving cell not configured for PUSCH/PUCCH transmission, or if the UE supports ce-srsEnhancement-r14

-	For trigger type 0, the UE can be configured with more than one configuration of SoundingRS-UL-ConfigDedicatedUpPTsExt and/or SoundingRS-UL-ConfigDedicated, and the SRS parameters in each of the configurations shall be used. The UE is not expected to receive more than one SRS configuration indicating SRS transmission on multiple symbols in different subframes within a half frame.

-	For trigger type 1, the UE can be configured with more than one configuration of SoundingRS-UL- ConfigDedicatedAperiodicUpPTsExt and/or SoundingRS-UL- ConfigDedicatedAperiodic, and the SRS parameters in each of the configurations shall be used. The UE is not expected to receive more than one SRS configuration indicating SRS transmission on multiple symbols in different subframes within a half frame.

- 	For trigger type 2, the parameters in SoundingRS-UL-ConfigDedicatedAdd shall be used.

-	Otherwise

-	For trigger type 0, if SoundingRS-UL-ConfigDedicatedUpPTsExt is configured, the SRS parameters in SoundingRS-UL-ConfigDedicatedUpPTsExt shall be used; otherwise, SoundingRS-UL-ConfigDedicated shall be used.

-	For trigger type 1, if SoundingRS-UL-ConfigDedicatedAperiodicUpPTsExt is configured, the SRS parameters in SoundingRS-UL-ConfigDedicatedAperiodicUpPTsExt shall be used; otherwise, SoundingRS-UL-ConfigDedicatedAperiodic shall be used.

-	For trigger type 2, the parameters in SoundingRS-UL-ConfigDedicatedAdd shall be used.

------------------------------------------------------End of Text Proposal------------------------------------

- References

- R1-2007274	Feature summary on [102-e-LTE-LTE_DL_MIMO_EE-02]		Moderator (Huawei)