3GPP TSG RAN WG1 #102-e Meeting	R1-2007348

e-Meeting, August 17th – 28th, 2020

Title:	[Draft] Reply LS on maximum data rate for NR sidelink

Response to:	LS on UE capability (R2-2004087)

Release:	Rel-16

Work Item:	5G_V2X_NRSL

Source:	RAN1, Samsung

To:	RAN2

Cc:

Contact Person:

Name:	Jeongho Yeo

- E-mail Address:	jeongho7.yeo@samsung.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments: 	-

1. Overall Description:

In RAN1#102-e, RAN1 reached the agreements (see Appendix) to define the maximum data rate for NR sidelink, from which RAN1 suggest the following for the maximum data rate for NR sidelink.

For NR sidelink, the approximate data rate is computed as follows. data rate (in Mbps)=10-6⋅vLayers⋅Qm⋅f⋅Rmax⋅NPRBBW,μ⋅12Tsμ⋅1-OH wherein Rmax = 948/1024, vLayers is the the maximum number of supported layers for sidelink transmission (or reception) according to UE capability, Qm is the maximum supported modulation order between 6 or 8 according to UE capability, f is the scaling factor for sidelink transmission and reception, respectively, and can take the values 1, 0.8, 0.75, and 0.4. is the numerology (as defined in TS 38.211 [6]) is the average OFDM symbol duration in a subframe for numerology , i.e. . Note that normal cyclic prefix is assumed. NPRBBW,μ is the maximum possible RB allocation in bandwidth BW for PSSCH, where BW is the UE supported maximum bandwidth in the given band or band combination, OH is the overhead and takes the following values [0.23], for frequency range FR1 for SL [0.25], for frequency range FR2 for SL

2. Actions:

To RAN2

RAN1 respectfully asks RAN2 to take the above information into account in their further work

3. Date of Next Meetings:

TSG-RAN WG1 Meeting #103-e 	October 26 – November 13, 2020	e-meeting

TSG-RAN WG1 Meeting #104 	March 1 – 5, 2021			Athens, Greece

- Appendix: Agreements from RAN1#102-e

Agreements:

- The supported max data rate for SL is based on NR’s formula (based on BW, layer..) in Clause 4.1.2 of TS38.306

- The supported max data rate for SL is dependent on UE capability (which is defined either per band or per band combinations or a combination thereof, to be decided by 08/20).

  - Defined per band combinations

- RAN1 further discusses Qm, f, OH values.

Agreements:

- For max data rate for SL Tx (or Rx), v_layers is the maximum number of supported layers for SL Tx (or Rx) according to UE capability.

Agreements:

- For max data rate for SL, Qm is determined:

  - between 64QAM and 256QAM (based on the existing UE capability)

Agreements:

- For max data rate for SL, N_PRB^BW is the maximum possible RB allocation in bandwidth BW for PSSCH, where BW is the UE supported maximum bandwidth in the given band or band combination.

Agreements:

- For max data rate for SL, OH value is determined as [0.23] for FR1 and [0.25] for FR2.

Agreements:

The scaling factor, f, is determined among {1, 0.8, 0.75, 0.4} for max data rate of SL Tx and Rx, respectively.