3GPP TSG RAN WG1 #102-e		R1-2007356

e-Meeting, 17th – 28th August, 2020

Title:	LS on evaluation methodology for UE power saving enhancements

Response to:

Release:	Release 17

Work Item:	NR_UE_pow_sav_enh-Core

Source:	MediaTek, RAN1

To:	RAN2

Cc:	RAN4

Contact Person:

#### Name: Wei-De Wu

#### E-mail Address:	Weide.Wu@mediatek.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	None

1. Overall Description:

In RAN1#102-e, RAN1 updates the evaluation methodology related to paging enhancement(s) for Rel-17 UE power saving enhancements. RAN1 would like to inform RAN2 the following related agreements:

Agreements:

For study of Rel-17 paging enhancement, the following are assumed as a baseline for FR1 and FR2:

- Reference configuration for FR1/FR2 as specified in Section 8.1.1/8.1.2 of TR 38.840

  - Note: the setting for some PDSCH parameters may not be applicable for RedCap UEs

- Baseline paging cycle length: [1.28] second

- SS burst related assumptions:

  - 20 ms periodicity

  - 2 ms duration for serving cell RRM measurement, which can overlap with the one for synchronization before PO

  - FFS time/frequency tracking

- Measurement related assumptions:

  - 20 ms SMTC periodicity

  - 2 ms SMTC window for intra-frequency RRM measurement, assuming synchronized deployment

  - [5 ms SMTC window and 6 ms measurement gap for inter-frequency RRM measurement]

    - Note: RAN4 requirement assumes one frequency layer per measurement gap, and 0.5 ms is assumed for switch in/out a frequency layer

- Note: the inclusion of potential TRS/CSI-RS occasions can be considered

Agreements:

The following power consumption model for FR1 is utilized for the evaluations of Rel-17 UE power saving enhancements in idle/inactive mode

- FFS: FR2 power consumption model for idle/inactive mode operations

| Power State | Relative Power (FR1 reference from TR 84.840) | Relative Power (Idle/inactive-mode operation with reception bandwidth 20 MHz) |
| --- | --- | --- |
| Deep Sleep (PDS) | 1 | 1 |
| Light Sleep (PLS) | 20 | 20 |
| Micro sleep (PMS) | 45 | 45 |
| PDCCH-only (PPDCCH) | 100 | 50Note |
| PDCCH + PDSCH (PPDCCH+PDSCH) | 300 | 120 |
| PDSCH-only (PPDSCH) | 280 | 112 |
| SSB/CSI-RS proc. (PSSB) | 100 (synchronization or serving cell measurement) | 50 |
| Intra-frequency RRM measurement (Pintra) | · 150 (synchronous case, N=8, measurement only; Pintra, meas-only) · 200 (combined search and measurement; Pintra, search+meas) | · [60] (synchronous case, N=8, measurement only; Pintra, meas-only) · [80] (combined search and measurement; Pintra, search+meas) |
| Inter-frequency RRM measurement (Pinter) | · 150 (measurement only per freq. layer; Pinter, meas-only) · 150 (neighbor cell search power per freq. layer; Pinter, search-only) · Micro sleep power assumed for switch in/out a freq. layer | · [60] (measurement only per freq. layer; Pinter, meas-only) · [150] (neighbor cell search power per freq. layer; Pinter, search-only) · Micro sleep power assumed for switch in/out a freq. layer |
| Note: Power scaling to 20MHz reception bandwidth follows the rule in Section 8.1.3 of TR 38.840, i.e., max{reference power * 0.4, 50}. |  |  |

Agreements:

Group paging rate of 10% is assumed for the evaluation of Rel-17 paging enhancement

- FFS: Another group paging rate > 10%

- Note: If UE sub-grouping is applied, the sub-group paging rate can be reduced w.r.t. the total sub-group number for a PO

Agreements:

For the study on paging enhancements to reduce unnecessary paging reception, the following metrics are considered:

- UE power saving gain (relative to a given feature or overall)

- Impact to UE paging detection probability

  - FFS: Link level simulation assumptions

- System impact, including

  - Additional resource overhead and its implications

  - Impact to Rel-15/Rel-16 idle/inactive-mode UEs and connected-mode UEs

  - Impact to other legacy functionalities, including SI change and ETWS indication

  - [Note: NW energy consumption evaluation is not precluded]

Agreements:

- For the study of paging enhancement, 1, 2, or 3 SS burst processing is assumed before PO

  - Note: in choosing one or more values (1, 2, or 3) for the evaluations, companies to provide justification

2. Actions:

To RAN2:

RAN1 respectfully asks RAN2 to take the evaluation methodology for paging enhancement(s) into account in the corresponding RAN2 study.

3. Date of Next RAN1 Meetings:

TSG-RAN WG1 Meeting 103-e 	26 October – 13 November 2020		e-Meeting

TSG-RAN WG1 Meeting 104	01 – 05 March 2021 	 		Athens, Greece