3GPP TSG RAN WG1 Meeting #102-e		                                                      R1-2007369

e-Meeting, August 17th – 28th, 2020

Source:	Moderator (CATT)

Title:	Text proposal from Email discussion thread #01 for AI 7.2.4.3 Sidelink synchronization mechanism

Agenda Item:	7.2.4.3

Document for:	Discussion and Decision

Introduction

In the email discussion [102-e-NR-5G_V2X_NRSL-SYNC-01] of this meeting, one agreement is made as follows:

Agreements: - When both tdd-UL-DL-ConfigurationCommon and sl-TDD-Configuration are not provided [for FDD band or an ITS band], a bit sequence of sl-TDD-Config in PSBCH is set to the following values representing the meaning that all the slots are uplink slots: - a0=1 - a1, a2, a3,a4=1, 1, 1, 1 - a5,a6, a7, a8,a9, a10, a11=1, 1, 1, 1, 1, 1, 1

The coversheet information can be found in section 2. The corresponding TP is also endorsed as it in section 3.

Coversheet information

- Reason for change: For FDD case or UE out of coverage, higher layer parameter tdd-UL-DL-ConfigCommon and sl-TDD-Configuration will be absent in SIB1. A code point in PSBCH should be used to indicate this scenario when it is happen.

- Summary of changes: When both tdd-UL-DL-ConfigurationCommon and sl-TDD-Configuration are not provided for FDD band or an ITS band, a bit sequence a0~a11 of sl-TDD-Config in PSBCH is set to all “1” representing the meaning that all the slots are uplink slots.

- Specs/sections impacted: TS 38.213, section 16.1

- Consequences if not approved: Incomplete specification; UE cannot do UL slots transformation and mapping it to PSBCH.

Text proposal for TS 38.213

------------------------------------------------------ Start of Draft TP of 38.213--------------------------------------------------

16.1 Synchronization procedures

------------------------------------------------ < Unchanged parts are omitted > --------------------------------------------

For transmission of an S-SS/PSBCH block, a UE includes a bit sequence a0, a1, a2, a3, …, a11 in the PSBCH payload to indicate sl-TDD-Config and provide a slot format over a number of slots, where for a paired spectrum, or

if neither tdd-UL-DL-ConfigurationCommon nor sl-TDD-Configuration is provided for a spectrum indicated with only PC5 interface in Table 5.2E.1-1 defined in Clause 5.2 [TS 38.101-1],

-	a0, a1, a2, a3,a4a5, a6, a7,a8, a9, a10, a11 are set to all “1”;

otherwise

-	a0=0 if pattern1 is provided by tdd-UL-DL-ConfigurationCommon; a0=1 if both pattern1 and pattern2 are provided by tdd-UL-DL-ConfigurationCommon as described in Clause 11.1

-	a1, a2, a3,a4 are determined based on

-	P in pattern1 as described in Table 16.1-1 for a0=0

-	P in pattern1 and P2 in pattern2 as described in Table 16.1-2 for a0=1

where P and P2 are as described in Clause 11.1

-	a5, a6, a7,a8, a9, a10, a11 are the 7th to 1st LSBs of uslotsSL, respectively

-	for a0=0, uslotsSL=uslots*2μ-μref+usym*2μ-μrefL+I1

-	for a0=1, uslotsSL=uslots,2*2μ-μref+usym,2*2μ-μrefL+I2w*P*2μ+1w+uslots*2μ-μref+usym*2μ-μrefL+I1w

where

-	L is the number of symbols in a slot: L=12 if cyclicPrefix-SL = "ECP"; else, L=14

-	I1 is 1 if usym*2μ-μref mod L≥ L-Y, else I1 is 0

-	I2 is 1 if usym,2*2μ-μref mod L≥L-Y, else I2 is 0

-	Y is the sidelink starting symbol index provided by sl-StartSymbol

-	w is the granularity of slots indication as described in Table 16.1-2

-	μref, uslots, usym, uslots,2, usym,2 are described in Clause 11.1

-	μ=0, 1, 2, 3 corresponds to SL SCS as defined in [4, TS 38.211]

If a UE would transmit or receive an S-SS/PSBCH block, and the transmission or reception would overlap in time with transmissions or receptions on the sidelink using E-UTRA radio access, the UE transmits or receives the signal/channel with the higher priority.

If a UE would transmit or receive sidelink synchronization signals for E-UTRA radio access, and the transmission or reception would overlap in time with sidelink transmissions or receptions using NR radio access, the UE transmits or receives the signal/channel with the higher priority.

------------------------------------------------ < Unchanged parts are omitted > --------------------------------------------

----------------------------------------------------- End of Draft TP of 38.213 ------------------------------------------------