3GPP TSG RAN WG1 Meeting #102-e														R1-2007393

e-Meeting, August 17th – 28th, 2020

Source:	Intel Corporation

Title:	Summary of HST-SFN evaluation assumptions after RAN1#102-e

Agenda item:	8.1.2.4

Document for:	Discussion and Decision

# Introduction

In RAN#86 meeting the work item on enhanced MIMO support was agreed for Rel-17 [1]. The objectives of WID include enhancements to multi-TRP transmission scheme in HST-SFN scenario.

2. Enhancement on the support for multi-TRP deployment, targeting both FR1 and FR2: … d. Enhancement to support HST-SFN deployment scenario: i. Identify and specify solution(s) on QCL assumption for DMRS, e.g. multiple QCL assumptions for the same DMRS port(s), targeting DL-only transmission ii. Evaluate and, if the benefit over Rel.16 HST enhancement baseline is demonstrated, specify QCL/QCL-like relation (including applicable type(s) and the associated requirement) between DL and UL signal by reusing the unified TCI framework

The document contains summary of the evaluation assumptions agreed in RAN1#102-e meeting with some editorial updates, where applicable. This document is for information purposes.

# Summary of evaluations assumptions agreed in RAN1#102-e meeting

- LLS to be used for Rel-17 HST evaluations

- Use bi-directional as mandatory and uni-directional as optional TRP antenna orientation

- CDL extension is baseline channel model for HST-SFN evaluations in addition to 4-tap channel model

- CDL based channel model in Table 2 for HST-SFN evaluation

- Antenna downtilt and azimuth directions point to the midpoint between the two TRPs

- Perfect synchronization as baseline

- Non-perfect time and frequency synchronization between the TRPs and UE, i.e., modeling of TRP CFO error (where CFO have temporal variation), UE receiver CFO, TRP timing errors may be optionally considered

  - Companies to provide details on how this was modelled in their evaluations

    - For example, uniform distribution between [-ppm ppm]*fc (Hz) for each simulation point where fc is the carrier center frequency and the values of maximum frequency error in ppm are captured TR 38.101-1/2 and TR 38.104

- Rel-15 SFN is used as the baseline for comparison. Performance comparison with other schemes (e.g., Rel-16 URLLC, DPS, etc.) can be also provided

- The results should be reported

  - Per track location (at specific SNR) or

  - Throughput vs SNR at specific location

    - Ds/2 (mid track point)

    - Results for other locations can be reported by each company.

- It is recommended to provide results for SNR = 8, 12, 16, 20 dB

  - Other SNR values are not precluded

  - SNR defined relative only to the reference point closest to TRP

Table 1 LLS simulation assumption for HST-SFN deployment

| Parameter | FR1 |  |  | FR2 |
| --- | --- | --- | --- | --- |
| Duplexing | FDD |  | TDD | TDD |
| TRP layout (Ds, Dmin, etc) | Ds=700m, Dmin=150m For CDL based model – TRP height: 35m, UE height: 1.5m Results for other UE heights can be reported by each company |  |  | Alt 2-1: Ds=700m, Dmin=150m Alt 2-3: Ds=200-300m, Dmin=30-50m Alt 2-4: Ds=580m, Dmin=5m Alt 2-3 is mandatory Alt 2-4 and Alt. 2-1 are optional For Alt 2-1 TRP height is 35m For Alt 2-3 TRP height is 20m For Alt 2-4 TRP height is 5m UE height: 1.5m Results for other UE heights can be reported by each company |
| TRP antenna configuration including number of antennas, pattern, ports, orientation, etc | CDL based extension: 2 ports: [Mg, Ng, M, N, P]=[1, 1, 8, 2, 2], antenna model in Table 5, 16-to-1 mapping is used to virtualize the 16 antenna elements in the adjacent columns with fixed weight to form an antenna 4 ports: [Mg, Ng, M, N, P]=[1,1,8,4,2], antenna model in Table 5, virtualization, 16-to-1 mapping is used to virtualize the 16 antenna elements in the two adjacent columns with fixed weight to form an antenna Optional 8 ports: [Mg, Ng, M, N, P]=[1, 1, 8, 4, 2], antenna model in Table 5, 8-to-1 mapping is used to virtualize the 8 antenna elements in a column with fixed weight to form an antenna port 4-tap channel model: 2 ports: omni-directional, MIMO matrix according to TS 38.101-4 (Annex B.1) 4 ports and 8 ports: antenna model and mapping are the same as for CDL based extension Note: The results for other antenna configurations can be also provided |  |  | CDL based extension: 2 ports: [Mg, Ng, M, N, P]=[1, 1, 4, 8, 2], Antenna model in Table 5 Note: The results for other antenna configurations can be also provided |
| UE antenna configuration including number of antennas, pattern, ports, orientation, etc | 2 ports: [Mg, Ng, M, N, P]=[ 1, 1, 1, 1, 2] or 4 ports: [Mg, Ng, M, N, P]=[1, 1, 1, 2, 2], one-to-one mapping between antenna elements and TXRUs omni-directional antenna |  |  | 2 ports: [Mg, Ng, M, N, P]=[1, 1, 2, 4, 2], Directional antenna model in Table 6 |
| DMRS type | Mandatory: DM-RS type 1 Optional: DM-RS type 2 |  |  |  |
| Number of DMRS symb. | 1+1+1 |  |  |  |
| TDD pattern | N/A | DDDDDDDSUU, S: 6D 4G 4U |  | DDDDDDDSUU, S: 6D 4G 4U |
| MCS | MCS 4/MCS 13/MCS 17 based on 64QAM table Note: Companies can also provide results with MCS adaptation |  |  |  |
| Number of scheduled RBs | 10 or 50. Other values are optional. |  |  |  |
| Propagation condition | 4-tap channel model (TS 36.101 (Annex B.3A) / TR 36.878) CDL extension (CDL D/E, DS = 100ns) |  |  | CDL extension (CDL D/E, DS = 20ns/30ns) |
| TRS configuration, TRS periodicity | 10ms, 2-slot pattern Note: results for 20ms periodicity can be also provided |  |  |  |
| PDSCH mapping | Type A, Start symbol 2, Duration 12 |  |  |  |
| Rank | Rank 1 Optional: other ranks or rank adaptation |  |  |  |
| BW | 10 MHz or 20 MHz |  |  | 20MHz or 50MHz or 80MHz |
| Carrier frequency or maximum Doppler shift | 2GHz, 350kmph or 500kmph | 3.5GHz, 350kmph or 500kmph |  | 30 GHz 200 kmph or 350kmph |
| Performance metric | Throughput; BLER |  |  |  |
| Other assumptions or simulation parameters, e.g., correlation | 1) SCS: 30kHz, 15kHz as optional 2) Note: precoding method should be provided by each company | 1) SCS: 30kHz 2) Note: precoding method should be provided by each company |  | 1) SCS: 120kHz 2) Note: precoding method and analog beamforming details should be provided by each company |

Table 2 CDL based channel model for HST-SFN deployment

CDL based channel model proposal for HST: Combination of the CDL channel model in TR38.901 and the 4-tap channel model in TS36.101 Annex B.3A could be considered. As illustrated in figure below, 2-tap channel model for simplicity could be assumed which is similar to RAN4’s 4-tap assumption in order to reflect the characteristic of SFN-based transmission, and for each tap, CDL channel model in TR38.901 could be used to model the effect of the directional antenna of gNB. - The delay for k’th TRP is modified as τk,n=τk+τn where τk is the delay of k’th TRP, which can be derived as τk=y(t)-xkC where τn is the delay of the n’th channel cluster as in Table 7.7.1-1~7.7.1-5 in 38.901 and assume the location of the k’th TRP is xk, and the UE’s location is y(t). The delay spread for different TRPs could be modelled as different. - The normalized power for k’th TRP is modified as Pk=10lg1y(t)-xk2i=1,21y(t)-xi2(dB) - To generate the modified angle parameters, the scaling method mentioned in subclause 7.7.5.1 in TS 38.901 is used φn,scaled=ASdesiredASmodelφn,model-μφ,model+μφ,desired where ASmodel=ASdesired could be assumed, and μφ,desired of the k’th TRP is the AOD, AOA, ZOD and ZOA of LOS direction derived based on the locations and antenna heights of UE and TRPs. - is the tabulated CDL ray angle - is the rms angular spread of the tabulated CDL including the offset ray angles, calculated using the angular spread definition in Annex A in TS 38.901 - is the mean angle of the tabulated CDL, calculated using the definition in Annex A in TS 38.901 - is the desired mean angle - is the desired rms angular spread - is the resulting scaled ray angle. of the k’th TRP is the AOD, AOA, ZOD and ZOA of LOS cluster derived by the locations and antenna heights of UE and TRPs. If is used to denote the distance between UE and TRP1. For AOD1 of TRP1, For AOA1 of TRP1, For AOD2 of TRP2, For AOA2 of TRP2, For ZOD1 of TRP1, For ZOD1 of TRP2, For ZOA2 of TRP1 , For ZOA2 of TRP2, Fig. 1. Simplified and updated HST-SFN channel model for evaluation The gNB antenna boresight could direct to the middle point on the railway between two TRPs. CDL-D and CDL-E channels models are recommended for evaluations.

Table 5 Antenna radiation pattern for TRP

| Radiation power pattern of a single antenna element for TRP | Vertical cut of the radiation power pattern (dB) |  |
| --- | --- | --- |
|  | Horizontal cut of the radiation power pattern (dB) |  |
|  | 3D radiation power pattern (dB) |  |
|  | Maximum directional gain of an antenna element GE,max | 8 dBi |

Table 6 Antenna radiation pattern for UE

| Parameter | Values |
| --- | --- |
| Antenna element radiation pattern in θ'' dim (dB) |  |
| Antenna element radiation pattern in φ'' dim (dB) |  |
| Combining method for 3D antenna element pattern (dB) |  |
| Maximum directional gain of an antenna element GE,max | 5dBi |