3GPP TSG RAN WG1 #102-e						   	                R1-2007431

August 17th – 28th, 2020

Agenda item:	7.2.5.4

Source: 	Moderator (Qualcomm)

Title: 	Endorsed TP for [102-e-NR-L1enh-URLLC-Scheduling and HARQ-02]

Document for:	Discussion and Decision

# Introduction

This document provides a text proposal for the list of issues identified during the preparation pertaining to the HARQ and scheduling aspects (AI 7.2.5.4) of NR eURLLC.

[102-e-NR-L1enh-URLLC-Scheduling and HARQ-01] Prioritization order involving semi-static DL symbols, SSB and dynamic SFI – Kianoush (Qualcomm)

1.  Processing order between semi-static TDD configuration and intra-UE prioritization

1.  Order of intra-UE prioritization and cancellation due to collision with configured DL symbols and SSB

2.  Clarification on the 3-step UE behavior and UE behavior in case there is collision after step 3.

2.  Processing order between dynamic SFI and intra-UE prioritization

- Proposed agreement:

  - UE behavior of handling intra-UE prioritization/multiplexing for overlapping UL transmissions on semi-static flexible symbols is not affected by UL cancellation due to dynamic SFI or DL grant.

  - Note that the proposed agreement is to start the discussions.

# Text Proposal for Processing Order between Dynamic SFI and Intra-UE prioritization

| Reason for change: | - Define an order between applying dynamic SFI and intra-UE prioritization - Editorial modifications for uplink cancellation indication. |
| --- | --- |
| Summary of change: | Intra-UE prioritization is not impacted by dynamic SFI |
| Consequences if not approved: | Ambiguity between a UE and a gNB |

TP for TS 38.213. 11.1 Slot configuration -Unchanged text is omitted- For operation on a single carrier in unpaired spectrum, if a UE is configured by higher layers to transmit SRS, or PUCCH, or PUSCH, or PRACH in a set of symbols of a slot and the UE detects a DCI format indicating to the UE to receive CSI-RS or PDSCH in a subset of symbols from the set of symbols, then - the UE does not expect to cancel the transmission in symbols from the set of symbols that occur,relative to a last symbol of a CORESET where the UE detects the DCI format, after a number of symbols that is smaller than the PUSCH preparation time for the corresponding UE processingcapability [6, TS 38.214] assuming and corresponds to the smallest SCS configuration between the SCS configuration of the PDCCH carrying the DCI format and the SCS configuration of the SRS, PUCCH, PUSCH or mr, where mr corresponds to the SCS configuration of the PRACH if it is 15kHz or higher; otherwise mr=0 - the UE cancels the PUCCH, or a PUSCH transmission, or an actual repetition of a PUSCH transmission [6, TS 38.214] if the PUSCH transmission is with PUSCH repetitions Type B, as determined in Clauses 9 and 9.2.5 or in Clauses 6.1 of [6, TS 38.214], or PRACH transmission in remaining symbols from the set of symbols and cancels the SRS transmission in remaining symbols from the subset of symbols -Unchanged text is omitted- 11.1.1 UE procedure for determining slot format -Unchanged text is omitted- If a UE is configured by higher layers to transmit SRS, or PUCCH, or PUSCH, or PRACH in a set of symbols of a slot and the UE detects a DCI format 2_0 with a slot format value other than 255 that indicates a slot format with a subset of symbols from the set of symbols as downlink or flexible, or the UE detects a DCI format indicating to the UE to receive CSI-RS or PDSCH in a subset of symbols from the set of symbols, then - the UE does not expect to cancel the transmission in symbols from the set of symbols that occur, relative to a last symbol of a CORESET where the UE detects the DCI format 2_0 or the DCI format, after a number of symbols that is smaller than the PUSCH preparation time for the corresponding PUSCH processing capability [6, TS 38.214] assuming and corresponds to the smallest SCS configuration between the SCS configuration of the PDCCH carrying the DCI format 2_0 or the DCI format and the SCS configuration of the SRS, PUCCH, PUSCH or mr, where mrcorresponds to the SCS configuration of the PRACH if it is 15kHz or higher; otherwise mr=0 - the UE cancels the PUCCH, or a PUSCH transmission, or an actual repetition of a PUSCH transmission [6, TS 38.214] if the PUSCH transmission is with PUSCH repetitions Type B, as determined in Clauses 9 and 9.2.5 or in Clauses 6.1 of [6, TS 38.214], or PRACH transmission in remaining symbols from the set of symbols and cancels the SRS transmission in remaining symbols from the subset of symbols. -Unchanged text is omitted- 11.2A Cancellation indication -Unchanged text is omitted- A UE that detects a DCI format 2_4 for a serving cell cancels a PUSCH transmission or an actual repetition of a PUSCH transmission [6, TS 38.214] if the PUSCH transmission is with repetitions Type B, as determined in Clauses 9 and 9.2.5 or in Clause 6.1 of [6, TS 38.214], or an SRS transmission on the serving cell if, respectively, -Unchanged text is omitted- End.