3GPP TSG RAN WG1 #102-e		R1-2007419

e-Meeting, Aug 17th – 28nd, 2020

Title:	LS on evaluation methodology for connected mode UE power saving enhancements

Response to:

Release:	Release 17

Work Item:	NR_UE_pow_sav_enh

Source:	RAN1

To:	RAN2

Cc:	RAN4

Contact Person:

Name: Xiaodong SHEN, Wei-De WU

E-mail Address:	shenxiaodong@vivo.com, Weide.Wu@MEDIATEK.COM

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	None

1. Overall Description:

In response to WID of Rel-17 UE power saving enhancements, RAN1 is expected to inform RAN2 about the evaluation methodology updates related to enhancements on power saving techniques for connected-mode UE.

- Specify enhancements for idle/inactive-mode UE power saving, considering system performance aspects [RAN2, RAN1] - Study and specify paging enhancement(s) to reduce unnecessary UE paging receptions, subject to no impact to legacy UEs [RAN2, RAN1] - NOTE: RAN1 to check and update, if needed, evaluation methodology in RAN1 #102-e meeting - Specify means to provide potential TRS/CSI-RS occasion(s) available in connected mode to idle/inactive-mode UEs, minimizing system overhead impact [RAN1] - NOTE: Always-on TRS/CSI-RS transmission by gNodeB is not required - Study and specify, if agreed, enhancements on power saving techniques for connected-mode UE, subject to minimized system performance impact [RAN1, RAN4] - Study and specify, if agreed, extension(s) to Rel-16 DCI-based power saving adaptation during DRX Active Time for an active BWP, including PDCCH monitoring reduction when C-DRX is configured [RAN1] - NOTE: Rel-15 and Rel-16 available power saving solutions should be supported by the UE and included in the evaluation. RAN1 will ask the confirmation from RAN2 that Rel-15 and Rel-16 available power saving solutions are properly utilized. - Study the feasibility and performance impact of relaxing UE measurements for RLM and/or BFD, particularly for low mobility UE with short DRX periodicity/cycle, and specify, if agreed, relaxation in the corresponding requirements [RAN4] - NOTE: Supplementary RAN2 work, if needed, can be triggered by RAN4 LS

Accordingly, RAN1 would like to inform RAN2 the following related agreements for connected mode:

Agreements:

- Reusing power model in TR38.840 for evaluation of DCI-based power saving adaptation schemes.

  - Note: company reporting additional power model for missing state or update is not precluded.

Agreements:

- Company should report assumptions used for periodic measurement activities for the Rel-17 DCI-based power saving adaptation evaluation.

  - The periodic activities defined in TR38.840 can be reused.

  - Measurement for RLM/BFD every C-DRX cycle can be optionally modelled

Agreements:

- The performance metrics described in TR38.840 section 8.2 is reused for power saving evaluation of Rel-17 DCI-based power saving adaptation during ActiveTime.

- The following Rel-15 / 16 features is recommended of the power consumption as reference for baseline. Company can report the feature(s) being used in the baseline.

  - DRX

    - C-DRX cycle 40msec for VoIP

      - 10ms IAT, 8ms On-duration

      - Assume max two packets bundled

    - C-DRX cycle 160msec for FTP

      - Alt 1: 20 msec IAT, 8ms On-duration

      - Alt 2: short DRX

        - 20 ms [or 40ms as optional] IAT, 8ms On-duration

        - 20 ms for short DRX cycle, 4 cycles

      - Note: 100 msec IAT, 8ms On-duration can also be used with sufficient justifications that available Rel-15/16 Techniques being used to reduce UE power saving

  - DCP for DRX adaptation,

    - DCP offset  to DRX ON = 2 ms, other values are not precluded

  - Cross-slot scheduling adaptation

    - Minimum K0 can be adapted from 0 to 1 for FR1, 0 to [4] for FR2

  - BWP switching, including

    - MIMO layer adaptation,

      - Max # of MIMO layer can be adapted from 4 layer to 2 layer for FR1, 2 layer to 1 layer for FR2

    - PDCCH monitoring period adaptation

      - PDCCH monitoring period can be adapted from per slot monitoring to X slot monitoring

        - X = [2] for FR1 and [8] for FR2

    - Bandwidth adaptation

      - Bandwidth can be adapted from 100MHz to 20MHz for FR1,FFS for FR2

    - Note:

      - BWP transition time type 2 is assumed, BWP transition duration is

        - 5 slot @ 30kHz SCS for FR1,

        - 18 slot@120kHz SCS for FR2

        - the slot-average power level for BWP transition duration is according to TR38.840

        - BWP transition time type 1 can be optional modelled

      - BWP switching is Y (ms) after last packet/data burst.

        - Y = [8], other values are not precluded

      - Whether BWP switching is modeled depends on the assumed UE capability and evaluated schemes.

  - Scell dormancy assumption for CA capable UEs

    - FR1 & FR2: SCell dormancy with [160 ms] periodic CSI measurement and reporting

- Other settings

  - CA assumption if configured for CA capable UEs

    - For FR1, FFS

    - For FR2, 4*100MHz can be considered.

  - Assumptions for scheduler

    - For FR1, no restriction on the beam assumptions being used in each slot

    - For FR2, up to each company, e.g., gNB equally schedule the slots for UEs targeting to different beams.

    - Note: the assumptions does not necessary mean to restrict or precluded any implementation. Other assumptions are not precluded and can be reported by companies.

  - Company to report the used assumption for the interruption and also power savings impact due to presence/absence of interruptions .

Agreements:

Legacy traffic models in TR38.840 can be considered for Rel-17 DCI-based power saving adaptation evaluation, other traffic models can be optionally modelled and company report which traffic model(s) is used.

2. Actions:

To RAN2:

RAN1 respectfully asks RAN2 to take the above evaluation methodology for the connected-mode into account .

3. Date of Next RAN1 Meetings:

TSG-RAN WG1 Meeting 103-e 	26 October – 13 November 2020		e-Meeting

TSG-RAN WG1 Meeting 104	01 – 05 March 2021 	 		Athens, Greece