3GPP TSG RAN WG1 #103-e	R1-2008179

# e-Meeting, Oct. 27th – Nov. 12th, 2020

Agenda Item:		8.8.1.1

Source:				Samsung

Title:					Baseline coverage performance using LLS for FR1

Document for:	Discussion

Introduction

This contribution provides baseline coverage performance for FR1 based on the simulation assumptions agreed in RAN1#102-e meeting and follow-up RAN1 email discussion. Note the link budget excel sheet is attached in a separate file. Simulation results for FR2 are given in [1].

Simulation assumptions

In RAN1#102-e meeting (and post e-mail discussion), simulation parameters were agreed for several scenarios, services and channels. In this contribution, we give the simulation results for various scenarios, such as urban, rural, and rural with long distance scenarios, for uplink and downlink data and control channels, such as PUSCH, PUCCH, PDSCH, and PDCCH. In detail, the values of the simulation parameters used in the evaluation for FR1 are summarized in the Appendix.

Baseline coverage performance for FR1

Link budget results are obtained based on the agreed link budget template [2]. In Table 1 below, required SNR, Maximum Coupling Loss (MCL), Maximum Isotropic Loss (MIL) and Maximum Path Loss (MPL) are summarized for each scenario and channel. The underlying assumption is that all of antenna gain correction factors (Δ1, Δ2, Δ3) are set to zero. Bottleneck channels are determined (marked in yellow) based on relative difference between channels. Overall, we observe that PUSCH and PUCCH need improvement in most evaluated cases.

The evaluated scenarios include urban 4 GHz TDD, rural 4 GHz TDD, rural 2 GHz FDD and rural 700 MHz FDD. For rural scenarios, we also consider O2I/O2O and NLOS/LOS.

Table 1. Baseline coverage performance for FR1

| Urban 4 GHz TDD |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Channel | The required SNR |  | MCL |  | MIL |  | MPL |  | Key assumptions |
| PUSCHforeMBB | -0.95 |  | 116.16 |  | 139.98 |  | 109.25 |  | 30 PRBs |
| PUSCH for VoIP | -3.9 |  | 127.36 |  | 151.18 |  | 120.45 |  | The max # of HARQ: 4/ Latency: 50 ms/ Repetition type A |
|  | -5.9 |  | 129.36 |  | 153.18 |  | 122.45 |  | The max # of HARQ: 4/ Latency: 50 ms/ Repetition type B |
| PUCCH | -7.95 |  | 134.36 |  | 158.18 |  | 124.37 |  | Format 1/ UCI:2 bits |
|  | -7.65 |  | 134.06 |  | 157.88 |  | 124.07 |  | Format 3/ UCI: 4 bits |
|  | -4.65 |  | 131.06 |  | 154.88 |  | 121.07 |  | Format 3/ UCI:_11 bits |
|  | -1.55 |  | 127.96 |  | 151.78 |  | 117.97 |  | Format 3/ UCI:_22 bits |
| PDCCH | -11.25 |  | 138.27 |  | 165.11 |  | 131.30 |  | 48 PRBs |
| PDSCH for eMBB | -3.3 |  | 130.82 |  | 157.66 |  | 126.93 |  | 90 PRBs |
| Rural 4 GHz TDD |  |  |  |  |  |  |  |  |  |
| Channel | The required SNR | MCL |  | MIL | MPL | LOS/ NLOS |  | O2I/ O2O | Key assumptions |
| PUSCH for eMBB | -0.5 | 124.46 |  | 143.51 | 131.3 | NLOS |  | O2I | 4 PRBs |
|  | -0.85 | 124.81 |  | 143.86 | 128.25 | NLOS |  | O2O | 4 PRBs |
| PUSCH for VoIP | -3.9 | 127.36 |  | 146.41 | 128.78 | NLOS |  | O2I | The max # of HARQ: 4/ Latency: 50 ms/ Repetition type A |
|  | -5.9 | 129.36 |  | 148.41 | 130.78 | NLOS |  | O2I | The max # of HARQ: 4/ Latency: 50 ms/ Repetition type B |
|  | -6.9 | 130.36 |  | 149.41 | 133.80 | NLOS |  | O2O | The max # of HARQ: 4/ Latency: 50 ms/ Repetition type A |
|  | -7.6 | 131.06 |  | 150.11 | 134.50 | NLOS |  | O2O | The max # of HARQ: 4/ Latency: 50 ms/ Repetition type B |
| PUCCH | -7.95 | 134.36 |  | 153.41 | 132.46 | NLOS |  | O2I | Format 1/ 2 bits |
|  | -7.65 | 134.06 |  | 153.11 | 132.16 | NLOS |  | O2I | Format 3/ 4 bits |
|  | -4.65 | 131.06 |  | 150.11 | 129.16 | NLOS |  | O2I | Format 3/ 11 bits |
|  | -1.55 | 127.96 |  | 147.01 | 126.06 | NLOS |  | O2I | Format 3/ 22 bits |
|  | -7.95 | 134.36 |  | 153.41 | 133.96 | NLOS |  | O2O | Format 1/ 2 bits |
|  | -7.65 | 134.06 |  | 153.11 | 133.66 | NLOS |  | O2O | Format 3/ 4 bits |
|  | -4.65 | 131.06 |  | 150.11 | 130.66 | NLOS |  | O2O | Format 3/ 11 bits |
|  | -1.55 | 127.96 |  | 147.01 | 127.56 | NLOS |  | O2O | Format 3/ 22 bits |
| PDCCH | -11.30 | 154.52 |  | 159.53 | 138.58 | NLOS |  | O2I | 48 PRBs |
|  | -11.20 | 154.42 |  | 159.43 | 139.98 | NLOS |  | O2O | 48 PRBs |
| PDSCH for eMBB | -2.65 | 146.37 |  | 151.38 | 133.75 | NLOS |  | O2I | 10 PRBs |
|  | -2.95 | 146.67 |  | 151.68 | 136.07 | NLOS |  | O2O | 10 PRBs |
| Rural 2 GHz FDD |  |  |  |  |  |  |  |  |  |
| Channel | The required SNR | MCL |  | MIL | MPL | LOS/ NLOS |  | O2I/ O2O | Key assumptions |
| PUSCH for eMBB | -2.7 | 129.67 |  | 145.71 | 128.08 | NLOS |  | O2I | 4 PRBs |
|  | -2 | 128.97 |  | 145.01 | 129.40 | NLOS |  | O2O | 4 PRBs |
| PUSCH for VoIP | -4.55 | 131.02 |  | 147.06 | 129.43 | NLOS |  | O2I | The # of repetition: 4/ The max # of HARQ tx: 5/ Latency: 32 ms/ Repetition type A |
|  | -8.7 | 135.17 |  | 151.21 | 135.60 | NLOS |  | O2O | The # of repetition: 4/ The max # of HARQ tx: 5/ Latency: 32 ms/ Repetition type A |
| PUCCH | -7.95 | 137.37 |  | 153.41 | 132.46 | NLOS |  | O2I | Format 1/ 2 bits |
|  | -7.65 | 137.07 |  | 153.11 | 132.16 | NLOS |  | O2I | Format 3/ 4 bits |
|  | -4.65 | 134.07 |  | 150.11 | 129.16 | NLOS |  | O2I | Format 3/ 11 bits |
|  | -1.55 | 130.97 |  | 147.01 | 126.06 | NLOS |  | O2I | Format 3/ 22 bits |
|  | -7.95 | 137.37 |  | 153.41 | 133.96 | NLOS |  | O2O | Format 1/ 2 bits |
|  | -7.65 | 137.07 |  | 153.11 | 133.66 | NLOS |  | O2O | Format 3/ 4 bits |
|  | -4.65 | 134.07 |  | 150.11 | 130.66 | NLOS |  | O2O | Format 3/ 11 bits |
|  | -1.55 | 130.97 |  | 147.01 | 127.56 | NLOS |  | O2O | Format 3/ 22 bits |
| PDCCH | -11.40 | 151.58 |  | 168.63 | 147.68 | NLOS |  | O2I | 48 PRBs |
|  | -11.40 | 151.58 |  | 168.63 | 149.18 | NLOS |  | O2O | 48 PRBs |
| PDSCH for eMBB | -1.6 | 142.28 |  | 159.33 | 141.70 | NLOS |  | O2I | 10 PRBs |
|  | -1.9 | 142.58 |  | 159.63 | 144.02 | NLOS |  | O2O | 10 PRBs |
| Rural with long distance 700 MHz FDD |  |  |  |  |  |  |  |  |  |
| Channel | The required SNR | MCL |  | MIL | MPL | LOS/ NLOS |  | O2I/ O2O | Key assumptions |
| PUSCH for eMBB | -7.8 | 134.77 |  | 147.80 | 134.01 | LOS |  | O2O | 4 PRBs |
| PUSCH for VoIP | -15 | 141.47 |  | 154.50 | 140.71 | LOS |  | O2O | The # of repetition: 4/ The max # of HARQ tx: 5/ Latency: 32 ms/ Repetition type A |
| PUCCH | -10.10 | 139.52 |  | 152.55 | 135.49 | LOS |  | O2O | Format 1/ 2 bits |
|  | -10.20 | 139.62 |  | 152.65 | 135.59 | LOS |  | O2O | Format 3/ 4 bits |
|  | -8 | 137.42 |  | 150.45 | 133.39 | LOS |  | O2O | Format 3/ 11 bits |
|  | -5.1 | 134.52 |  | 147.55 | 130.49 | LOS |  | O2O | Format 3/ 22 bits |
| PDCCH | -12.30 | 152.48 |  | 163.51 | 146.45 | LOS |  | O2O | 48 PRBs |
| PDSCH for eMBB | -4.9 | 145.58 |  | 156.61 | 142.82 | LOS |  | O2O | 10 PRBs |

For each scenario, the bottleneck channels are summarized as following:

- Urban (4 GHz, TDD): PUSCH for eMBB, PUCCH with 22 UCI bits

- Rural (4 GHz, O2I, TDD): PUCCH with 22 UCI bits, PUSCH for VoIP with PUSCH repetition type A

- Rural (4 GHz, O2O, TDD): PUCCH with 22 UCI bits, PUSCH for eMBB

- Rural (2 GHz, O2I, FDD): PUCCH with 11 and 22 UCI bits, PUSCH for eMBB and VoIP with repetition type A

- Rural (2 GHz, O2O, FDD): PUCCH with 22 UCI bits, PUSCH for eMBB

- Rural (700 MHz, long distance, FDD): PUCCH with 22 UCI bits

Based on the results, it is observed that coverage enhancements for PUSCH and PUCCH are needed for FR1.

Observation 1. In terms of coverage enhancement, PUSCH and PUCCH are bottleneck compared to other channels for FR1.

Proposal 1. Support coverage enhancements for PUSCH and PUCCH for FR1.

Conclusion

This contribution presents FR1 simulation results to identify baseline coverage performance of several UL and DL channels in FR1. The following observations and proposals are made:

Observation 1. In terms of coverage enhancement, PUSCH and PUCCH are bottleneck compared to other channels for FR1.

Proposal 2. Support coverage enhancements for PUSCH and PUCCH for FR1.

- References

- R1-2008180, Baseline coverage performance using LLS for FR2, Samsung

- R1-2007480, [102-e-Post-NR-CovEnh-02] Summary on email discussion/approval of Phase 1 - link budget template, Moderator (SoftBank)

- Appendix

Table A-1 Simulation parameters

| Parameters | Values |
| --- | --- |
| Scenario and frequency | Urban: 4GHz (TDD) Rural: 4GHz (TDD), 2GHz (FDD) Rural with long distance: 700MHz (FDD) |
| Frame structure for TDD | DDDSUDDSUU (S: 10D:2G:2U) only for 4GHz |
| Pathloss model (select from LoS or NLoS) | Urban: NLoS Rural: NLoS and LoS |
| BWP | 100MHz for 4GHz. 20MHz for 2GHz and 700MHz |
| SCS | 30kHz for TDD, 15kHz for FDD. |
| Channel model for link-level simulation | TDL-C for NLOS, TDL-D for LOS. |
| UE velocity | Urban: 3km/h for indoor Rural: 3km/h for indoor, 120km/h for outdoor |
| Number of receive antenna elements for BS | Urban: 192 antenna elements for 4GHz, Rural: 64 antenna elements for 4GHz 32 antenna elements for 2GHz 16 antenna elements for 700MHz |
| Number of TxRUs for BS | gNB modeling in LLS for TDL: 2 gNB receive chains in LLS. |
| Delay spread | Urban: 300ns Rural: 300ns Rural with long distance: 30ns |

Table A-2. Channel specific parameters for PUSCH

| Parameters | Values |
| --- | --- |
| BLER for PUSCH | For eMBB, w/o HARQ, 10% iBLER. For VoIP, 2% rBLER. |
| Number of UE transmit chains for PUSCH | 1 |
| DMRS configuration for PUSCH | For 120km/h: Type I, 2 DMRS symbol, no multiplexing with data. For 3km/h: Type I, 1 DMRS symbol, no multiplexing with data. |
| Waveform for PUSCH | DFT-s-OFDM |
| Repetitions for PUSCH | For eMBB, w/o repetition as baseline, For VoIP, w/ repetition and # of actual repetitions = 3 (14/14/14 symbols in DDDSUDDSUU with repetition type A) For VoIP, w/ repetition and # of actual repetitions = 5 (2/14/2/14/14 symbols in DDDSUDDSUU with repetition type B) |
| HARQ configuration for PUSCH | For eMBB, no HARQ retransmission. For VoIP, w/ HARQ and the maximum number of HARQ transmission = 4 due to latency requirements. |
| Latency requirements for voice | 50ms |
| PUSCH duration | 14 OS |
| PRBs/TBS/MCS for eMBB for PUSCH | 30 PRBs for 1Mbps and 4 PRBs for 100kbps QPSK modulation |
| PRBs/MCS for VoIP for PUSCH | 4 PRBs for VoIP QPSK modulation |
| Frequency hopping | w/o frequency hopping for PUSCH |

Table A-3. Channel specific parameters for PUCCH

| Parameters | Values |
| --- | --- |
| PUCCH format type | Format 1, 2bits UCI. Format 3, 4/11/22 bits UCI |
| BLER for PUCCH | For PUCCH format 1: ACK missed detection probability: 1%. For PUCCH format 3: BLER: 1% |
| Number of PRBs for PUCCH | 1 PRB |
| Number of UE transmit chains for PUCCH | 1 |
| Number of repetitions for PUCCH | w/o repetition for PUCCH. |
| PUCCH duration | 14 OS |
| DMRS configuration for PUCCH | Additional 1 DMRS symbols for PUCCH Format 3. |
| Frequency hopping | w/ frequency hopping for PUCCH. |

Table A-4. Channel specific parameters for PDSCH

| Parameters | Values |
| --- | --- |
| Waveform | CP-OFDM |
| PRBs/MCS/TBS | 90 PRBs for urban scenario 10 PRBs for other scenarios QPSK modulation |
| PDSCH duration | 12 OS |
| Other parameters | FFS |

Table A-5. Channel specific parameters for PDCCH

| Parameters | Values |
| --- | --- |
| Aggregation level | 16 |
| Payload | 40 bits |
| CORESET size | 2 symbols, 48 PRBs |
| Tx Diversity | Precoder cycling |
| BLER for PDCCH | 1% BLER |